3GPP TSG-RAN WG5 #79 	R5-182395

# Busan, Korea, May 21st – May 25th, 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 4178 | Rev |  | Current version: | 15.1.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updated In Band Blocking test for 4DLCA combo CA_40E and 5DLCA-2ULCA combo CA_41F |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R14-UEConTest |  |  |  |  |  |  | Date: |  | 2018-05-21 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | New combos introduced : CA_40E(4DL CA) and CA_41F(5DL-2UL CA) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated respective tables with the combo addition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This is new addition. Without adding it to specifications, conformance testing cannot be defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

[Start of Changes]

#### 7.6.1A.0	Minimum conformance requirements

<Unchanged section skipped>

Table 7.6.1A.0-2: In-band blocking

| CA configuration | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| CA_1C, CA_2C, CA_3B, CA_3C, CA_5B, CA_7C, CA_8B, CA_12B, CA_23B, CA_27B, CA_38C, CA_39C, CA_40C, CA_40D, CA_40E, CA_41C, CA_41D, CA_41E, CA_41F, CA_42C, CA_42D, CA_42E, CA_42F, CA_43C, CA_48C, CA_48D, CA_48E, CA_48F, CA_66B, CA_66C, CA_70C | FInterferer (Range) | MHz | (Note 2) | FDL_low – 15 to FDL_high + 15 |
| Note 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band Note 2: For each carrier frequency the requirement is valid for two frequencies: a) the carrier frequency -Foffset - FIoffset, case 1 and b) the carrier frequency +Foffset + FIoffset, case 1 Note 3: Foffset is the frequency offset from the centre frequency of the adjacent CC being tested to the edge of aggregated channel bandwidth. Note 4: The Finterferer (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

<Unchanged section skipped>

#### 7.6.1A.7	In-band blocking for 4DL CA

<Unchanged section skipped>

##### 7.6.1A.7.5	Test Requirement

Table 7.6.1A.7.5-3: In-band blocking (intra-band contiguous)

| CA configuration | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| CA_1C, CA_2C, CA_3C, CA_5B, CA_7C, CA_8B, CA_12B, CA_23B, CA_27B, CA_38C, CA_39C, CA_40C, CA_41C, CA_40D, CA_40E, CA_41D, CA_42C, CA_42D, CA_42E, CA66B, CA_66C, CA_70C, CA_66D | FInterferer (Range) | MHz | (Note 2) | FDL_low – 15 to FDL_high + 15 |
| Note 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band. Note 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -Foffset - FIoffset, case 1 and b. the carrier frequency +Foffset + FIoffset, case 1 Note 3: Foffset is the frequency offset from the centre frequency of the adjacent CC being tested to the edge of aggregated channel bandwidth. Note 4: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

<Unchanged section skipped>

#### 7.6.1A.8	In-band blocking for 5DL CA

<Unchanged section skipped>

Table 7.6.1A.8.5-3: In-band blocking (intra-band contiguous)

| CA configuration | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| CA_1C, CA_2C, CA_3B, CA_3C, CA_5B, CA_7C, CA_8B, CA_12B, CA_23B, CA_27B, CA_38C, CA_39C, CA_40C, CA_40D, CA_40E, CA_41C, CA_41D, CA_41E, CA_41F, CA_42C, CA_42D, CA_42E, CA_42F, CA_43C, CA_48C, CA_48D, CA_48E, CA_48F, CA_66B, CA_66C, CA_70C, CA_66D | FInterferer (Range) | MHz | (Note 2) | FDL_low – 15 to FDL_high + 15 |
| Note 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band. Note 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -Foffset - FIoffset, case 1 and b. the carrier frequency +Foffset + FIoffset, case 1 Note 3: Foffset is the frequency offset from the centre frequency of the adjacent CC being tested to the edge of aggregated channel bandwidth. Note 4: The Finterferer (offset) is the frequency separation of the centre frequency of the carrier closest to the interferer and the centre frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

[End of Changes]