3GPP TSG-RAN5 Meeting #81 	R5-186428r1

# Spokane, USA, 12-16 Nov 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 499 | rev | - | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new Test Configuration on TC 6.2.2A.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | DEKRA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R14-UEConTest |  |  |  |  |  | Date: |  |  | 2018-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A CA Configuration for inter-band CA_4A_12A is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Configuration for CA_4A_12A added to TS 36.521-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New CA configuration combination in Test Configuration will be missing in TS 36.521-1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: Original text updated to version 15.3.1 (previously 15.2.0). |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

##### 6.2.2A.2.4	Test description

6.2.2A.2.4.1	Initial conditions

Same as in clause 6.2.2A.1.4.1 with the following exceptions:

-	Instead of Table 5.4.2A.1-1 use Table 5.4.2A.1-2 for CA configurations with two bands or Table 5.4.2A.1-2a for CA configurations with three bands.

-	Instead of Table 6.2.2A.1.4.1-1 use Table 6.2.2A.2.4.1-1.

-	Instead of clause 6.2.2A.1.4.3 use clause 6.2.2A.2.4.3.

Table 6.2.2A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC (Note 3) |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/Afor this test | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 25 |  | QPSK | 2 | P_1@5 | S_1@24 | - | - |
| 6 | 25 |  | QPSK | 13 | P_5@0 | S_8@0 | - | - |
| 6 | 25 |  | QPSK | 13 | P_5@1 | S_8@17 | - | - |
| 6 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 50 |  | QPSK | 2 | P_1@5 | S_1@49 | - | - |
| 6 | 50 |  | QPSK | 17 | P_5@0 | S_12@0 | - | - |
| 6 | 50 |  | QPSK | 17 | P_5@1 | S_12@38 | - | - |
| 15 | 25 |  | QPSK | 2 | S_1@0 | P_1@0 |  |  |
| 15 | 25 |  | QPSK | 2 | S_1@14 | P_1@24 |  |  |
| 15 | 25 |  | QPSK | 13 | S_5@0 | P_8@0 |  |  |
| 15 | 25 |  | QPSK | 13 | S_5@10 | P_8@17 |  |  |
| 25 | 6 |  | QPSK | 2 | P_1@0 | P_1@0 |  |  |
| 25 | 6 |  | QPSK | 2 | P_1@24 | P_1@5 |  |  |
| 25 | 6 |  | QPSK | 13 | P_8@0 | P_5@0 |  |  |
| 25 | 6 |  | QPSK | 13 | P_8@17 | P_5@1 |  |  |
| 25 | 15 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 15 |  | QPSK | 2 | P_1@24 | S_1@14 | - | - |
| 25 | 15 |  | QPSK | 12 | P_8@0 | S_5@0 | - | - |
| 25 | 15 |  | QPSK | 12 | P_8@17 | S_5@10 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@24 | S_1@24 | - | - |
| 25 | 25 |  | QPSK | 16 | P_8@0 | S_8@0 | - | - |
| 25 | 25 |  | QPSK | 16 | P_8@17 | S_8@17 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@24 | S_1@49 | - | - |
| 25 | 50 |  | QPSK | 20 | P_8@0 | S_12@0 | - | - |
| 25 | 50 |  | QPSK | 20 | P_8@17 | S_12@38 | - | - |
| 50 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 25 |  | QPSK | 2 | P_1@49 | S_1@24 | - | - |
| 50 | 25 |  | QPSK | 8 | P_12@0 | S_8@0 | - | - |
| 50 | 25 |  | QPSK | 8 | P_12@38 | S_8@17 | - | - |
| 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 50 |  | QPSK | 2 | P_1@49 | S_1@49 | - | - |
| 50 | 50 |  | QPSK | 24 | P_12@0 | S_12@0 | - | - |
| 50 | 50 |  | QPSK | 24 | P_12@38 | S_12@38 | - | - |
| 25 | 75 |  | QPSK | 2 | P_1@0 | P_1@0 |  |  |
| 25 | 75 |  | QPSK | 2 | P_1@24 | P_1@74 |  |  |
| 25 | 75 |  | QPSK | 24 | P_8@0 | P_16@0 |  |  |
| 25 | 75 |  | QPSK | 24 | P_8@17 | P_16@59 |  |  |
| 50 | 100 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 100 |  | QPSK | 2 | P_1@49 | S_1@99 | - | - |
| 50 | 100 |  | QPSK | 30 | P_12@0 | S_18@0 | - | - |
| 50 | 100 |  | QPSK | 30 | P_12@38 | S_18@82 | - | - |
| 75 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 25 |  | QPSK | 2 | P_1@74 | S_1@24 | - | - |
| 75 | 25 |  | QPSK | 24 | P_16@0 | S_8@0 | - | - |
| 75 | 25 |  | QPSK | 24 | P_16@59 | S_8@17 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@74 | S_1@74 | - | - |
| 75 | 75 |  | QPSK | 32 | P_16@0 | S_16@0 | - | - |
| 75 | 75 |  | QPSK | 32 | P_16@59 | S_16@59 | - | - |
| 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 50 |  | QPSK | 2 | P_1@99 | S_1@49 | - | - |
| 100 | 50 |  | QPSK | 30 | P_18@0 | S_12@0 | - | - |
| 100 | 50 |  | QPSK | 30 | P_18@82 | S_12@38 | - | - |
| 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 75 |  | QPSK | 2 | P_1@99 | S_1@74 | - | - |
| 100 | 75 |  | QPSK | 34 | P_18@0 | S_16@0 | - | - |
| 100 | 75 |  | QPSK | 34 | P_18@82 | S_16@59 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@99 | S_1@99 | - | - |
| 100 | 100 |  | QPSK | 36 | P_18@0 | S_18@0 | - | - |
| 100 | 100 |  | QPSK | 36 | P_18@82 | S_18@82 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. Note 3: For Low range use only test points with RBstart=0. For High range use only test points with RBstart=RBmax-LCRB+1. |  |  |  |  |  |  |  |  |

6.2.2A.2.4.2	Test procedure

Same as in clause 6.2.2A.1.4.2 with the following exceptions:

-	Instead of Table 6.2.2A.1.4.1-1 use Table 6.2.2A.2.4.1-1.

-	Instead of clause 6.2.2A.1.4.3 use clause 6.2.2A.2.4.3.

6.2.2A.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

< End of changes >