3GPP TSG-RAN5 Meeting #81 	R5-186438r1

# Spokane, USA, 12 -16 Nov 2018

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 4509 | rev | - | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new Test Configuration on TC 6.5.2A.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | DEKRA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_R14-UEConTest |  |  |  |  |  | Date: |  |  | 2018-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | A CA Configuration for inter-band CA_4A_12A is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Configuration for CA_4A_12A added to TS 36.521-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New CA configuration combination in Test Configuration will be missing in TS 36.521-1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2A.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: Original text updated to version 15.3.1 (previously 15.2.0). |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

##### 6.5.2A.1.2	Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA)

6.5.2A.1.2.1	Test Purpose

For the inter-band carrier aggregation, the Error Vector Magnitude requirement should be defined for each component carrier. Requirement applies for the allocated component carrier, when all other component carriers are activated, but not allocated.

Similar transmitter impairment removal procedures are applied for CA waveform before EVM calculation as is specified for non-CA waveform in sub-section 6.5.2.1.

6.5.2A.1.2.2	Test applicability

This test case applies to all types of E-UTRA UE Release 11 and forward that support inter-band DL CA and UL CA.

This test case also applies to all types of E-UTRA UE release 14 and forward that supports uplink LAA.

6.5.2A.1.2.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.5.2A.1.0.

6.5.2A.1.2.4	Test description

6.5.2A.1.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and CC combinations based on E-UTRA CA configurations specified in table 5.4.2A.1-2. All of these configurations shall be tested with applicable test parameters for each CA Configuration, and are shown in table 6.5.2A.1.2.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5.2A.1.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |  |  |
| Config ID |  | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  |  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 |  | 6 | 25 | N.A. | QPSK | 1 | P_1@0 | S_0@0 | - | - |  |
| 2 |  | 6 | 25 |  | QPSK | 1 | P_1@5 | S_0@0 | - | - |
| 3 |  | 6 | 25 |  | QPSK | 6 | P_6@0 | S_0@0 | - | - |
| 4 |  | 6 | 25 |  | 16QAM | 1 | P_1@0 | S_0@0 | - | - |
| 5 |  | 6 | 25 |  | 16QAM | 1 | P_1@5 | S_0@0 | - | - |
| 6 |  | 6 | 25 |  | 16QAM | 6 | P_6@0 | S_0@0 | - | - |
| 7 |  | 6 | 50 |  | QPSK | 1 | P_1@0 | S_0@0 | - | - |
| 8 |  | 6 | 50 |  | QPSK | 1 | P_1@5 | S_0@0 | - | - |
| 9 |  | 6 | 50 |  | QPSK | 6 | P_6@0 | S_0@0 | - | - |
| 10 |  | 6 | 50 |  | 16QAM | 1 | P_1@0 | S_0@0 | - | - |
| 11 |  | 6 | 50 |  | 16QAM | 1 | P_1@5 | S_0@0 | - | - |
| 12 |  | 6 | 50 |  | 16QAM | 6 | P_6@0 | S_0@0 | - | - |
| 13 |  | 15 | 25 |  | QPSK | 4 | P_4@0 | S_0@0 | - | - |
| 14 |  | 15 | 25 |  | QPSK | 4 | P_4@11 | S_0@0 | - | - |
| 15 |  | 15 | 25 |  | QPSK | 15 | P_15@0 | S_0@0 | - | - |
| 16 |  | 15 | 25 |  | 16QAM | 4 | P_4@0 | S_0@0 | - | - |
| 17 |  | 15 | 25 |  | 16QAM | 4 | P_4@11 | S_0@0 | - | - |
| 18 |  | 15 | 25 |  | 16QAM | 15 | P_15@0 | S_0@0 | - | - |
| 19 |  | 25 | 6 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 20 |  | 25 | 6 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 21 |  | 25 | 6 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 22 |  | 25 | 6 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 23 |  | 25 | 6 |  | 16QAM | 8 | P_8@17 | S_0@0 | - | - |
| 24 |  | 25 | 6 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 25 |  | 25 | 15 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 26 |  | 25 | 15 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 27 |  | 25 | 15 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 28 |  | 25 | 15 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 29 |  | 25 | 15 |  | 16QAM | 8 | P_8@17 | S_0@0 | - | - |
| 30 |  | 25 | 15 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 31 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 32 | 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 33 | 25 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 34 | 25 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 35 | 25 | 25 |  | 16QAM | 8 | P_8@17 | S_0@0 | - | - |
| 36 | 25 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 37 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 38 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 39 | 25 | 50 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 40 | 25 | 50 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 41 | 25 | 50 |  | 16QAM | 8 | P_8@17 | S_0@0 | - | - |
| 42 | 25 | 50 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 43 | 50 | 6 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 44 | 50 | 6 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 45 | 50 | 6 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 46 | 50 | 6 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 47 | 50 | 6 |  | 16QAM | 12 | P_12@38 | S_0@0 | - | - |
| 48 | 50 | 6 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 49 | 50 | 25 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 50 | 50 | 25 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 51 | 50 | 25 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 52 | 50 | 25 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 53 | 50 | 25 |  | 16QAM | 12 | P_12@38 | S_0@0 | - | - |
| 54 | 50 | 25 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 55 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 56 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 57 | 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 58 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 59 | 50 | 50 |  | 16QAM | 12 | P_12@38 | S_0@0 | - | - |
| 60 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 61 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 62 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 63 | 50 | 100 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 64 | 50 | 100 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 65 | 50 | 100 |  | 16QAM | 12 | P_12@38 | S_0@0 | - | - |
| 66 | 50 | 100 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 67 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 68 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 69 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 70 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 71 | 75 | 75 |  | 16QAM | 16 | P_16@59 | S_0@0 | - | - |
| 72 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 73 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 74 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 75 | 75 | 100 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 76 | 75 | 100 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 77 | 75 | 100 |  | 16QAM | 16 | P_16@59 | S_0@0 | - | - |
| 78 | 75 | 100 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 79 | 100 | 50 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 80 | 100 | 50 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 81 | 100 | 50 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 82 | 100 | 50 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 83 | 100 | 50 |  | 16QAM | 18 | P_18@82 | S_0@0 | - | - |
| 84 | 100 | 50 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 85 | 100 | 75 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 86 | 100 | 75 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 87 | 100 | 75 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 88 | 100 | 75 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 89 | 100 | 75 |  | 16QAM | 18 | P_18@82 | S_0@0 | - | - |
| 90 | 100 | 75 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 91 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 92 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 93 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 94 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 95 | 100 | 100 |  | 16QAM | 18 | P_18@82 | S_0@0 | - | - |
| 96 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 97 | 25 | 75 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 98 | 25 | 75 |  | QPSK | 8 | P_8@17 | S_0@0 |  |  |
| 99 | 25 | 75 |  | QPSK | 25 | P_25@0 | S_0@0 |  |  |
| 100 | 25 | 75 |  | 16QAM | 8 | P_8@0 | S_0@0 |  |  |
| 101 | 25 | 75 |  | 16QAM | 8 | P_8@17 | S_0@0 |  |  |
| 102 | 25 | 75 |  | 16QAM | 25 | P_25@0 | S_0@0 |  |  |
| X+1 | 75 | 25 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| X+2 | 75 | 25 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| X+3 | 75 | 25 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| X+4 | 75 | 25 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| X+5 | 75 | 25 |  | 16QAM | 16 | P_16@59 | S_0@0 | - | - |
| X+6 | 75 | 25 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |  |  |  |  |  |  |  |

Table 6.5.2A.1.2.4.1-2: Test Configuration Table for Uplink LAA CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |  |
| Test Parameters for the uplink LAA CA Configurations |  |  |  |  |  |  |  |  |  |  |  |
| Config ID |  | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |  |
|  |  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC RB Allocation (LCRB @ RBstart) SCC RB Allocations(L@ RBstart) Note2 |  |  |  |
| 1 | 25 | 100 | NA | QPSK | 10 | P_0@0 | S_1@0 |  |  |
| 2 | 25 | 100 |  | QPSK | 10 | P_0@0 | S_1@9 |  |  |
| 3 | 25 | 100 |  | QPSK | 100 | P_0@0 | S_10@0 |  |  |
| 4 | 25 | 100 |  | 16QAM | 10 | P_0@0 | S_1@0 |  |  |
| 5 | 25 | 100 |  | 16QAM | 10 | P_0@0 | S_1@9 |  |  |
| 6 | 25 | 100 |  | 16QAM | 100 | P_0@0 | S_10@0 |  |  |
| 7 | 50 | 100 |  | QPSK | 10 | P_0@0 | S_1@0 |  |  |
| 8 | 50 | 100 |  | QPSK | 10 | P_0@0 | S_1@9 |  |  |
| 9 | 50 | 100 |  | QPSK | 100 | P_0@0 | S_10@0 |  |  |
| 10 | 50 | 100 |  | 16QAM | 10 | P_0@0 | S_1@0 |  |  |
| 11 | 50 | 100 |  | 16QAM | 10 | P_0@0 | S_1@9 |  |  |
| 12 | 50 | 100 |  | 16QAM | 100 | P_0@0 | S_10@0 |  |  |
| 13 | 75 | 100 |  | QPSK | 10 | P_0@0 | S_1@0 |  |  |
| 14 | 75 | 100 |  | QPSK | 10 | P_0@0 | S_1@9 |  |  |
| 15 | 75 | 100 |  | QPSK | 100 | P_0@0 | S_10@0 |  |  |
| 16 | 75 | 100 |  | 16QAM | 10 | P_0@0 | S_1@0 |  |  |
| 17 | 75 | 100 |  | 16QAM | 10 | P_0@0 | S_1@9 |  |  |
| 18 | 75 | 100 |  | 16QAM | 100 | P_0@0 | S_10@0 |  |  |
| 19 | 100 | 100 |  | QPSK | 10 | P_0@0 | S_1@0 |  |  |
| 20 | 100 | 100 |  | QPSK | 10 | P_0@0 | S_1@9 |  |  |
| 21 | 100 | 100 |  | QPSK | 100 | P_0@0 | S_10@0 |  |  |
| 22 | 100 | 100 |  | 16QAM | 10 | P_0@0 | S_1@0 |  |  |
| 23 | 100 | 100 |  | 16QAM | 10 | P_0@0 | S_1@9 |  |  |
| 24 | 100 | 100 |  | 16QAM | 100 | P_0@0 | S_10@0 |  |  |
| Note 1: CA Configuration, Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths for uplink LAA are specified in Table 5.4.2A.1-2. Note 2: For eLAA SCC, the RBstart and L are defined in TS 36.213[10] clause 8.1.4. |  |  |  |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A, Figure group A.32 as appropriate.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL RMC is set according to Table 6.5.2A.1.2.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.5.2A.1.2.4.3.

6.5.2A.1.2.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and Annex C.3.0 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 36.508 [7] clause 5.2A.4. Message contents are defined in clause 6.5.2A.3.2.4.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 36.321 [13], clauses 5.13, 6.1.3.8). Wait for at least 2 seconds (Refer TS 36.133, clauses 8.3.3.2).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 (or DCI format 0A for SCC in the case of uplink LAA) for C_RNTI to schedule the UL RMC according to Table 6.5.2A.1.2.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

6.	Measure the EVM and  on PCC using Global In-Channel Tx-Test (Annex E).

7.	Send the appropriate TPC commands in the uplink scheduling information to the UE until UE output power is –36.8dBm ±3.2dB for carrier frequency f ≤ 3.0GHz or -36.5dBm ±3.5 dB for carrier frequency 3.0GHz < f ≤ 4.2GHz.

8.	Measure the EVM and  on PCC using Global In-Channel Tx-Test (Annex E).

6.5.2A.1.2.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6. In test procedure step 2, for SCC configuration there are no additional message contents.

For SCC in the case of uplink LAA, message contents are according to TS 36.508[7] subclause4.6 with the following exceptions specified in Tables 6.5.2A.1.2.4.3-1 and 6.5.2A.1.2.4.3-2.

Table 6.5.2A.1.2.4.3-1 LogicalChannelConfig-DRB (Initial Condition, step 14, Table 4.5.2.3-1)

| Derivation Path: 36.508 Table 4.8.2.1.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| LogicalChannelConfig-DRB ::= SEQUENCE { |  |  |  |  |
| ul-SpecificParameters SEQUENCE { |  |  |  |  |
| laa-UL-Allowed-r14 |  | true |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 6.5.2A.1.2.4.3-2 PhysicalConfigDedicatedSCell (Step 2, test procedure)

| Derivation Path: 36.508 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10-DEFAULT ::= SEQUENCE { |  |  |  |
| laa-SCellConfiguration-v1430 ::= SEQUENCE { |  |  |  |
|  | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

6.5.2A.1.2.5	Test requirement

The PUSCH EVM derived in Annex E.4.2 shall not exceed 17,5 % for QPSK and BPSK, 12,5% for 16 QAM. The PUSCH derived in Annex E.4.6.2 shall not exceed 17,5 % when embedded with data symbols of QPSK and BPSK, 12,5% for 16 QAM.

< End of changes >