3GPP TSG-RAN WG5 Meeting #81	R5-187613r2

# Spoken, USA, 12th Nov 2018 - 16th Nov 2018

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 0093 | rev | - | Current version: | 15.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LTE Anchor Link configuration for FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2018-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - LTE configuration details for FR2 for anchor-agnostic approach is needed. - According to the agreement in R5-186315 UL open loop power control parameter is changed to attain maximum UL power from the UE(only outside the measurement period). - The anchor power level at RSRP reference point is proposed to band group dependent rather than determining it from worst REFSENS band among all bands to not to impose unnecessarily big link budget for TE and to extend the test coverage as much as possible. Power levels can be determined from REFSENS+5dB as per the agreement in R5-186315 - Other parameters are from agreement in R5-177455, except for UL BW, which can be 5MHz(Same as FR1) as it is supported by all the E-UTRA bands. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added section 4.7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing details in test spec and it will not be aligned with core spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R1 : Missing “≥” added to Table 4.7-3. Correct reference to tables. R2 : Typo corrected. |  |  |  |  |  |  |  |  |

## << Start of changes >>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

●	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

●	For a specific reference, subsequent revisions do not apply.

●	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

Editor’s note: intended to capture more references

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[3]	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”

[4]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

[5]	3GPP TS 36.101: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”

[6]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment"

[7]	3GPP TR 38.905: "NR; Derivation of test points for radio transmission and reception conformance test cases"

[8]	3GPP TS 38.521-1:” User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone

[9]	3GPP TS 38.521-2:” NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone”

[10]	3GPP TS 36.521-1:” Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance testing”

[11]	3GPP TS 36.508:” Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common test environments for User Equipment (UE) conformance testing”

[12]	3GPP TS 36.133:” Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management”

## << Unchnaged sections skipped >>

## 4.7	E-UTRA configuration for EN-DC FR2 tests applying the E-UTRA anchor-agnostic approach

This section applies to EN-DC test cases where E-UTRA anchor needs to be configured as per the anchor-agnostic approach outlined in section 6.1 and 7.1 of TS 38.101-3 [4].The LTE anchor-agnostic approach is defined as measurements on the NR carrier under conditions where the LTE anchor resources do not interfere with NR operation.The configuration defined in this section ensures establishment of such conditions.

For baseline configuration, the E-UTRA carrier will be configured for each test case in section 6 and 7 as defined in the equivalent standalone E-UTRA test in TS 36.521-1. However, the below exceptions defined in Table 4.7-1 to 4.7-7 are applied to ensure that the E-UTRA anchor resources do not interfere with NR operation

Table 4.7-1: E-UTRA configuration for EN-DC FR2 tests applying anchor agnostic approach

| Parameter | Value | Comments |
| --- | --- | --- |
| Test Frequency during and after connection setup |  |  | Mid (See Table 4.7-2) |  |  | As defined in TS 36.508 for the LTE band under test |  |  |
| Bandwidth during and after connection setup |  |  | 5 MHz (See Table 4.7-2) |  |  | Supported by all LTE bands. |  |  |
| DL signal levels |  |  | See table 4.7-3 |  |  | DL physical channels as defined in Annex C0, C.1, C.2 and Annex C.3 of TS 36.521-1 |  |  |
| UL Signal levels for connection setup and UBF transmisson | PUSCH Power | Attained by enabling open loop power control and setting up UL signal levels according to Annex H.0, H.2 and H.3 of TS 36.521-1 with the exception for power control message exception defined in Table 4.7-4 |
| DL/UL RMC after connection setup except for UBF transmission |  |  | 0 RB allocation on both DL and UL (see Table 4.7-2) |  |  | Once the LTE link is established, then LTE Tx can be restricted by configuring 0 RB allocation on DL and UL. TimeAlignmentTimerDedicated IE to be set to infinity to ensure UE doesn’t look for TA adjustments (See Table 4.7-7) |  |  |
| CQI Reports and SRS after connection setup | Disabled (See Table 4.7-4 and 4.7-6) | Disable periodic and aperiodic CQI reports to ensure none of these transmissions occur on the LTE uplink. Since LTE transmissions could easily exceed spurious emissions limits, tests that are intended to measure RF parametrics on the NR should simply avoid LTE transmit altogether. |

Table 4.7-2: E-UTRA Test Configuration Table

| E-UTRA Test Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| E-UTRA Channel Bandwidth | E-UTRA Test Frequency | Downlink |  | Uplink |  |
|  |  | Modulation | RB allocation | Modulation | RB allocation |
| 5 MHz2 | MidRange1 | N/A | 0 | N/A | 0 |
| NOTE 1: E-UTRA Test Frequency as specified in TS 36.508 [11] subclause 4.3.1 NOTE 2: For EN-DC Intra-band tests that need to apply E-UTRA anchor agnostic approach, refer to and pick applicable E-UTRA channel bandwidth from subclause 5.3B.1 and indicate within test case if it is different than 5 MHz. |  |  |  |  |  |

Table 4.7-3: Default Downlink power levels for E-UTRA anchor

|  | Unit | Band Group | Channel Bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5MHz | 10MHz | 15 MHz | 20 MHz |
| RS EPRE | dBm/15kHz | FDD_A, TDD_A | N/A | N/A | ≥ -120.0 | N/A | N/A | N/A |
|  |  | FDD_B1, TDD_B1 | N/A | N/A | ≥ -119.5 | N/A | N/A | N/A |
|  |  | FDD_C, TDD_C | N/A | N/A | ≥ -119.0 | N/A | N/A | N/A |
|  |  | FDD_D, TDD_D | N/A | N/A | ≥ -118.5 | N/A | N/A | N/A |
|  |  | FDD_E, TDD_E | N/A | N/A | ≥ -118.0 | N/A | N/A | N/A |
|  |  | FDD_G, TDD_G | N/A | N/A | ≥ -117.0 | N/A | N/A | N/A |
|  |  | FDD_H, TDD_H | N/A | N/A | ≥ -116.5 | N/A | N/A | N/A |
|  |  | FDD_N, TDD_N | N/A | N/A | ≥ -113.5 | N/A | N/A | N/A |
| Note 1: The power level is specified at RSRP reference point as defined in TS 36.214 [21] Note 2: E-UTRA Band groups are defined in TS 36.133 [12] clause 3.5.1. |  |  |  |  |  |  |  |  |

Table 4.7-4 -CQI-ReportConfig-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.6.3, Table 4.6.3-2 CQI-ReportConfig-DEFAULT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CQI-ReportConfig-DEFAULT ::= SEQUENCE { |  |  |  |  |
| cqi-ReportModeAperiodic |  | NOT PRESENT |  |  |
| cqi-ReportPeriodic |  | NOT PRESENT |  |  |
| } |  |  |  |  |

Table 4.7-5: UplinkPowerControlCommon-DEFAULT : Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.6.3, UplinkPowerControlCommon-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-NominalPUSCH | 24 (24 dBm) | To attain maximum power from the DUT |  |
| } |  |  |  |

Table 4.7-6: PhysicalConfigDedicated-DEFAULT: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2, Table 4.8.2.1.6-1 PhysicalConfigDedicated-DEFAULT |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |  |
| soundingRS-UL-ConfigDedicated |  | Not present |  | RBC |
| } |  |  |  |  |

Table 4.7-7: MAC-MainConfig-RBC: Additional E-UTRA Anchor Configuration

| Derivation Path: TS 36.508 [7] clause 4.8.2.1.5, Table 4.8.2.1.5-1 MAC-MainConfig-RBC |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| timeAlignmentTimerDedicated |  | Infinity |  |  |