3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210087

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1521 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC test case 7.1.1.3.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At step 13-17 we are not saving the recieved NR_DRB_COMMON_IND and not updating v_TimingDRB. V_DurationSysInfo is calculated using timing from the previous indication, causing test case failure at step 16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Store timing of reieved NR_DRB_COMMON_IND at step 13-17. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A confirmant UE will fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.3.ENDC, 7.1.1.3.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_7_1_1_3_3_NR_TestBody |
| --- | --- |
| Reason for change | At step 13-17 we are not saving the recieved NR_DRB_COMMON_IND and not updating v_TimingDRB. V_DurationSysInfo is calculated using timing from the previous indication, causing test case failure at step 16. |
| Summary of change | Store timing of reieved NR_DRB_COMMON_IND at step 13-17. |
| TTCN module | MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before change

function f_TC_7_1_1_3_3_NR_TestBody(DRB_Identity p_NR_DRB_Id1,

DRB_Identity p_NR_DRB_Id2) runs on NR_BASE_PTC

{

var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id);

//@sic R5s201617 sic@

var NR_ResourceAllocation_Type v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(208, v_NR_UplinkBWP);

var SubFrameTiming_Type v_TimingDL;

var SubFrameTiming_Type v_TimingUL;

var NR_SYSTEM_IND v_Received_NR_SysInd;

var integer v_SR_Count := 0;

var integer v_Duration;

var boolean v_Flag := false;

var boolean v_PRACH_Flag := false;

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1);

var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters);

//@sic R5s200747 sic@

var SubFrameTiming_Type v_TimingDRB;

var SubFrameTiming_Type v_TimingSysInd;

//var SubFrameTiming_Type v_TimingPRACH;

var integer v_DurationSysInd;

//var integer v_DurationPRACH;

var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND;

timer t_Watchdog := 2.0;

// Enable SR reception indication

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable));

// Enable PRACH reception indication

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable));

// configure CBRA CRNTI Based RACH procedure

f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure);

// Stop UL Grant configuration

f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep);

//@siclog "Step 1" siclog@

//3 B PDCP H + 3 Byte RLC H + 9B PDCP SDU = 12Byte RLC SDU, 15 B RLC PDU +2 Byte MAC SH =17 Byte MAC Sub PDU = 136 bit, allowed TB size

v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL

DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1,

p_NR_DRB_Id2,

//crs_RlcBearerRouting_NR(nr_Cell1),

cs_TimingInfo_NR(v_TimingDL),

{crs_NR_PDCP_SDU_9B} ));

//@siclog "Step 4" siclog@

//@sic R5s190933 sic@

v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 100 +510); // 100 ms later; 510 for expiry of SR-Delay timer

f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation.Imcs,  cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs)); //32 bits

// @siclog "Step 2-5" siclog@

v_Flag := false;

v_SR_Count := 0;

t_Watchdog.start;

alt {

[v_SR_Count == 0] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) -> value v_Received_NR_SysInd

{

v_TimingUL := v_Received_NR_SysInd.Common.TimingInfo.SubFrame;

v_Duration := f_SubFrameTiming_Duration(v_TimingDL, v_TimingUL);

if (f_NR_SubFrameTimingCheckDuration(nr_Cell1, l2Timer, v_Duration, 520)) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2, 3");

}

else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2");

}

v_SR_Count := 1;

repeat;

}

[not v_Flag] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?))

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3");

repeat;

}

[not v_Flag ]  DRB.receive (car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id2, -, {crs_NR_PDCP_SDU_9B}))-> value v_NR_DRB_COMMON_IND

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5");

v_TimingDRB := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame;

v_Flag := true;

repeat;

}

[v_Flag] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?))-> value v_Received_NR_SysInd

{

v_TimingSysInd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame;

v_DurationSysInd := f_SubFrameTiming_Duration(v_TimingSysInd, v_TimingDRB);

if (v_DurationSysInd < 0)

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3");

}

t_Watchdog.stop;

}

[]t_Watchdog.timeout

{

if ( v_SR_Count >= 1 and v_Flag){

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3,5");

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3,5");

}

};

};

// Stop UL Grant configuration

//@sic r5s190933

f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep);

// @siclog "Step 6" siclog@

//3 B PDCP H + 3 Byte RLC H + 9B PDCP SDU = 12Byte RLC SDU, 15 B RLC PDU +2 Byte MAC SH =17 Byte MAC Sub PDU = 136 bit, allowed TB size

v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL

DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1,

p_NR_DRB_Id1,

//crs_RlcBearerRouting_NR(nr_Cell1),

cs_TimingInfo_NR(v_TimingDL),

{crs_NR_PDCP_SDU_9B} ));

//@siclog "Step 8" siclog@

v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 100); // 100 ms later

f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation.Imcs,  cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs)); //32 bits

v_Flag := false;

v_SR_Count := 0;

// @siclog "Step 7-10" siclog@

t_Watchdog.start(1.2);

alt {

[v_SR_Count == 0] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) -> value v_Received_NR_SysInd

{

v_TimingUL := v_Received_NR_SysInd.Common.TimingInfo.SubFrame;

v_Duration := f_SubFrameTiming_Duration(v_TimingDL, v_TimingUL);

if (v_Duration < 100) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7");

}

else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 7");

}

v_SR_Count := 1;

repeat;

}

[not v_Flag] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?))

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7");

v_SR_Count := v_SR_Count + 1;

repeat;

}

[ v_Flag] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?))-> value v_Received_NR_SysInd

{

v_TimingSysInd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame;

v_DurationSysInd := f_SubFrameTiming_Duration(v_TimingSysInd, v_TimingDRB);

if (v_DurationSysInd < 0)

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10");

t_Watchdog.stop;

} else {  repeat;  }

}

[not v_Flag]  DRB.receive (car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id1, -, {crs_NR_PDCP_SDU_9B}))-> value v_NR_DRB_COMMON_IND

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9");

v_TimingDRB := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame;

v_Flag := true;

repeat;

}

[] t_Watchdog.timeout

{

if ( v_SR_Count >= 1 and v_Flag){

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10");

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10");

}

}

};

// Stop SS to send UL grant and stop Time alignement transmission

//@sic R5s190933 sic@

f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep);

//@siclog "Step 12" siclog@

DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1,

p_NR_DRB_Id1,

-,

{crs_NR_PDCP_SDU_9B} ));

//@siclog "Step 13-17" siclog@

v_Flag := false;

v_PRACH_Flag := false;

t_Watchdog.start;

alt {

[not v_PRACH_Flag] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?))

{

repeat;

}

[v_PRACH_Flag] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?))

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13");

}

[not v_Flag] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1,-, ?))

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13");

v_PRACH_Flag := true;

repeat;

}

[v_Flag] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1,-, ?))-> value v_Received_NR_SysInd

{

v_TimingSysInd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame;

v_DurationSysInd := f_SubFrameTiming_Duration(v_TimingSysInd, v_TimingDRB);

if (v_DurationSysInd < 0)

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13");

}

t_Watchdog.stop;

}

[not v_Flag]  DRB.receive (car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id1, -, {crs_NR_PDCP_SDU_9B}))

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16");

v_Flag := true;

}

[]t_Watchdog.timeout

{

if ( v_PRACH_Flag and v_Flag){

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13,16");

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13,16");

}

};

};

// Disable SR reception indication

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable));

// disable PRACH reception indication

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable));

}

After change

function f_TC_7_1_1_3_3_NR_TestBody(DRB_Identity p_NR_DRB_Id1,

DRB_Identity p_NR_DRB_Id2) runs on NR_BASE_PTC

{

var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id);

//@sic R5s201617 sic@

var NR_ResourceAllocation_Type v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(208, v_NR_UplinkBWP);

var SubFrameTiming_Type v_TimingDL;

var SubFrameTiming_Type v_TimingUL;

var NR_SYSTEM_IND v_Received_NR_SysInd;

var integer v_SR_Count := 0;

var integer v_Duration;

var boolean v_Flag := false;

var boolean v_PRACH_Flag := false;

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1);

var template(value) NR_RachProcedureConfig_Type v_NR_RachProcedure := cs_NR_RachProcedureConfig_CBRA_CrntiBased(v_NR_PhysicalParameters);

//@sic R5s200747 sic@

var SubFrameTiming_Type v_TimingDRB;

var SubFrameTiming_Type v_TimingSysInd;

//var SubFrameTiming_Type v_TimingPRACH;

var integer v_DurationSysInd;

//var integer v_DurationPRACH;

var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND;

timer t_Watchdog := 2.0;

// Enable SR reception indication

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable));

// Enable PRACH reception indication

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable));

// configure CBRA CRNTI Based RACH procedure

f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure);

// Stop UL Grant configuration

f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep);

//@siclog "Step 1" siclog@

//3 B PDCP H + 3 Byte RLC H + 9B PDCP SDU = 12Byte RLC SDU, 15 B RLC PDU +2 Byte MAC SH =17 Byte MAC Sub PDU = 136 bit, allowed TB size

v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL

DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1,

p_NR_DRB_Id2,

//crs_RlcBearerRouting_NR(nr_Cell1),

cs_TimingInfo_NR(v_TimingDL),

{crs_NR_PDCP_SDU_9B} ));

//@siclog "Step 4" siclog@

//@sic R5s190933 sic@

v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 100 +510); // 100 ms later; 510 for expiry of SR-Delay timer

f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation.Imcs,  cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs)); //32 bits

// @siclog "Step 2-5" siclog@

v_Flag := false;

v_SR_Count := 0;

t_Watchdog.start;

alt {

[v_SR_Count == 0] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) -> value v_Received_NR_SysInd

{

v_TimingUL := v_Received_NR_SysInd.Common.TimingInfo.SubFrame;

v_Duration := f_SubFrameTiming_Duration(v_TimingDL, v_TimingUL);

if (f_NR_SubFrameTimingCheckDuration(nr_Cell1, l2Timer, v_Duration, 520)) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2, 3");

}

else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2");

}

v_SR_Count := 1;

repeat;

}

[not v_Flag] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?))

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3");

repeat;

}

[not v_Flag ]  DRB.receive (car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id2, -, {crs_NR_PDCP_SDU_9B}))-> value v_NR_DRB_COMMON_IND

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5");

v_TimingDRB := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame;

v_Flag := true;

repeat;

}

[v_Flag] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?))-> value v_Received_NR_SysInd

{

v_TimingSysInd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame;

v_DurationSysInd := f_SubFrameTiming_Duration(v_TimingSysInd, v_TimingDRB);

if (v_DurationSysInd < 0)

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3");

}

t_Watchdog.stop;

}

[]t_Watchdog.timeout

{

if ( v_SR_Count >= 1 and v_Flag){

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3,5");

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3,5");

}

};

};

// Stop UL Grant configuration

//@sic r5s190933

f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep);

// @siclog "Step 6" siclog@

//3 B PDCP H + 3 Byte RLC H + 9B PDCP SDU = 12Byte RLC SDU, 15 B RLC PDU +2 Byte MAC SH =17 Byte MAC Sub PDU = 136 bit, allowed TB size

v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL

DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1,

p_NR_DRB_Id1,

//crs_RlcBearerRouting_NR(nr_Cell1),

cs_TimingInfo_NR(v_TimingDL),

{crs_NR_PDCP_SDU_9B} ));

//@siclog "Step 8" siclog@

v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 100); // 100 ms later

f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation.Imcs,  cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs)); //32 bits

v_Flag := false;

v_SR_Count := 0;

// @siclog "Step 7-10" siclog@

t_Watchdog.start(1.2);

alt {

[v_SR_Count == 0] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) -> value v_Received_NR_SysInd

{

v_TimingUL := v_Received_NR_SysInd.Common.TimingInfo.SubFrame;

v_Duration := f_SubFrameTiming_Duration(v_TimingDL, v_TimingUL);

if (v_Duration < 100) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7");

}

else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 7");

}

v_SR_Count := 1;

repeat;

}

[not v_Flag] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?))

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7");

v_SR_Count := v_SR_Count + 1;

repeat;

}

[ v_Flag] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?))-> value v_Received_NR_SysInd

{

v_TimingSysInd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame;

v_DurationSysInd := f_SubFrameTiming_Duration(v_TimingSysInd, v_TimingDRB);

if (v_DurationSysInd < 0)

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10");

t_Watchdog.stop;

} else {  repeat;  }

}

[not v_Flag]  DRB.receive (car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id1, -, {crs_NR_PDCP_SDU_9B}))-> value v_NR_DRB_COMMON_IND

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9");

v_TimingDRB := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame;

v_Flag := true;

repeat;

}

[] t_Watchdog.timeout

{

if ( v_SR_Count >= 1 and v_Flag){

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10");

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10");

}

}

};

// Stop SS to send UL grant and stop Time alignement transmission

//@sic R5s190933 sic@

f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep);

//@siclog "Step 12" siclog@

DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1,

p_NR_DRB_Id1,

-,

{crs_NR_PDCP_SDU_9B} ));

//@siclog "Step 13-17" siclog@

v_Flag := false;

v_PRACH_Flag := false;

t_Watchdog.start;

alt {

[not v_PRACH_Flag] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?))

{

repeat;

}

[v_PRACH_Flag] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?))

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13");

}

[not v_Flag] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1,-, ?))

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13");

v_PRACH_Flag := true;

repeat;

}

[v_Flag] SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1,-, ?))-> value v_Received_NR_SysInd

{

v_TimingSysInd := v_Received_NR_SysInd.Common.TimingInfo.SubFrame;

v_DurationSysInd := f_SubFrameTiming_Duration(v_TimingSysInd, v_TimingDRB);

if (v_DurationSysInd < 0)

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13");

}

t_Watchdog.stop;

}

[not v_Flag]  DRB.receive (car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id1, -, {crs_NR_PDCP_SDU_9B})) -> value v_NR_DRB_COMMON_IND

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16");

v_TimingDRB := v_NR_DRB_COMMON_IND.Common.TimingInfo.SubFrame;

v_Flag := true;

repeat;

}

[]t_Watchdog.timeout

{

if ( v_PRACH_Flag and v_Flag){

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 13,16");

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 13,16");

}

};

};

// Disable SR reception indication

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable));

// disable PRACH reception indication

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable));

}