3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220254

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2261 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR testcase 8.1.3.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-12-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In 21wk49 Implementation at steps 1-2 of the function : f_TC_8_1_3_2_3_TestBody measurement interval is made as 120ms as proposed in R5-218335 .But the same is not taken care at step 5 when receiving the measurement report to check for the measurement interval .. this has to be corrected |  |  |  |  |  |  |  |  |
| Summary of change: |  | Measurement report interval check while receiving the measurement report is corrected at step 5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function f_TC_8_1_3_2_3_TestBody()	2

# Corrections required

## Correction to function f_TC_8_1_3_2_3_TestBody()

| Function name | f_TC_8_1_3_2_3_TestBody() |
| --- | --- |
| Reason for change | In 21wk49 Implementation at steps 1-2 of the function : f_TC_8_1_3_2_3_TestBody measurement interval is made as 120ms as proposed in R5-218335 .But the same is not taken care at step 5 when receiving the measurement report to check for the measurement interval .. this has to be corrected |
| Summary of change | Measurement report interval check while receiving the measurement report is corrected at step 5 |
| TTCN module | NR5GC_IRAT\RRC_Measurement_NR5GC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_8_1_3_2_3_TestBody() runs on NR_BASE_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var NR_AbsoluteCellPower_Type v_CellPower_T0_FR1 := -88; var NR_AbsoluteCellPower_Type v_CellPower_T1_FR1 := -88; //@sic R5s200725 sic@ var integer v_Noc := -94; var PhysCellId v_PhysCellId; var ARFCN_ValueEUTRA v_ARFCN_ValueEUTRA; var EUTRA_PhysCellId v_EUTRA_CellId; var integer v_MeasurementBandwidth_int; var EUTRA_AllowedMeasBandwidth v_MeasurementBandwidth; var Hysteresis v_Hysteresis := 0; var EventTriggerConfigInterRAT.reportAmount v_ReportAmount := r2; //@sic R5s200725 sic@ var RSRQ_Range v_B2_Threshold1 := 79; //@sic R5s200725 sic@ var RSRQ_RangeEUTRA v_B2_Threshold2EUTRA := 23; //@sic R5s200725 sic@ var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList_NR; var template (value) MeasObjectParamsList_Type_EUTRA v_MeasuredObjectList_EUTRA; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; //Power levels of NR cell 1: Table 8.1.3.2.3.3.2-1, Table 8.1.3.2.3.3.2-2 /* ¦ FR1 ¦ FR2 * ------------------------------- * T0 ¦ -88 ¦ FFS * ------------------------------- * T1 ¦ -94 ¦ FFS * */ v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo (EUTRA); v_ARFCN_ValueEUTRA := v_CoOrd_SysInfo.Eutra[0].Arfcn; v_EUTRA_CellId := bit2int (v_CoOrd_SysInfo.Eutra[0].CellId); v_MeasurementBandwidth_int := v_CoOrd_SysInfo.Eutra[0].DL_Bandwidth; v_MeasurementBandwidth := f_NR5GC_IRAT_ConvertIntegerToEUTRA_AllowedMeasBandwidth(v_MeasurementBandwidth_int); v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, v_CellPower_T0_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, v_CellPower_T1_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; f_NR_SetCellPowerList(v_CellPowerList_AtT0); // Activate Noise Generator for NR Cell1 f_NR_ConfigureAndActivateVNG_MultipleCells({cs_CellNoiseLevel(nr_Cell1, v_Noc)}); //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell1 including MeasConfig to setup inter-RAT measurement and reporting for event B2 //The UE transmits an RRCReconfigrationComplete message v_MeasuredObjectList_NR := {cs_MeasObjectId1(nr_Cell1)}; v_MeasuredObjectList_EUTRA := {cs_MeasObjectId2_EUTRA(v_EUTRA_CellId, v_ARFCN_ValueEUTRA)}; v_ReportConfigList := {cs_NR_ReportConfig_reportConfigInterRAT(tsc_NR_IdReportConfigId1, cs_NR_ReportConfigInterRAT_Trigger_eventB2_EUTRA_Rsrq(v_B2_Threshold1, v_B2_Threshold2EUTRA, v_Hysteresis, v_ReportAmount, true, false, ms120))}; //@sic R5-218335, R5-203359, R5s201111 sic@ v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj2_conf1}; //v_MeasConfig := f_NR_GenerateMeasurementConfigNR_EUTRA(v_ARFCN_ValueEUTRA, v_MeasurementBandwidth, true, v_MeasuredObjectList_NR, v_MeasuredObjectList_EUTRA, v_ReportConfigList, v_MeasIdConfigList, cs_MeasGapConfig_FR1, cs_38508_QuantityConfig_EUTRA); v_MeasConfig := f_NR_GenerateMeasurementConfigNR_EUTRA(v_ARFCN_ValueEUTRA, v_MeasurementBandwidth, -, v_MeasuredObjectList_NR, v_MeasuredObjectList_EUTRA, v_ReportConfigList, v_MeasIdConfigList, cs_38508_MeasGapConfig_UE(cs_NR_GapConfig(0, ms6, ms40, ms0)), cs_38508_QuantityConfig_EUTRA); //v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList); //f_NR_SendRRCReconfiguration(nr_Cell1,-,-,-,-, v_MeasConfig); f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1,v_MeasConfig); //@sic R5-198871 sic@ //@siclog "Steps 3" siclog@ //Check: Does the UE transmit a MeasurementReport message on NR Cell 1 within the next 10s? f_NR_Check_NoMeasurementReport(10.0, nr_Cell1, cr_NR_MeasurementReport_Any); //@siclog "Step 4" siclog@ //SS re-adjusts the cell-specific reference signal level according to row "T1" in Table 8.1.3.2.3.3.2-1/2. f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); //@sic R5s200543 sic@ f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); //@sic R5s200543 sic@ f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 5" siclog@ //Check: Does the UE transmit a MeasurementReport message on Cell 1 to report event B2 with the measured RSRP and RSRQ values for E-UTRA Cell 1? v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellId, tsc_NR_MeasId1, 2, 48.0, cr_NR_MeasResults_MeasResultNeighCells_EUTRA({cr_NR_MeasResultEUTRA_CellResults(v_EUTRA_CellId, cr_NR_MeasQuantityResultsEUTRA_Rsrp_Rsrq)})); //@sic R5-204437, R5s201111, R5s200725, R5-201097, R5-211389 sic@ // Deactivate Noise Generator for NR Cell1 f_NR_DeactivateVNG_MultipleCells(f_NR_GetCellIdListWithActiveVNG({cs_CellNoiseLevel(nr_Cell1, v_Noc)})); }//End of f_TC_8_1_3_2_3_TestBody()

After Change:

function f_TC_8_1_3_2_3_TestBody() runs on NR_BASE_PTC { var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var NR_AbsoluteCellPower_Type v_CellPower_T0_FR1 := -88; var NR_AbsoluteCellPower_Type v_CellPower_T1_FR1 := -88; //@sic R5s200725 sic@ var integer v_Noc := -94; var PhysCellId v_PhysCellId; var ARFCN_ValueEUTRA v_ARFCN_ValueEUTRA; var EUTRA_PhysCellId v_EUTRA_CellId; var integer v_MeasurementBandwidth_int; var EUTRA_AllowedMeasBandwidth v_MeasurementBandwidth; var Hysteresis v_Hysteresis := 0; var EventTriggerConfigInterRAT.reportAmount v_ReportAmount := r2; //@sic R5s200725 sic@ var RSRQ_Range v_B2_Threshold1 := 79; //@sic R5s200725 sic@ var RSRQ_RangeEUTRA v_B2_Threshold2EUTRA := 23; //@sic R5s200725 sic@ var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo; var template (value) MeasObjectParamsList_Type v_MeasuredObjectList_NR; var template (value) MeasObjectParamsList_Type_EUTRA v_MeasuredObjectList_EUTRA; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (omit) MeasConfig v_MeasConfig; //Power levels of NR cell 1: Table 8.1.3.2.3.3.2-1, Table 8.1.3.2.3.3.2-2 /* ¦ FR1 ¦ FR2 * ------------------------------- * T0 ¦ -88 ¦ FFS * ------------------------------- * T1 ¦ -94 ¦ FFS * */ v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo (EUTRA); v_ARFCN_ValueEUTRA := v_CoOrd_SysInfo.Eutra[0].Arfcn; v_EUTRA_CellId := bit2int (v_CoOrd_SysInfo.Eutra[0].CellId); v_MeasurementBandwidth_int := v_CoOrd_SysInfo.Eutra[0].DL_Bandwidth; v_MeasurementBandwidth := f_NR5GC_IRAT_ConvertIntegerToEUTRA_AllowedMeasBandwidth(v_MeasurementBandwidth_int); v_CellPowerList_AtT0 := {cs_NR_CellPower(nr_Cell1, v_CellPower_T0_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; v_CellPowerList_AtT1 := {cs_NR_CellPower(nr_Cell1, v_CellPower_T1_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE)}; f_NR_SetCellPowerList(v_CellPowerList_AtT0); // Activate Noise Generator for NR Cell1 f_NR_ConfigureAndActivateVNG_MultipleCells({cs_CellNoiseLevel(nr_Cell1, v_Noc)}); //@siclog "Steps 1-2" siclog@ //The SS transmits an RRCReconfiguration message on NR Cell1 including MeasConfig to setup inter-RAT measurement and reporting for event B2 //The UE transmits an RRCReconfigrationComplete message v_MeasuredObjectList_NR := {cs_MeasObjectId1(nr_Cell1)}; v_MeasuredObjectList_EUTRA := {cs_MeasObjectId2_EUTRA(v_EUTRA_CellId, v_ARFCN_ValueEUTRA)}; v_ReportConfigList := {cs_NR_ReportConfig_reportConfigInterRAT(tsc_NR_IdReportConfigId1, cs_NR_ReportConfigInterRAT_Trigger_eventB2_EUTRA_Rsrq(v_B2_Threshold1, v_B2_Threshold2EUTRA, v_Hysteresis, v_ReportAmount, true, false, ms120))}; //@sic R5-218335, R5-203359, R5s201111 sic@ v_MeasIdConfigList := {cs_NR_MeasId_Config_id1_obj2_conf1}; //v_MeasConfig := f_NR_GenerateMeasurementConfigNR_EUTRA(v_ARFCN_ValueEUTRA, v_MeasurementBandwidth, true, v_MeasuredObjectList_NR, v_MeasuredObjectList_EUTRA, v_ReportConfigList, v_MeasIdConfigList, cs_MeasGapConfig_FR1, cs_38508_QuantityConfig_EUTRA); v_MeasConfig := f_NR_GenerateMeasurementConfigNR_EUTRA(v_ARFCN_ValueEUTRA, v_MeasurementBandwidth, -, v_MeasuredObjectList_NR, v_MeasuredObjectList_EUTRA, v_ReportConfigList, v_MeasIdConfigList, cs_38508_MeasGapConfig_UE(cs_NR_GapConfig(0, ms6, ms40, ms0)), cs_38508_QuantityConfig_EUTRA); //v_MeasConfig :=f_NR_GenerateMeasurementConfigNR(v_MeasuredObjectList, v_ReportConfigList, v_MeasIdConfigList); //f_NR_SendRRCReconfiguration(nr_Cell1,-,-,-,-, v_MeasConfig); f_NR_SendRRCReconfiguration_MeasNR(nr_Cell1,v_MeasConfig); //@sic R5-198871 sic@ //@siclog "Steps 3" siclog@ //Check: Does the UE transmit a MeasurementReport message on NR Cell 1 within the next 10s? f_NR_Check_NoMeasurementReport(10.0, nr_Cell1, cr_NR_MeasurementReport_Any); //@siclog "Step 4" siclog@ //SS re-adjusts the cell-specific reference signal level according to row "T1" in Table 8.1.3.2.3.3.2-1/2. f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); //@sic R5s200543 sic@ f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); //@sic R5s200543 sic@ f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 5" siclog@ //Check: Does the UE transmit a MeasurementReport message on Cell 1 to report event B2 with the measured RSRP and RSRQ values for E-UTRA Cell 1? v_PhysCellId := f_NR_CellInfo_GetPhysicalCellId(nr_Cell1); f_NR_ReceiveMeasurementReports(nr_Cell1, v_PhysCellId, tsc_NR_MeasId1, 2, 12.0, cr_NR_MeasResults_MeasResultNeighCells_EUTRA({cr_NR_MeasResultEUTRA_CellResults(v_EUTRA_CellId, cr_NR_MeasQuantityResultsEUTRA_Rsrp_Rsrq)})); //@sic R5-204437, R5s201111, R5s200725, R5-201097, R5-211389 sic@ // Deactivate Noise Generator for NR Cell1 f_NR_DeactivateVNG_MultipleCells(f_NR_GetCellIdListWithActiveVNG({cs_CellNoiseLevel(nr_Cell1, v_Noc)})); }//End of f_TC_8_1_3_2_3_TestBody()