3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220474

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2450 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR MAC test case 7.1.1.1.1a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to 38.321 Table 7.4-1 , PRACH occasion index starts from 1, and PRACH Mask Index = 9 means “Every even PRACH occasion” For the test case 7.1.1.1.1a, only one PRACH occasion is configured. So PRACH Mask Index = 9 is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change PRACH Mask Index to 0 (“All PRACH occasions”) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.1a.ENDC, 7.1.1.1.1a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# -Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | cs_NR_PDCCH_Order |
| --- | --- |
| Reason for change | According to 38.321 Table 7.4-1 , PRACH occasion index starts from 1, and PRACH Mask Index = 9 means “Every even PRACH occasion” For the test case 7.1.1.1.1a, only one PRACH occasion is configured. So PRACH Mask Index = 9 is incorrect. |
| Summary of change | Change PRACH Mask Index to 0 (“All PRACH occasions”) |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment |  |

Before Change

template (value) NR_PDCCH_Order_Type cs_NR_PDCCH_Order(SSB_Index p_SSB_Index, integer p_PreambleIndex := tsc_PreambleIndex) := { /* @status APPROVED (ENDC, NR5GC) */ RA_PreambleIndex := int2bit (p_PreambleIndex,6), /* tsc_PreambleIndex randomly selected */ UL_SUL_Indicator := { None := true}, /* indicates which UL carrier in the cell to transmit the PRACH if the UE is configured with SUL in the cell and RA_PreambleIndex != '000000'B; "None" otherwise */ SSB_Index := int2bit (p_SSB_Index,6), /* @sic R5-201148 sic@*/ PrachMaskIndex := int2bit (9,4) /* Every even PRACH occasion */ };

After Change

template (value) NR_PDCCH_Order_Type cs_NR_PDCCH_Order(SSB_Index p_SSB_Index, integer p_PreambleIndex := tsc_PreambleIndex) := { /* @status APPROVED (ENDC, NR5GC) */ RA_PreambleIndex := int2bit (p_PreambleIndex,6), /* tsc_PreambleIndex randomly selected */ UL_SUL_Indicator := { None := true}, /* indicates which UL carrier in the cell to transmit the PRACH if the UE is configured with SUL in the cell and RA_PreambleIndex != '000000'B; "None" otherwise */ SSB_Index := int2bit (p_SSB_Index,6), /* @sic R5-201148 sic@*/ PrachMaskIndex := int2bit (0,4) /* all PRACH occasion */ };