3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s220807

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2618 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for function f_NR5GC_RRC_Idle_Steps16_20() |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-06-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The parameter p_ReleaseConnection is currently not passed on to the function f_NR5GC_PDUSessionEstablishment(). Note : Because of this the parameter p_ConnectionWillBeReleased in f_NR5GC_PDUSessionEstablishment() will always default to rrcConnectionRelease, which means the function f_NR_DelayForUserPlaneSignalling() is always called so the 10s timer t_WaitForUPlaneSignalling is started. For e.g. 34.229-5 test cases where the test case will starte executing on IMS PTC as soon as IMS signalling begins, this timer will block IPCAN requests (e.g.IPCAN_Release) for 10s. This timer should not be started in the case where the calling function (e.g. f_NR5GC_IPCAN_IdleUpdated()) specifies “noRrcConnectionRelease” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Pass the parameter p_ReleaseConnection through to _NR5GC_PDUSessionEstablishment() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | f_NR5GC_RRC_Idle_Steps16_20() |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D12wk23 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | f_NR5GC_RRC_Idle_Steps16_20() |
| --- | --- |
| Reason for change | The parameter p_ReleaseConnection is currently not passed on to the function f_NR5GC_PDUSessionEstablishment(). Note : Because of this the parameter p_ConnectionWillBeReleased in f_NR5GC_PDUSessionEstablishment() will always default to rrcConnectionRelease, which means the function f_NR_DelayForUserPlaneSignalling() is always called so the 10s timer t_WaitForUPlaneSignalling is started. For e.g. 34.229-5 test cases where the test case will starte executing on IMS PTC as soon as IMS signalling begins, this timer will block IPCAN requests (e.g.IPCAN_Release) for 10s. This timer should not be started in the case where the calling function (e.g. f_NR5GC_IPCAN_IdleUpdated()) specifies “noRrcConnectionRelease” |
| Summary of change | Pass the parameter p_ReleaseConnection through to _NR5GC_PDUSessionEstablishment() |
| TTCN module | NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before Change

function f_NR5GC_RRC_Idle_Steps16_20(NR_CellId_Type p_NR_CellId, integer p_ExpectedNumberOfNewPDUSessions := pc_noOf_PDUsSameConnection, // @sic R5s201358 sic@, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, RRCConnectionReleaseRequired_Type p_ReleaseConnection := noRrcConnectionRelease, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (omit) SDAP_Config p_SDAP_Config := omit, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) B4_Type p_ServiceType := '0000'B) runs on NR5GC_PTC { //Steps 16 - 19a1 // No SRB2 so must be on SRB1 if (p_ExpectedNumberOfNewPDUSessions < 0 ) { // @sic R5-194797, R5-199075, R5-204329 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "3GPP RAN5 5GS test specifications do not support UEs configured with pc_noOf_PDUsSameConnection // @sic R5s201358 sic@ < 0"); } f_NR5GC_PDUSessionEstablishment (p_NR_CellId, p_ExpectedNumberOfNewPDUSessions, tsc_NR_RbId_SRB1, p_SDAP_Config, // @sic R5s210134 sic@ p_ExpectedS_NSSAI, p_ExpectedDNN, p_SSC_Mode, p_DNN, p_QoS_Rules, p_Session_AMBR, p_S_NSSAI, p_QoSFlowDescr); …………………………. }

After Change

function f_NR5GC_RRC_Idle_Steps16_20(NR_CellId_Type p_NR_CellId, integer p_ExpectedNumberOfNewPDUSessions := pc_noOf_PDUsSameConnection, // @sic R5s201358 sic@, NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF, RRCConnectionReleaseRequired_Type p_ReleaseConnection := noRrcConnectionRelease, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (omit) SDAP_Config p_SDAP_Config := omit, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) B4_Type p_ServiceType := '0000'B) runs on NR5GC_PTC { //Steps 16 - 19a1 // No SRB2 so must be on SRB1 if (p_ExpectedNumberOfNewPDUSessions < 0 ) { // @sic R5-194797, R5-199075, R5-204329 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "3GPP RAN5 5GS test specifications do not support UEs configured with pc_noOf_PDUsSameConnection // @sic R5s201358 sic@ < 0"); } f_NR5GC_PDUSessionEstablishment (p_NR_CellId, p_ExpectedNumberOfNewPDUSessions, tsc_NR_RbId_SRB1, p_SDAP_Config, // @sic R5s210134 sic@ p_ExpectedS_NSSAI, p_ExpectedDNN, p_SSC_Mode, p_DNN, p_QoS_Rules, p_Session_AMBR, p_S_NSSAI, p_QoSFlowDescr,-,-,-,-,-,-,-,-,-,-,-,-,-,-,-,-,-,-,-,-,-,-,p_ReleaseConnection); ………………….. }