3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221265

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2852 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_GetDefaultValueNetworkFeatureSupport , f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq and f_EUTRA38_NR5GC_Preamble |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the current implementation of the function f_EUTRA38_NR5GC_Preamble, the IE NetworkFeatureSupport is set with template cs_EPS_NwkFtSup_NR as default. This template hardcodes the spare bits 6,7 and 8 in Octet 5 to '0'B respectively, but sets the remaining 5 bits to OMIT. 24.301 cl 9.9.3.12A mentions that “Bits 6 to 8 in octet 5 are spare and shall be coded as zero if included.” Thus, these should only be included if the rest of bits in Octet 5 are included. Otherwise these should also be OMIT. The current TTCN implementation (setting bits bits 6,7 and 8 in Octet 5 to '0'B ) causes encoding issues for ATTACH ACCEPT and TAU ACCEPT messages, and needs to be fixed. This will cause ENDC, NR- IRAT and legacy LTE testcases to fail. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the TTCN implementation of functions f_GetDefaultValueNetworkFeatureSupport, f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq, f_EUTRA38_NR5GC_Preamble. Please check details below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All the ENDC testcases, NR-IRAT testcases and legacy Eutra Testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	3

2	Corrections required	3

2.1	Correction to the function f_GetDefaultValueNetworkFeatureSupport	3

2.2	Correction to the function f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq	5

2.3	Correction to the function f_EUTRA38_NR5GC_Preamble	10

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk43 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to the function f_GetDefaultValueNetworkFeatureSupport

| Template name | f_GetDefaultValueNetworkFeatureSupport |
| --- | --- |
| Reason for change | Please see the main description of the issue above. This function is used to initialize the IE EPS Network Feature Support, which is then used in Attach Accept and TAU Accept. This function initializes the IE NetworkFeatureSupport to template cs_EPS_NwkFtSup_IMSVoice_EMCBS, which sets the bits 6,7 and 8 in Octet 5 to ‘0’b, but does not initialize the rest of the Octet 8 bits. This needs to be ddressed. |
| Summary of change | Updated the implementation of this function to check the remaining bits of Octet 5, and depending on whether these are initialized or not, set the bits 6,7 and 8 to ‘0’B. Please see below. |
| TTCN module | EPS_NAS_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

function f_GetDefaultValueNetworkFeatureSupport() return template (value) EPS_NetworkFeatureSupport { // @sic R5s180648 sic@ var template (value) EPS_NetworkFeatureSupport v_NtwkFeatSupport := cs_EPS_NwkFtSup_IMSVoice_EMCBS; // @sic R5-211505 sic@ // @sic R5s130681, R5-173000 sic@ select (f_GetTestcaseAttrib_Eutra_Release(testcasename())) { // @sic R5s130508 sic@ case ("rel_8") { // @sic R5-211505 sic@ v_NtwkFeatSupport := cs_EPS_NwkFtSup_IMSVoice; } case else {} } if (f_GetTestcaseAttrib_CIOT_CP(testcasename()) or f_GetTestcaseAttrib_CIOT_UP(testcasename())) { // @sic R5-173000 sic@ v_NtwkFeatSupport := cs_EPS_NwkFtSup_CIOT; } return v_NtwkFeatSupport; }

After Change:

function f_GetDefaultValueNetworkFeatureSupport() return template (value) EPS_NetworkFeatureSupport { // @sic R5s180648 sic@ var template (value) EPS_NetworkFeatureSupport v_NtwkFeatSupport := cs_EPS_NwkFtSup_IMSVoice_EMCBS; // @sic R5-211505 sic@ // @sic R5s130681, R5-173000 sic@ select (f_GetTestcaseAttrib_Eutra_Release(testcasename())) { // @sic R5s130508 sic@ case ("rel_8") { // @sic R5-211505 sic@ v_NtwkFeatSupport := cs_EPS_NwkFtSup_IMSVoice; } case else {} } if (f_GetTestcaseAttrib_CIOT_CP(testcasename()) or f_GetTestcaseAttrib_CIOT_UP(testcasename())) { // @sic R5-173000 sic@ v_NtwkFeatSupport := cs_EPS_NwkFtSup_CIOT; } //WA#9_3_1_1 : Start if (not (isvalue(v_NtwkFeatSupport.ptcc) and isvalue(v_NtwkFeatSupport.pr) and isvalue(v_NtwkFeatSupport.rpr) and isvalue(v_NtwkFeatSupport.piv) and isvalue(v_NtwkFeatSupport.ncr))) { v_NtwkFeatSupport.spare := omit; }else { v_NtwkFeatSupport.iel := '03'O; //Octet 5 is included, so IEL should be '03'O } //WA#9_3_1_1 : Stop return v_NtwkFeatSupport; }

## Correction to the function f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq

| Template name | f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq |
| --- | --- |
| Reason for change | Please see the main description of the issue above. This function initializes the IE NetworkFeatureSupport to template cs_EPS_NwkFtSup_NR, which sets the bits 6,7 and 8 in Octet 5 to ‘0’b, but does not initialize the rest of the Octet 8 bits. This needs to be ddressed. |
| Summary of change | Updated the implementation of this function to check the remaining bits of Octet 5, and depending on whether these are initialized or not, set the bits 6,7 and 8 to ‘0’B. Please see below. |
| TTCN module | EUTRA38_NR5GCProcedures.ttcn |
| MCC160 Comment |  |

Before Change:

function f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(EUTRA_CellId_Type p_CellId, NAS_MSG_Indication_Type p_NasInd, N1ToS1_SecurityContext_Type p_CxtType := MAPPED_CXT, RRCConnectionReleaseRequired_Type p_ReleaseRequired := rrcConnectionRelease, template (omit) PLMN_List p_EPLMNlist := omit, boolean p_UseSRB2 := false, NR_TESTMODE_STATE_Type p_TestModeState := TESTMode_OFF, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED, boolean p_FallbackSignalling := false) runs on EUTRA_Multi5GS_PTC return NasCount_Type { var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get(); var GutiParameters_Type v_GutiParams:= f_EUTRA_CellInfo_GetGuti(p_CellId); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId (v_GutiParams.PLMN_Identity); var EPS_UpdateTypeValue v_UpdateResult := f_GetEPSTAUType(NORMAL); var template (value) EPS_NetworkFeatureSupport v_NwkFeatSup := cs_EPS_NwkFtSup_NR; var boolean v_FailWhenK_LessN := false; var integer v_SRB := tsc_SRB1; var template (omit) UERadioCapId v_UERadioCapId:= omit; if (p_CxtType == NEW_CXT) { //4a1 4a2: Authentication Procedure /* Authentication Procedure according to 24.301 cl. 5.4.2 */ v_SecurityParams := f_EUTRA_NAS_Authentication(p_CellId, tsc_SRB1, v_SecurityParams, tsc_SHT_NoSecurityProtection, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected), v_PLMN); f_EUTRA_Security_Set(v_SecurityParams); } if (p_CxtType == NEW_CXT or p_CxtType == EXISTING_CXT) { // 4a3 4a4/4b1 4b2 f_EUTRA_NAS_ActivateSecurity(p_CellId, v_SecurityParams, -, -, false, p_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability); // @sic R5-216280, R5-222047 sic@ } else { // @sic R5s191181 sic@ v_SecurityParams.KSIasme := p_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.asmeNasKeySetId.nasKeySetId; f_EUTRA_Security_Set(v_SecurityParams); } if (p_IwkN26 == SUPPORTED) { // @sic R5-204327 sic@ v_NwkFeatSup.iwkN26 := '1'B; v_FailWhenK_LessN := true; // The only difference between 8a1 and 8b1 branch } if (pc_EPC_RACS) { // @sic R5-216280, R5-222047 sic@ v_UERadioCapId := f_EUTRA_MobileInfo_GetRxdRACS_Id(); v_UERadioCapId.iei := '66'O; } /* Step 5 - 6: send TAU ACCEPT and receive TAU COMPLETE */ f_EUTRA_TrackingAreaUpdate_Accept_Complete(p_CellId, v_GutiParams, v_UpdateResult, // @sic R5s191111 sic@ p_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus, p_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType, NORMAL, p_EPLMNlist, v_NwkFeatSup, -, p_UseSRB2, // @sic R5s191111 change 9 sic@ v_UERadioCapId); // @sic R5-216280 sic@ if (p_UseSRB2){ // @sic R5-204327 sic@ v_SRB := tsc_SRB2; } // Fallback test cases have subsequent signalling so currently only specify up to step 6 if (not p_FallbackSignalling) { // @sic R5s210149 sic@ // Step 8a1 / 8b1 @sic R5-204327, R5-206289 sic@ f_EUTRA38_MultiPDN_EstablishAdditionalPDNs (p_CellId, pc_noOf_PDNsSameConnection, v_FailWhenK_LessN, v_SRB, p_NasInd.SecurityProtection.NasCount); if (p_ReleaseRequired == rrcConnectionRelease or pc_noOf_PDNsNewConnection > 0) { // Step 8a2a1/b1 f_EUTRA_RRC_ConnectionRelease(p_CellId); if (pc_noOf_PDNsNewConnection > 0) { // Step 8a2a2 f_EUTRA38_MultiPDN_RRC_Idle_Extension (p_CellId, p_TestModeState, p_ReleaseRequired, v_FailWhenK_LessN); } } } // AS Security uses NAS Count of TAU Request return p_NasInd.SecurityProtection.NasCount; }

After Change:

function f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq(EUTRA_CellId_Type p_CellId, NAS_MSG_Indication_Type p_NasInd, N1ToS1_SecurityContext_Type p_CxtType := MAPPED_CXT, RRCConnectionReleaseRequired_Type p_ReleaseRequired := rrcConnectionRelease, template (omit) PLMN_List p_EPLMNlist := omit, boolean p_UseSRB2 := false, NR_TESTMODE_STATE_Type p_TestModeState := TESTMode_OFF, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED, boolean p_FallbackSignalling := false) runs on EUTRA_Multi5GS_PTC return NasCount_Type { var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get(); var GutiParameters_Type v_GutiParams:= f_EUTRA_CellInfo_GetGuti(p_CellId); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId (v_GutiParams.PLMN_Identity); var EPS_UpdateTypeValue v_UpdateResult := f_GetEPSTAUType(NORMAL); var template (value) EPS_NetworkFeatureSupport v_NwkFeatSup := cs_EPS_NwkFtSup_NR; var boolean v_FailWhenK_LessN := false; var integer v_SRB := tsc_SRB1; var template (omit) UERadioCapId v_UERadioCapId:= omit; if (p_CxtType == NEW_CXT) { //4a1 4a2: Authentication Procedure /* Authentication Procedure according to 24.301 cl. 5.4.2 */ v_SecurityParams := f_EUTRA_NAS_Authentication(p_CellId, tsc_SRB1, v_SecurityParams, tsc_SHT_NoSecurityProtection, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected), v_PLMN); f_EUTRA_Security_Set(v_SecurityParams); } if (p_CxtType == NEW_CXT or p_CxtType == EXISTING_CXT) { // 4a3 4a4/4b1 4b2 f_EUTRA_NAS_ActivateSecurity(p_CellId, v_SecurityParams, -, -, false, p_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.ueRadioCapIdAvailability); // @sic R5-216280, R5-222047 sic@ } else { // @sic R5s191181 sic@ v_SecurityParams.KSIasme := p_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.asmeNasKeySetId.nasKeySetId; f_EUTRA_Security_Set(v_SecurityParams); } if (p_IwkN26 == SUPPORTED) { // @sic R5-204327 sic@ v_NwkFeatSup.iwkN26 := '1'B; v_FailWhenK_LessN := true; // The only difference between 8a1 and 8b1 branch } //WA#9_3_1_1 : Start if (not (isvalue(v_NwkFeatSup.ptcc) and isvalue(v_NwkFeatSup.pr) and isvalue(v_NwkFeatSup.rpr) and isvalue(v_NwkFeatSup.piv) and isvalue(v_NwkFeatSup.ncr))) { v_NwkFeatSup.spare := omit; }else { v_NwkFeatSup.iel := '03'O; //Octet 5 is included, so IEL should be '03'O } //WA#9_3_1_1 : Stop if (pc_EPC_RACS) { // @sic R5-216280, R5-222047 sic@ v_UERadioCapId := f_EUTRA_MobileInfo_GetRxdRACS_Id(); v_UERadioCapId.iei := '66'O; } /* Step 5 - 6: send TAU ACCEPT and receive TAU COMPLETE */ f_EUTRA_TrackingAreaUpdate_Accept_Complete(p_CellId, v_GutiParams, v_UpdateResult, // @sic R5s191111 sic@ p_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus, p_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType, NORMAL, p_EPLMNlist, v_NwkFeatSup, -, p_UseSRB2, // @sic R5s191111 change 9 sic@ v_UERadioCapId); // @sic R5-216280 sic@ if (p_UseSRB2){ // @sic R5-204327 sic@ v_SRB := tsc_SRB2; } // Fallback test cases have subsequent signalling so currently only specify up to step 6 if (not p_FallbackSignalling) { // @sic R5s210149 sic@ // Step 8a1 / 8b1 @sic R5-204327, R5-206289 sic@ f_EUTRA38_MultiPDN_EstablishAdditionalPDNs (p_CellId, pc_noOf_PDNsSameConnection, v_FailWhenK_LessN, v_SRB, p_NasInd.SecurityProtection.NasCount); if (p_ReleaseRequired == rrcConnectionRelease or pc_noOf_PDNsNewConnection > 0) { // Step 8a2a1/b1 f_EUTRA_RRC_ConnectionRelease(p_CellId); if (pc_noOf_PDNsNewConnection > 0) { // Step 8a2a2 f_EUTRA38_MultiPDN_RRC_Idle_Extension (p_CellId, p_TestModeState, p_ReleaseRequired, v_FailWhenK_LessN); } } } // AS Security uses NAS Count of TAU Request return p_NasInd.SecurityProtection.NasCount; }

## Correction to the function f_EUTRA38_NR5GC_Preamble

| Template name | f_EUTRA38_NR5GC_Preamble |
| --- | --- |
| Reason for change | Please see the main description of the issue above. This function initializes the IE NetworkFeatureSupport to template cs_EPS_NwkFtSup_NR, which sets the bits 6,7 and 8 in Octet 5 to ‘0’b, but does not initialize the rest of the Octet 8 bits. This needs to be ddressed. |
| Summary of change | Updated the implementation of this function to check the remaining bits of Octet 5, and depending on whether these are initialized or not, set the bits 6,7 and 8 to ‘0’B. Please see below. |
| TTCN module | EUTRA38_NR5GCProcedures.ttcn |
| MCC160 Comment |  |

Before Change:

function f_EUTRA38_NR5GC_Preamble(EUTRA_CellId_Type p_CellId, NR_TESTMODE_STATE_Type p_TestModeState, EUTRA_RRC_STATE_Type p_RRC_State, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, template (omit) EPS_NetworkFeatureSupport p_NtwkFeatSupport := cs_EPS_NwkFtSup_NR, template (omit) PLMN_List p_EquivalentPlmnList := omit, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED) runs on EUTRA_Multi5GS_PTC { var NR_TESTMODE_STATE_Type v_IdleUpdatedState := p_TestModeState; var AbsoluteCellPower_Type v_AbsoluteCellPowerLevel := tsc_ServingCellRS_EPRE; var template (value) EPS_NetworkFeatureSupport v_NtwkFeatSupport := p_NtwkFeatSupport; select (p_TestModeState) { // @sic R5-204399 sic@ case (PING_Or_TESTModeB_ON, PING_Or_TEST_LOOPModeB_ON){ if (pc_IP_Ping) { // don't need to activate test mode if using Ping @sic R5s201654 sic@ v_IdleUpdatedState := TESTMode_OFF; } } } if (p_IwkN26 == SUPPORTED) { // @sic R5-221406 sic@ v_NtwkFeatSupport.iwkN26 := '1'B; } // Store the value of NetworkFeatureSupport to be used throughout the test f_EUTRA38_SetNetworkSupport(p_IwkN26); // @sic R5-221406 sic@ f_EUTRA_MobileInfo_SetNetworkFeatureSupport (v_NtwkFeatSupport); f_EUTRA38_MultiPDN_InitialRegistration(p_CellId, // @sic R5-204399 sic@ v_IdleUpdatedState, p_ReleaseConnection, p_EquivalentPlmnList, v_AbsoluteCellPowerLevel); if (p_RRC_State == RRC_CONNECTED) { f_EUTRA38_MultiPDN_RbEst_Def(p_CellId); if (p_TestModeState == TEST_LOOPModeB_ON or (p_TestModeState == PING_Or_TEST_LOOPModeB_ON and not pc_IP_Ping)) { // @sic R5s200887, R5s201654 sic@ f_EUTRA_CloseUE_TestLoopModeB (p_CellId, '00'O); } } }

After Change:

function f_EUTRA38_NR5GC_Preamble(EUTRA_CellId_Type p_CellId, NR_TESTMODE_STATE_Type p_TestModeState, EUTRA_RRC_STATE_Type p_RRC_State, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, template (omit) EPS_NetworkFeatureSupport p_NtwkFeatSupport := cs_EPS_NwkFtSup_NR, template (omit) PLMN_List p_EquivalentPlmnList := omit, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED) runs on EUTRA_Multi5GS_PTC { var NR_TESTMODE_STATE_Type v_IdleUpdatedState := p_TestModeState; var AbsoluteCellPower_Type v_AbsoluteCellPowerLevel := tsc_ServingCellRS_EPRE; var template (value) EPS_NetworkFeatureSupport v_NtwkFeatSupport := p_NtwkFeatSupport; select (p_TestModeState) { // @sic R5-204399 sic@ case (PING_Or_TESTModeB_ON, PING_Or_TEST_LOOPModeB_ON){ if (pc_IP_Ping) { // don't need to activate test mode if using Ping @sic R5s201654 sic@ v_IdleUpdatedState := TESTMode_OFF; } } } if (p_IwkN26 == SUPPORTED) { // @sic R5-221406 sic@ v_NtwkFeatSupport.iwkN26 := '1'B; } //WA#9_3_1_1 : Start if (not (isvalue(v_NtwkFeatSupport.ptcc) and isvalue(v_NtwkFeatSupport.pr) and isvalue(v_NtwkFeatSupport.rpr) and isvalue(v_NtwkFeatSupport.piv) and isvalue(v_NtwkFeatSupport.ncr))) { v_NtwkFeatSupport.spare := omit; }else { v_NtwkFeatSupport.iel := '03'O; //Octet 5 is included, so IEL should be '03'O } //WA#9_3_1_1 : Stop // Store the value of NetworkFeatureSupport to be used throughout the test f_EUTRA38_SetNetworkSupport(p_IwkN26); // @sic R5-221406 sic@ f_EUTRA_MobileInfo_SetNetworkFeatureSupport (v_NtwkFeatSupport); f_EUTRA38_MultiPDN_InitialRegistration(p_CellId, // @sic R5-204399 sic@ v_IdleUpdatedState, p_ReleaseConnection, p_EquivalentPlmnList, v_AbsoluteCellPowerLevel); if (p_RRC_State == RRC_CONNECTED) { f_EUTRA38_MultiPDN_RbEst_Def(p_CellId); if (p_TestModeState == TEST_LOOPModeB_ON or (p_TestModeState == PING_Or_TEST_LOOPModeB_ON and not pc_IP_Ping)) { // @sic R5s200887, R5s201654 sic@ f_EUTRA_CloseUE_TestLoopModeB (p_CellId, '00'O); } } }