3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221279

# Online, , 13th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4684 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_EUTRA_WaitForIMSReRegister |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2022-07-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Post IMS Reregistration procedure, the state of the IMS DefaultHandler is not updated to set to WaitForDeregistration, and hence while performing IMS Deregistration procedure, ttcn incorrectly invokes Reregistration functions in place of Deregistration functions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Coordination message is sent to indicate end of IMS Reregistration procedure by using template cms_IPCAN_IMS_ReregistrationStop |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.1, 6.1.1.1b, 6.1.1.2, 6.1.1.2a, 6.1.1.3b, 6.1.1.6, 6.1.1.6a, 9.2.1.1.7, 9.2.1.1.7a, 9.2.3.1.4, 9.2.3.1.25, 9.2.3.2.13, 11.2.6, 11.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | R5s220228, R5s220229 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D22wk23 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_EUTRA_WaitForIMSReRegister |
| --- | --- |
| Reason for change | Post IMS Reregistration procedure, the state of the IMS DefaultHandler is not updated to set to WaitForDeregistration, and hence while performing IMS Deregistration procedure, ttcn incorrectly invokes Reregistration functions in place of Deregistration functions |
| Summary of change | Coordination message is sent to indicate end of IMS Reregistration procedure by using template cms_IPCAN_IMS_ReregistrationStop |
| TTCN module | EUTRA_NASSteps.ttcn |
| MCC160 Comment |  |

Before change:

function f_EUTRA_WaitForIMSReRegister(EUTRA_CellId_Type p_CellId) runs on EUTRA_PTC

{

var IPCAN_RAN_Type v_RanType := f_EUTRA_IMS_GetRanType();

var NAS_MSG_Indication_Type v_NasInd;

var NAS_KsiValue v_KsiValue := f_EUTRA_SecurityKSIasme_Get();

timer t_WaitForIMS := 10.0;

if (pc_IMS) { // @sic R5s220228, R5s220229 sic@

t_WaitForIMS.start;

alt {

[] t_WaitForIMS.timeout {} // Don't do anything

[] SRB.check(receive(car_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest(cr_EstablishmentCause_Any)))) {

t_WaitForIMS.stop;

v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(p_CellId,

tsc_RRC_TI_Def,

cr_EstablishmentCause_Any,

cr_NAS_Indication(tsc_SHT_IntegrityProtected,

cr_508_SERVICE_REQUEST(v_KsiValue)));

f_IP_ChangeEutraCell(IP, p_CellId);

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_ReregistrationStart(v_RanType));

f_EUTRA_Activate_SRB2_DRB_SendRrcMsg(p_CellId,

tsc_RRC_TI_Def,

v_NasInd.SecurityProtection.NasCount);

f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(p_CellId,

tsc_RRC_TI_Def);

f_EUTRA_DelayForUserPlaneSignalling(waitForIMS);

f_EUTRA_RRC_ConnectionRelease(p_CellId);

}

}

}

}

After change:

function f_EUTRA_WaitForIMSReRegister(EUTRA_CellId_Type p_CellId) runs on EUTRA_PTC

{

var IPCAN_RAN_Type v_RanType := f_EUTRA_IMS_GetRanType();

var NAS_MSG_Indication_Type v_NasInd;

var NAS_KsiValue v_KsiValue := f_EUTRA_SecurityKSIasme_Get();

timer t_WaitForIMS := 10.0;

if (pc_IMS) { // @sic R5s220228, R5s220229 sic@

t_WaitForIMS.start;

alt {

[] t_WaitForIMS.timeout {} // Don't do anything

[] SRB.check(receive(car_SRB0_RrcPdu_IND(?, cr_RRCConnectionRequest(cr_EstablishmentCause_Any)))) {

t_WaitForIMS.stop;

v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(p_CellId,

tsc_RRC_TI_Def,

cr_EstablishmentCause_Any,

cr_NAS_Indication(tsc_SHT_IntegrityProtected,

cr_508_SERVICE_REQUEST(v_KsiValue)));

f_IP_ChangeEutraCell(IP, p_CellId);

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_ReregistrationStart(v_RanType));

f_EUTRA_Activate_SRB2_DRB_SendRrcMsg(p_CellId,

tsc_RRC_TI_Def,

v_NasInd.SecurityProtection.NasCount);

f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(p_CellId,

tsc_RRC_TI_Def);

f_EUTRA_DelayForUserPlaneSignalling(waitForIMS);

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_ReregistrationStop);

f_EUTRA_RRC_ConnectionRelease(p_CellId);

}

}

}

}