3GPP TSG-RAN5 Meeting #2022-TTCN email	R5s221333

# Online, , 10th Dec 2021 - 31st Dec 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 2864 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RACS testcase 9.1.9.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2022-11-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At step 13, REGISTRATION ACCEPT message sent by SS should not include any ueRadioCapabilityId. Current implementation will assign ueRadioCapabilityId sent by UE in step 12 to message at step 13. That will not suppose any problem in case UE does not populate the field but will do in case includes a Manufacturer assigned UE radio capability ID which is valid in step 12. This issue can be solved setting this field to omit before is sent with REGISTRATION ACCEPT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set ueRadioCapabilityId field to omit before is sent in REGISTRATION ACCEPT message at step 13. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.9.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	3

2.1	Correction to function fl_TC_9_1_9_5_Body	3

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2020-09_D22wk37 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Correction to function fl_TC_9_1_9_5_Body

| Function name | fl_TC_9_1_9_5_Body |
| --- | --- |
| Reason for change | Current implementation will assign ueRadioCapabilityId sent by UE in step 12 to message at step 13. That will not suppose any problem in case UE does not populate the field but will do in case includes a Manufacturer assigned UE radio capability ID which is valid in step 12. |
| Summary of change | Set ueRadioCapabilityId field to omit before is sent in REGISTRATION ACCEPT message at step 13. |
| TTCN module | \NR5GC\9.1.9\RACS_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 13" siclog@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList ); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellB, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@sic R5s211655 sic@ //@siclog "Step 14" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellB, tsc_NR_RbId_SRB1); //@sic R5s211655 sic@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 13" siclog@ v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Mobility, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList ); v_MsgToSend.registration_Accept.ueRadioCapabilityId := omit; SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellB, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@sic R5s211655 sic@ //@siclog "Step 14" siclog@ f_NR5GC_Registration_Complete(v_NGC_NASCellB, tsc_NR_RbId_SRB1); //@sic R5s211655 sic@ <<SKIPPED CODE>>