3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240123

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0942 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Voice Call Control test cases 7.5 and 7.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the test peocedure for 7.5 and 7.12 test cases, after the transmision of 180 Ringing message, the subsequent messages in the expected sequence shall be the PRACK and 200 OK messages. These are missing from the test procedure and consequently in TTCN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added handling of PRACK and 200 OK messages after SS transmits 180 Ringing message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformance device will fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.NR5GC, 7.12.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-5 CR xxxx |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D23wk49 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required

## Change 1

| Function name | function f_TC_7_5_NR5GC_IMS1() |
| --- | --- |
| Reason for change | After the transmision of 180 Ringing message, the subsequent messages in the expected sequence shall be the PRACK and 200 OK messages. These are missing from the test procedure and consequently in TTCN. |
| Summary of change | Added handling of PRACK and 200 OK messages after SS transmits 180 Ringing message. Please note draft prose CRs for this change is associated with this TTCN CR. |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment | Accepted in principle: implemented according to R5-241190, R5-241655. |

Before Change:

function f_TC_7_5_NR5GC_IMS1() runs on IMS_PTC { // MTSI MO Voice Call without preconditions at both originating UE and terminating UE / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); // @siclog "Step 2" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_2a_Step1(); // @sic R5-236896: A_4_2 -> A_4_2a sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 2"); …. … // @siclog "Step 7" siclog@ 180 Ringing // f_IMS_MOCallSetup_NR5GC_A_4_2a_Step6(v_InviteRequestWithSdp); // @sic R5-236896: A.4.2a sic@ // @siclog "Step 8 - 9" siclog@ 200 OK, ACK f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 9");

After Change:

function f_TC_7_5_NR5GC_IMS1() runs on IMS_PTC { // MTSI MO Voice Call without preconditions at both originating UE and terminating UE / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); // @siclog "Step 2" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_2a_Step1(); // @sic R5-236896: A_4_2 -> A_4_2a sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 2"); … …. // @siclog "Step 7" siclog@ 180 Ringing // f_IMS_MOCallSetup_NR5GC_A_4_2a_Step6(v_InviteRequestWithSdp); //Removed // @sic R5-236896: A.4.2a sic@ //@sinclog "Step 7a_7b" sinclog@ //KS_Changes f_IMS_MOCallSetup_Send180_Reliable(v_InviteRequestWithSdp); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 7a_7b"); // @siclog "Step 8 - 9" siclog@ 200 OK, ACK f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 9");

## Change 1

| Function name | function f_TC_7_12_NR5GC_IMS1() |
| --- | --- |
| Reason for change | After the transmision of 180 Ringing message, the subsequent messages in the expected sequence shall be the PRACK and 200 OK messages. These are missing from the test procedure and consequently in TTCN. |
| Summary of change | Added handling of PRACK and 200 OK messages after SS transmits 180 Ringing message. Please note draft prose CRs for this change is associated with this TTCN CR. |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment | Accepted in principle: implemented according to R5-241190, R5-241656. |

Before Change:

function f_TC_7_12_NR5GC_IMS1() runs on IMS_PTC { // MTSI MO Voice Call with preconditions at originating UE and without preconditions at terminating UE / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); // @siclog "Step 2" siclog@ Steps 2-7 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 // @siclog "Step 3" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1a_Step1(); // @sic R5-236898: A.4.1a sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3"); // @siclog "Step 4 - 7" siclog@ 100 Trying, 183 Session Progress, PRACK, 200 OK f_IMS_MOCallSetup_NR5GC_A_4_2a_Step2_5(v_InviteRequestWithSdp); // @sic R5-236898: A.4.2a sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); f_IMS_CC_TriggerDedicatedBearerActivation(IPCAN_MO_SpeechCall); // to trigger step 13 of 38.508-1 Table 4.9.15.2.2-1 // @siclog "Step 8" siclog@ 180 Ringing // f_IMS_MOCallSetup_NR5GC_A_4_2a_Step6(v_InviteRequestWithSdp); // @sic R5-236898: A.4.2a sic@ // @siclog "Step 9 - 10" siclog@ 200 OK, ACK f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10"); f_IMS_TestBody_Set(false); // Call release and postamble: f_IMS_CC_PostambleWithCallReleaseMT(IPCAN_MO_SpeechCall); }

After Change:

function f_TC_7_12_NR5GC_IMS1() runs on IMS_PTC { // MTSI MO Voice Call with preconditions at originating UE and without preconditions at terminating UE / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; f_IMS_CC_Preamble(IPCAN_SpeechCall, IMS_REGISTERED); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); // @siclog "Step 2" siclog@ Steps 2-7 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 // @siclog "Step 3" siclog@ INVITE v_InviteRequestWithSdp := f_IMS_MOCallSetup_NR5GC_A_4_1a_Step1(); // @sic R5-236898: A.4.1a sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 3"); // @siclog "Step 4 - 7" siclog@ 100 Trying, 183 Session Progress, PRACK, 200 OK f_IMS_MOCallSetup_NR5GC_A_4_2a_Step2_5(v_InviteRequestWithSdp); // @sic R5-236898: A.4.2a sic@ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); f_IMS_CC_TriggerDedicatedBearerActivation(IPCAN_MO_SpeechCall); // to trigger step 13 of 38.508-1 Table 4.9.15.2.2-1 // @siclog "Step 8" siclog@ 180 Ringing // f_IMS_MOCallSetup_NR5GC_A_4_2a_Step6(v_InviteRequestWithSdp); // @sic R5-236898: A.4.2a sic@ //Removed //@sinclog "Step 8a_8b" sinclog@ //KS_Changes f_IMS_MOCallSetup_Send180_Reliable(v_InviteRequestWithSdp); //Added f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 8a_8b"); // @siclog "Step 9 - 10" siclog@ 200 OK, ACK f_IMS_MOCallSetup_Send200OK_ReceiveACK(v_InviteRequestWithSdp); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10"); f_IMS_TestBody_Set(false); // Call release and postamble: f_IMS_CC_PostambleWithCallReleaseMT(IPCAN_MO_SpeechCall); }