3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240300

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3419 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR5GC Testcase 8.1.3.1.20 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2024-04-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test case is failing due to incorrect gapFR1 configuration in TTCN, the gapoffset is set as 0 while it should be 9 as per TS 38.523-1, Table 8.1.3.1.20.3.3-5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified the gapoffset to 9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.1.20.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	6

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2022-09_D24wk12 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function f_NR_TC_8_1_3_1_20_x_ReconfigureGapPatternAndCheckForReport () |
| --- | --- |
| Reason for change | The test case is failing due to incorrect gapFR1 configuration in TTCN, the gapoffset is set as 0 while it should be 9 as per TS 38.523-1, Table 8.1.3.1.20.3.3-5. |
| Summary of change | Modified the gapoffset to 9. |
| TTCN module | RRC_Measurement_NR5GC.ttcn |
| MCC160 Comment | Accepted, but will be implemented differently, see alternative proposal |

Before Change:

function f_NR_TC_8_1_3_1_20_x_ReconfigureGapPatternAndCheckForReport (integer p_GapPatternNo, template (present) UL_DCCH_Message p_ExpectedMeasReport ) runs on NR_BASE_PTC { var template (value) MeasConfig v_MeasConfig; var GapConfig.mgl v_Mgl; var GapConfig.mgrp v_Mgrp; //adjust mgl values select(p_GapPatternNo) { case(2,3,10,11) { v_Mgl := ms3; } /***************Skipping the code *********************/ case(5,9,11, 15, 19,23) { v_Mgrp := ms160; } } v_MeasConfig := cds_NR_MeasConfig_Modify(omit, cds_MeasGapConfig_FR1(cs_NR_GapConfig_ZeroOffset_ZeroAdvance(v_Mgl, v_Mgrp))); /***************Skipping the code *********************/ }

After Change:

function f_NR_TC_8_1_3_1_20_x_ReconfigureGapPatternAndCheckForReport (integer p_GapPatternNo, template (present) UL_DCCH_Message p_ExpectedMeasReport, boolean p_GapConfig := false ) runs on NR_BASE_PTC { var template (value) MeasConfig v_MeasConfig; var GapConfig.mgl v_Mgl; var GapConfig.mgrp v_Mgrp; //adjust mgl values select(p_GapPatternNo) { case(2,3,10,11) { v_Mgl := ms3; } /***************Skipping the code *********************/ case(5,9,11, 15, 19,23) { v_Mgrp := ms160; } } if(p_GapConfig)//Deepti@17391 { v_MeasConfig := cs_NR_MeasConfigCommon(-,-,-,-,-,-,-,-, cds_MeasGapConfig_FR1(cs_NR_GapConfig_ZeroAdvance(v_Mgl, v_Mgrp))); } else{ v_MeasConfig := cds_NR_MeasConfig_Modify(omit, cds_MeasGapConfig_FR1(cs_NR_GapConfig_ZeroOffset_ZeroAdvance(v_Mgl, v_Mgrp))); } /***************Skipping the code *********************/ }

MCC160 alternative implementation:

function f_NR_TC_8_1_3_1_20_x_ReconfigureGapPatternAndCheckForReport (integer p_GapPatternNo, template (present) UL_DCCH_Message p_ExpectedMeasReport ) runs on NR_BASE_PTC { var template (value) MeasConfig v_MeasConfig; var GapConfig.mgl v_Mgl; var GapConfig.mgrp v_Mgrp; //adjust mgl values select(p_GapPatternNo) { case(2,3,10,11) { v_Mgl := ms3; } /***************Skipping the code *********************/ case(5,9,11, 15, 19,23) { v_Mgrp := ms160; } } if(f_NR_CellInfo_GetIsFR1(nr_Cell1)) {//@sic R5s240300 sic@ v_MeasConfig := cs_NR_MeasConfigCommon(-,-,-,-,-,-,-,-, cds_MeasGapConfig_FR1(cs_NR_GapConfig_Offset9_ZeroAdvance(v_Mgl, v_Mgrp))); } else { v_MeasConfig := cs_NR_MeasConfigCommon(-,-,-,-,-,-,-,-, cds_MeasGapConfig_FR2(cs_NR_GapConfig_ZeroOffset_ZeroAdvance(v_Mgl, v_Mgrp))); } /***************Skipping the code *********************/ }

## Change 2

| Function name | function f_TC_8_1_3_1_20_TestBody() runs on NR_BASE_PTC |
| --- | --- |
| Reason for change | The test case is failing due to incorrect gapFR1 configuration in TTCN, the gapoffset is set as 0 while it should be 9 as per TS 38.523-1, Table 8.1.3.1.20.3.3-5. |
| Summary of change | Modified the gapoffset to 9. |
| TTCN module | RRC_Measurement_NR5GC.ttcn |
| MCC160 Comment | Rejected, not needed due to an alternative implementation of Change 1 |

Before Change:

function f_TC_8_1_3_1_20_TestBody() runs on NR_BASE_PTC { var PhysCellId v_PhysCellIdServing; var PhysCellId v_PhysCellIdNeighbour; var template (value) NR_CellPowerList_Type v_CellPowerList; //@sic R5s200815 sic@ var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (value) MeasGapConfig v_GapConfig; var template (value) MeasConfig v_MeasConfig; /***************Skipping the code *********************/ //@siclog "Step 4-10" siclog@ //The SS transmits an RRCConnectionReconfiguration to change gap pattern to patterns #1-11. //The UE transmits an RRCConnectionReconfigurationComplete message to confirm the change of pattern. //Check: Does the UE transmit at least 10 MeasurementReport messages to periodically report the measured RSRP value for NR Cell 3 within the next 30 seconds? while (v_GapPatternNumber < 12) { if(v_GapPatternNumber != 1) { if (bit2int(v_SupportedGaps and4b v_GapMask) == 1) { f_NR_TC_8_1_3_1_20_x_ReconfigureGapPatternAndCheckForReport(v_GapPatternNumber, v_ExpectedMeasReport); } v_SupportedGaps := v_SupportedGaps @> 1; } else {//gap#1 is always supported f_NR_TC_8_1_3_1_20_x_ReconfigureGapPatternAndCheckForReport(v_GapPatternNumber, v_ExpectedMeasReport); } v_GapPatternNumber := v_GapPatternNumber + 1; }

After Change:

function f_TC_8_1_3_1_20_TestBody() runs on NR_BASE_PTC { var PhysCellId v_PhysCellIdServing; var PhysCellId v_PhysCellIdNeighbour; var template (value) NR_CellPowerList_Type v_CellPowerList; //@sic R5s200815 sic@ var template (value) MeasObjectParamsList_Type v_MeasuredObjectList; var template (value) ReportConfigToAddModList v_ReportConfigList; var template (value) MeasIdToAddModList v_MeasIdConfigList; var template (value) MeasGapConfig v_GapConfig; var template (value) MeasConfig v_MeasConfig; /***************Skipping the code *********************/ //@siclog "Step 4-10" siclog@ //The SS transmits an RRCConnectionReconfiguration to change gap pattern to patterns #1-11. //The UE transmits an RRCConnectionReconfigurationComplete message to confirm the change of pattern. //Check: Does the UE transmit at least 10 MeasurementReport messages to periodically report the measured RSRP value for NR Cell 3 within the next 30 seconds? while (v_GapPatternNumber < 12) { if(v_GapPatternNumber != 1) { if (bit2int(v_SupportedGaps and4b v_GapMask) == 1) { f_NR_TC_8_1_3_1_20_x_ReconfigureGapPatternAndCheckForReport(v_GapPatternNumber, v_ExpectedMeasReport,true); } v_SupportedGaps := v_SupportedGaps @> 1; } else {//gap#1 is always supported f_NR_TC_8_1_3_1_20_x_ReconfigureGapPatternAndCheckForReport(v_GapPatternNumber, v_ExpectedMeasReport,true); } v_GapPatternNumber := v_GapPatternNumber + 1; } /***************Skipping the code *********************/ } }

## Change 3

| Function name | New template cs_NR_GapConfig_ZeroAdvance |
| --- | --- |
| Reason for change | The test case is failing due to incorrect gapFR1 configuration in TTCN, the gapoffset is set as 0 while it should be 9 as per TS 38.523-1, Table 8.1.3.1.20.3.3-5. |
| Summary of change | Modified the gapoffset to 9. |
| TTCN module | NR_Measurement_Templates.ttcn |
| MCC160 Comment | Accepted, for a clarity sake will be named as cs_NR_GapConfig_Offset9_ZeroAdvance |

After Change:

template (value) GapConfig cs_NR_GapConfig_ZeroAdvance (GapConfig.mgl p_Mgl, GapConfig.mgrp p_Mgrp) := /* @status APPROVED (NR5GC) */ cs_NR_GapConfig(9, p_Mgl, p_Mgrp, ms0, omit);