3GPP TSG-RAN5 Meeting #2024-TTCN email	R5s240707

# Online, , 8th Dec 2023 - 31st Dec 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4874 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT-NTN testcase 22.3.1.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2024-12-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The updated implementation of R5s240579 results in a runtime error when f_NBIOT_MobileInfo_IsNTN is called without f_NBIOT_Init is being called. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the test case sequence as f_NBIOT_Init and update SIB31. Confgiure the cell and call f_NBIOT_L2_Enter_State2B_NB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required for test case 22.3.1.6	2

2.1	Change to f_TC_22_3_1_6_NBIOT	2

# Overview

This document lists all the essential changes needed to correct problems in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required for test case 22.3.1.6

## Change to f_TC_22_3_1_6_NBIOT

| Function name | f_TC_22_3_1_6_NBIOT |
| --- | --- |
| Reason for change | The updated implementation of R5s240579 results in a runtime error when f_NBIOT_MobileInfo_IsNTN is called without f_NBIOT_Init is being called. |
| Summary of change | Updated the test case sequence as f_NBIOT_Init and update SIB31. Confgiure the cell and call f_NBIOT_L2_Enter_State2B_NB |
| TTCN module | NBIOT_MAC_Testcases.ttcn |
| MCC160 Comment | Accepted. Obsolete objects will be removed. |

Before Change:

function f_TC_22_3_1_6_NBIOT() runs on NBIOT_PTC { var NBIOT_CellId_Type v_CellId := nbiot_Cell1; var NBIOT_IDLEUPDATED_STATE_Type v_TestLoopMode := STATE2A_NB_TESTLOOP_ModeG; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef; // RLC status to be maintained by Send/Receive functions var SubFrameTiming_Type v_SubFrameTiming; var template (value) TimingInfo_Type v_TimingInfo; var Table_22_3_1_6_3_2__2_Entry_Type v_TestData; var RLC_SDU_Type v_RlcSdu; var RLC_SDUList_Type v_RlcSduList; var integer v_RlcSduSize; var integer v_Start := 0; var integer i; var integer v_NpdcchBundleOffset; var float v_SchedulingOffset; var template (omit) NB_NTN_SpecificSysInfoParams_Type v_NB_NTN_SpecificSysInfoParams := omit; if(f_NBIOT_MobileInfo_IsNTN()){ //@sic R5s240579, R5-246574 sic@ v_NB_NTN_SpecificSysInfoParams := cds_SpecificSyncValidityDuration(s900); /*@sic R5-247617 sic@*/ } f_NBIOT_L2_Preamble_State2B_NB(v_RLC_CountsInfoListByRef, v_CellId, TransparentMode_RLC_MAC, v_TestLoopMode, -,-,-,-, v_NB_NTN_SpecificSysInfoParams); //@sic R5s240579 sic@ v_NpdcchBundleOffset := f_NBIOT_GetNPDCCH_PeriodsInBundle(nbiot_Cell1); //@sic R5s240285, R5s240266, R5-241564 sic@

After Change:

function f_TC_22_3_1_6_NBIOT() runs on NBIOT_PTC { var NBIOT_CellId_Type v_CellId := nbiot_Cell1; var NBIOT_IDLEUPDATED_STATE_Type v_TestLoopMode := STATE2A_NB_TESTLOOP_ModeG; var NB_RLC_CountsInfoList_Type v_RLC_CountsInfoListByRef; // RLC status to be maintained by Send/Receive functions var SubFrameTiming_Type v_SubFrameTiming; var template (value) TimingInfo_Type v_TimingInfo; var Table_22_3_1_6_3_2__2_Entry_Type v_TestData; var RLC_SDU_Type v_RlcSdu; var RLC_SDUList_Type v_RlcSduList; var integer v_RlcSduSize; var integer v_Start := 0; var integer i; var integer v_NpdcchBundleOffset; var float v_SchedulingOffset; var template (omit) NB_NTN_SpecificSysInfoParams_Type v_NB_NTN_SpecificSysInfoParams := omit; //WA#WI=1368159 f_NBIOT_Init(c1); if(f_NBIOT_MobileInfo_IsNTN()){ f_NBIOT_CellInfo_ModifySIB31_UL_SyncValidityDuration_r17(v_CellId,s900); } //Create and configure all cells f_NBIOT_CellConfig_Def(v_CellId, CONTROL_PLANE); //f_NBIOT_L2_Preamble_State2B_NB(v_RLC_CountsInfoListByRef, v_CellId, TransparentMode_RLC_MAC, v_TestLoopMode, -,-,-,-, v_NB_NTN_SpecificSysInfoParams); //@sic R5s240579 sic@ f_NBIOT_L2_Enter_State2B_NB(v_RLC_CountsInfoListByRef, v_CellId, TransparentMode_RLC_MAC, v_TestLoopMode); v_NpdcchBundleOffset := f_NBIOT_GetNPDCCH_PeriodsInBundle(nbiot_Cell1); //@sic R5s240285, R5s240266, R5-241564 sic@

## Additional MCC160 Changes

| Function name | f_NBIOT_CellInfo_GetSIB31 |
| --- | --- |
| Reason for change | The current implementation treats only c9 sys info combination as a special case. Taking into account that as the system evolves the other special sys info combinations may be build up, it is proposed to extend the function with the further control over the possible sys info combinations. |
| Summary of change | Updated the function to use cases corresponding used sys info combination |
| TTCN module | NBIOT_CellInfo |

Before Change:

function f_NBIOT_CellInfo_GetSIB31(NBIOT_CellId_Type p_CellId) runs on NBIOT_PTC return SystemInformationBlockType31_NB_r17 {//@sic R5s240168, R5-235420, R5s230550 sic@ var NBIOT_CellInfo_Type v_CellInfo := f_NBIOT_CellInfo_Get(p_CellId); var integer v_Index := f_NBIOT_CellInfo_Get_SI_Index(v_CellInfo.Sysinfo.Combination, 31); if (v_CellInfo.Sysinfo.Combination!=c9){ return v_CellInfo.Sysinfo.BCCH_Info.SIs[v_Index].message_.c1.systemInformation_r13.criticalExtensions.systemInformation_r13.sib_TypeAndInfo_r13[0].sib31_v1700; } else{ return v_CellInfo.Sysinfo.BCCH_Info.SIs[v_Index].message_.c1.systemInformation_r13.criticalExtensions.systemInformation_r13.sib_TypeAndInfo_r13[1].sib31_v1700; } }

After Change:

function f_NBIOT_CellInfo_GetSIB31(NBIOT_CellId_Type p_CellId) runs on NBIOT_PTC return SystemInformationBlockType31_NB_r17 {//@sic R5s230550, R5-235420, R5s240168, R5s240707 add change sic@ var SystemInformationBlockType31_NB_r17 v_SIB31_NB; var NBIOT_CellInfo_Type v_CellInfo := f_NBIOT_CellInfo_Get(p_CellId); var integer v_Index := f_NBIOT_CellInfo_Get_SI_Index(v_CellInfo.Sysinfo.Combination, 31); // SIB31-NB is only present in combinations c8..c13 (see 36.508 cl. 8.1.4.3.1.2) select (valueof(v_CellInfo.Sysinfo.Combination)) { case (c8, c10, c11, c12, c13) { //The combination initiation can be found in f_NBIOT_InitSystemInformation v_SIB31_NB := v_CellInfo.Sysinfo.BCCH_Info.SIs[v_Index].message_.c1.systemInformation_r13.criticalExtensions.systemInformation_r13.sib_TypeAndInfo_r13[0].sib31_v1700; } case (c9) { v_SIB31_NB := v_CellInfo.Sysinfo.BCCH_Info.SIs[v_Index].message_.c1.systemInformation_r13.criticalExtensions.systemInformation_r13.sib_TypeAndInfo_r13[1].sib31_v1700; } case else { FatalError(__FILE__, __LINE__, "Invalid System Combination"); } } return v_SIB31_NB; }

| Function name | f_NBIOT_CellInfo_ModifySIB31_UL_SyncValidityDuration_r17 |
| --- | --- |
| Reason for change | The current implementation authonomously derrives SIB31-NB from the sys info. This process should be optimised to reduce maintenance costs |
| Summary of change | Updated the function to utilise f_NBIOT_CellInfo_GetSIB31 and f_NBIOT_CellInfo_SetSIB31 functions |
| TTCN module | NBIOT_CellInfo |

Before Change:

function f_NBIOT_CellInfo_ModifySIB31_UL_SyncValidityDuration_r17(NBIOT_CellId_Type p_CellId, template (value) SystemInformationBlockType31_NB_r17.servingSatelliteInfo_r17.ul_SyncValidityDuration_r17 p_UL_SyncValidityDuration_r17) runs on NBIOT_PTC { var template (value) NBIOT_CellInfo_Type v_CellInfo := f_NBIOT_CellInfo_Get(p_CellId); var integer v_Index := f_NBIOT_CellInfo_Get_SI_Index(valueof(v_CellInfo.Sysinfo.Combination), 31); // SIB31-NB is only present in combinations c8..c13 (see 36.508 cl. 8.1.4.3.1.2) select (valueof(v_CellInfo.Sysinfo.Combination)) { case (c8, c10, c11, c12, c13) { //The combination initiation can be found in f_NBIOT_InitSystemInformation v_CellInfo.Sysinfo.BCCH_Info.SIs[v_Index].message_.c1.systemInformation_r13.criticalExtensions.systemInformation_r13.sib_TypeAndInfo_r13[0].sib31_v1700.servingSatelliteInfo_r17.ul_SyncValidityDuration_r17 := p_UL_SyncValidityDuration_r17; } case (c9) { v_CellInfo.Sysinfo.BCCH_Info.SIs[v_Index].message_.c1.systemInformation_r13.criticalExtensions.systemInformation_r13.sib_TypeAndInfo_r13[1].sib31_v1700.servingSatelliteInfo_r17.ul_SyncValidityDuration_r17 := p_UL_SyncValidityDuration_r17; } case else { FatalError(__FILE__, __LINE__, "Invalid System Combination"); } } f_NBIOT_CellInfo_Set(p_CellId, v_CellInfo); }

After Change:

function f_NBIOT_CellInfo_ModifySIB31_UL_SyncValidityDuration_r17(NBIOT_CellId_Type p_CellId, SystemInformationBlockType31_NB_r17.servingSatelliteInfo_r17.ul_SyncValidityDuration_r17 p_UL_SyncValidityDuration_r17) runs on NBIOT_PTC {//@sic R5s240707 add changes sic@ var SystemInformationBlockType31_NB_r17 v_SIB31_NB; v_SIB31_NB := f_NBIOT_CellInfo_GetSIB31(p_CellId); v_SIB31_NB.servingSatelliteInfo_r17.ul_SyncValidityDuration_r17 := p_UL_SyncValidityDuration_r17; f_NBIOT_CellInfo_SetSIB31(p_CellId, v_SIB31_NB); }