3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250419

# Online,  , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-5 | CR | 0020 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MCPTT test case 5.3 to the MCX ATS with MCX-EUTRA model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add MCPTT TC 5.3 to the MCX ATS on MCX- EUTRA test model |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to the TTCN code required to obtain a pass using the MCX- EUTRA test model. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The testcase will not be added to MCX ATS with MCX-EUTRA model |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.MCPTT |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 37.579-1 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	6

3	Branches executed	9

4	Execution Log Files	10

4.1	Alea - a Leonardo Company, MCXPTT Platform v. 4.2.2 - Android Client v 1.12.1	10

5	References	10

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 5.3.MCPTT which is part of the MCX_EUTRA test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

## Verification Test Summary

Test Case: 	5.3.MCPTT

ATS Version:	iwd-TTCN3-B2024-06_D25wk38

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Alea - a Leonardo Company, MCXPTT Platform v. 4.2.2 - Android Client v 1.12.1 on COTS 5G UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_MCX_XML_DecryptAttributeUri |
| --- | --- |
| Reason for change | Current implementation use "mc1-encryption.3gppnetwork.org” as a hostname for decrypting the XML attributes. This is as per TS 37.579 Table 5.5.13.3-1, which refers to TS 24.379, cl. 6.6.2.3.4 and TS 23.003, cl. 26.2. While in TS 24.379 cl 6.6.2.3.4, it states “the string "@" followed by the domain name for MCPTT confidentiality protection as specified in 3GPP TS 23.203” As per TS 23.003 cl. 26.2: A Domain Name for MC Services confidentiality protection used in a host part of a SIP URI indicates that the user part of the SIP URI contains a confidentiality protected MC Services identity. This Domain Name shall be the string "mc1-encrypted.3gppnetwork.org". |
| Summary of change | Corrected the hostname as “mc1-encrypted.3gppnetwork.org" for decrypting the XML attributes. Note – prose CR is required to correct TS 37.579 Table 5.5.13.3-1 |
| TTCN module | MCX_XML_Encryption.ttcn |
| MCC160 Comment |  |

Before Change:

function f_MCX_XML_DecryptAttributeUri(universal charstring p_URI, charstring p_MCServiceID, template (omit) MCX_SecurityKey_Type p_XPK) return universal charstring { var MCX_SecurityKey_Type v_XPK; var template UserInfo v_UserInfo := ?; var template (present) charstring v_Host := "mc1-encryption.3gppnetwork.org"; var template (present) UriComponents v_SipUriComponents := cr_SipUriComponent(v_UserInfo, v_Host); var template SemicolonParam_List v_UriParameters := omit; var charstring v_CharString := f_UniCharstring2Charstring(p_URI); var SipUrl v_SipUrl := f_SIP_ParseURI(v_CharString); var CharStringList_Type v_UserInfoParts; var bitstring v_CipherKey; var octetstring v_EncryptedURI; var octetstring v_IV; if (isvalue(p_XPK) and tsc_MCX_XML_ConfidentialityProtection) { v_XPK := valueof(p_XPK); if (match(v_SipUrl, cr_SipUrl_Common(c_sipScheme, v_SipUriComponents, v_UriParameters))) { v_UserInfoParts := fl_MCX_XML_EncryptedUriUserInfo_GetParamValueList(v_SipUrl.components.sip.userInfo.userOrTelephoneSubscriber, v_XPK); if (lengthof(v_UserInfoParts) > 0) { v_CipherKey := f_MCX_XML_GenerateProtectionKey(v_XPK, tsc_MCX_FC_Confidentiality, p_MCServiceID); v_EncryptedURI := bit2oct(f_Base64Decode(v_UserInfoParts[0])); v_IV := bit2oct(f_Base64Decode(v_UserInfoParts[1])); return oct2char(fx_XML_Decrypt(v_EncryptedURI, AES_128_GCM, v_CipherKey, v_IV)); /* according to 24.379 clause 6.6.2.3.4 the encryption is as specified in RFC 5288; RFC 5288 clause 3 refers to AEAD_AES_128_GCM and AEAD_AES_256_GCM as described in RFC 5116; according RFC 5116 clause 5.1 the ciphertext consists of the ciphered data and the authentication tag */ } } return ""; } return p_URI; }

After Change:

function f_MCX_XML_DecryptAttributeUri(universal charstring p_URI, charstring p_MCServiceID, template (omit) MCX_SecurityKey_Type p_XPK) return universal charstring { var MCX_SecurityKey_Type v_XPK; var template UserInfo v_UserInfo := ?; var template (present) charstring v_Host := "mc1-encrypted.3gppnetwork.org"; //TS23.003 Clause 26.2 var template (present) UriComponents v_SipUriComponents := cr_SipUriComponent(v_UserInfo, v_Host); var template SemicolonParam_List v_UriParameters := omit; var charstring v_CharString := f_UniCharstring2Charstring(p_URI); var SipUrl v_SipUrl := f_SIP_ParseURI(v_CharString); var CharStringList_Type v_UserInfoParts; var bitstring v_CipherKey; var octetstring v_EncryptedURI; var octetstring v_IV; if (isvalue(p_XPK) and tsc_MCX_XML_ConfidentialityProtection) { v_XPK := valueof(p_XPK); if (match(v_SipUrl, cr_SipUrl_Common(c_sipScheme, v_SipUriComponents, v_UriParameters))) { v_UserInfoParts := fl_MCX_XML_EncryptedUriUserInfo_GetParamValueList(v_SipUrl.components.sip.userInfo.userOrTelephoneSubscriber, v_XPK); if (lengthof(v_UserInfoParts) > 0) { v_CipherKey := f_MCX_XML_GenerateProtectionKey(v_XPK, tsc_MCX_FC_Confidentiality, p_MCServiceID); v_EncryptedURI := bit2oct(f_Base64Decode(v_UserInfoParts[0])); v_IV := bit2oct(f_Base64Decode(v_UserInfoParts[1])); return oct2char(fx_XML_Decrypt(v_EncryptedURI, AES_128_GCM, v_CipherKey, v_IV)); /* according to 24.379 clause 6.6.2.3.4 the encryption is as specified in RFC 5288; RFC 5288 clause 3 refers to AEAD_AES_128_GCM and AEAD_AES_256_GCM as described in RFC 5116; according RFC 5116 clause 5.1 the ciphertext consists of the ciphered data and the authentication tag */ } } return ""; } return p_URI; }

## Change 2

| Function name | f_MCX_XML_EncryptAttributeUri |
| --- | --- |
| Reason for change | Same as change 2.1 |
| Summary of change | Corrected the hostname as “mc1-encrypted.3gppnetwork.org" for encrypting the XML attributes. |
| TTCN module | MCX_XML_Encryption.ttcn |
| MCC160 Comment |  |

Before Change:

function f_MCX_XML_EncryptAttributeUri(universal charstring p_URI, charstring p_MCServiceID, template (omit) MCX_SecurityKey_Type p_CSK) return universal charstring { var MCX_SecurityKey_Type v_CSK; var bitstring v_CipherKey; var octetstring v_IV; var octetstring v_OctetString; var CharStringList_Type v_UserInfoParts; if (isvalue(p_CSK) and tsc_MCX_XML_ConfidentialityProtection) { v_CSK := valueof(p_CSK); v_CipherKey := f_MCX_XML_GenerateProtectionKey(v_CSK, tsc_MCX_FC_Confidentiality, p_MCServiceID); v_IV := '5A5A5A5A5A5A5A5A5A5A5A5A'O; // arbitrarily selected v_OctetString := fx_XML_Encrypt(unichar2oct(p_URI), AES_128_GCM, v_CipherKey, v_IV); v_UserInfoParts := { f_Base64Encode(oct2bit(v_OctetString)), "iv=" & f_Base64Encode(oct2bit(v_IV)), "key-id=" & f_Base64Encode(v_CSK.Id), "alg=128-aes-gcm" }; return "sip:" & f_StringJoin(v_UserInfoParts, ";") & "@mc1-encryption.3gppnetwork.org"; } return p_URI; }

After Change:

function f_MCX_XML_EncryptAttributeUri(universal charstring p_URI, charstring p_MCServiceID, template (omit) MCX_SecurityKey_Type p_CSK) return universal charstring { var MCX_SecurityKey_Type v_CSK; var bitstring v_CipherKey; var octetstring v_IV; var octetstring v_OctetString; var CharStringList_Type v_UserInfoParts; if (isvalue(p_CSK) and tsc_MCX_XML_ConfidentialityProtection) { v_CSK := valueof(p_CSK); v_CipherKey := f_MCX_XML_GenerateProtectionKey(v_CSK, tsc_MCX_FC_Confidentiality, p_MCServiceID); v_IV := '5A5A5A5A5A5A5A5A5A5A5A5A'O; // arbitrarily selected v_OctetString := fx_XML_Encrypt(unichar2oct(p_URI), AES_128_GCM, v_CipherKey, v_IV); v_UserInfoParts := { f_Base64Encode(oct2bit(v_OctetString)), "iv=" & f_Base64Encode(oct2bit(v_IV)), "key-id=" & f_Base64Encode(v_CSK.Id), "alg=128-aes-gcm" }; return "sip:" & f_StringJoin(v_UserInfoParts, ";") & "@mc1-encrypted.3gppnetwork.org"; //TS23.003 Clause 26.2 } return p_URI; }

## Change 3

| Function name | f_MCX_5_3_IMS |
| --- | --- |
| Reason for change | When the UE initialise de-affiliation, the UE sends SIP PUBLISH request , with the Affiliation element shall be included in tuple status extension during affiliation status change(de-affiliation) procedure. As per TS 37.579 Table 5.5.3.5.1-1, affiliation shall be included on condition AFFILIATION, reference TS 24.379 clause 9.3.1 Also as per TS 24.379. cl. 9.2.1.2 Affiliation status change procedure In the SIP PUBLISH request, the MCPTT client: … … 6) shall include an application/pidf+xml MIME body indicating per-user affiliation information according to subclause 9.3.1. In the MIME body, the MCPTT client: a) shall include all MCPTT groups where the targeted MCPTT user indicates its interest at the targeted MCPTT client; b) shall include the MCPTT client ID of the targeted MCPTT client; c) shall not include the "status" attribute and the "expires" attribute in the <affiliation> element; and d) shall set the <p-id> child element of the <presence> root element to a globally unique value. |
| Summary of change | Added the affiliation element in tuple status extension for affiliation status change(de-affiliation) procedure. |
| TTCN module | MCX_ClientConfiguration_MCX_IMS.ttcn |
| MCC160 Comment |  |

Before Change:

function f_MCX_5_3_IMS(MCX_ServiceType_Type p_ServiceType) runs on MCX_IMS_PTC { … … … // @siclog "Step 4C" siclog@ f_UT_MCX_UserAction(MMI, "Make the UE (" & v_ServiceName & " client) request de-affiliation of user A from group A"); // @siclog "Step 4D - 4I" siclog@ v_P_id := f_MCX_Affiliation_Publish(p_ServiceType, v_AffiliationInfo_A, tsc_MCX_Expires0); /* Table 5.3.3.3-21: SIP PUBLISH, Table 5.3.3.3-22: MCPTT-INFO, Table 5.3.3.3-23: PIDF */ f_MCX_Affiliation_Notification(p_ServiceType, v_AffiliationInfo_A, deaffiliating, v_P_id); /* Table 5.3.3.3-47: SIP NOTIFY, Table 5.3.3.3-48: PIDF */ f_MCX_Affiliation_Notification(p_ServiceType, v_AffiliationInfo_A, omit, v_P_id); /* Table 5.3.3.3-46: SIP NOTIFY, Table 5.3.3.3-45: PIDF */ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4D"); // @siclog "Step 4J" siclog@ f_MCX_UserCheckPassFail(__FILE__, __LINE__, "Does the UE (" & v_ServiceName & " client) notify the user of being no longer affiliated with group A?", "Step 4J"); … … … // @siclog "Step 20" siclog@ f_UT_MCX_UserAction(MMI, "Make the UE (" & v_ServiceName & " client) send an affiliation status change request to de-affiliate targeted user B from group A (mandatory mode)"); // @siclog "Step 21 - 23C" siclog@ v_P_id := f_MCX_Affiliation_Publish(p_ServiceType, v_AffiliationInfo_B, tsc_MCX_Expires0); /* Table 5.3.3.3-18: SIP PUBLISH, Table 5.3.3.3-19: MCPTT-INFO, Table 5.3.3.3-20: PIDF */ f_MCX_Affiliation_Notification(p_ServiceType, v_AffiliationInfo_B, deaffiliating, v_P_id); /* Table 5.3.3.3-34: SIP NOTIFY, Table 5.3.3.3-35: PIDF */ f_MCX_Affiliation_Notification(p_ServiceType, v_AffiliationInfo_B, omit, v_P_id); /* Table 5.3.3.3-46: SIP NOTIFY, Table 5.3.3.3-45: PIDF */ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 21"); // @siclog "Step 24" siclog@ f_MCX_UserCheckPassFail(__FILE__, __LINE__, "Does the UE (" & v_ServiceName & " client) notify the user of the target user (user B) being no longer affiliated with group A?", "Step 24"); … … … // @siclog "Step 34" siclog@ f_UT_MCX_UserAction(MMI, "Make the UE (" & v_ServiceName & " client) request de-affiliation of user A from group A"); // @siclog "Step 35 - 37C" siclog@ v_P_id := f_MCX_Affiliation_Publish(p_ServiceType, v_AffiliationInfo_A, tsc_MCX_Expires0); /* Table 5.3.3.3-21: SIP PUBLISH, Table 5.3.3.3-22: MCPTT-INFO, Table 5.3.3.3-23: PIDF */ f_MCX_Affiliation_Notification(p_ServiceType, v_AffiliationInfo_A, deaffiliating, v_P_id); /* Table 5.3.3.3-36: SIP NOTIFY, Table 5.3.3.3-37: PIDF */ f_MCX_Affiliation_Notification(p_ServiceType, v_AffiliationInfo_A, omit, v_P_id); /* Table 5.3.3.3-46: SIP NOTIFY, Table 5.3.3.3-45: PIDF */ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 35"); // @siclog "Step 38" siclog@ f_MCX_UserCheckPassFail(__FILE__, __LINE__, "Does the UE (" & v_ServiceName & " client) notify the user of being no longer affiliated with group A?", "Step 38"); … … … }

After Change:

function f_MCX_5_3_IMS(MCX_ServiceType_Type p_ServiceType) runs on MCX_IMS_PTC { … … … // @siclog "Step 4C" siclog@ f_UT_MCX_UserAction(MMI, "Make the UE (" & v_ServiceName & " client) request de-affiliation of user A from group A"); // @siclog "Step 4D - 4I" siclog@ v_P_id := f_MCX_Affiliation_Publish(p_ServiceType, v_AffiliationInfo_A, tsc_MCX_Expires0, v_WithAffiliation); /* Table 5.3.3.3-21: SIP PUBLISH, Table 5.3.3.3-22: MCPTT-INFO, Table 5.3.3.3-23: PIDF */ f_MCX_Affiliation_Notification(p_ServiceType, v_AffiliationInfo_A, deaffiliating, v_P_id); /* Table 5.3.3.3-47: SIP NOTIFY, Table 5.3.3.3-48: PIDF */ f_MCX_Affiliation_Notification(p_ServiceType, v_AffiliationInfo_A, omit, v_P_id); /* Table 5.3.3.3-46: SIP NOTIFY, Table 5.3.3.3-45: PIDF */ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4D"); // @siclog "Step 4J" siclog@ f_MCX_UserCheckPassFail(__FILE__, __LINE__, "Does the UE (" & v_ServiceName & " client) notify the user of being no longer affiliated with group A?", "Step 4J"); … … … // @siclog "Step 20" siclog@ f_UT_MCX_UserAction(MMI, "Make the UE (" & v_ServiceName & " client) send an affiliation status change request to de-affiliate targeted user B from group A (mandatory mode)"); // @siclog "Step 21 - 23C" siclog@ v_P_id := f_MCX_Affiliation_Publish(p_ServiceType, v_AffiliationInfo_B, tsc_MCX_Expires0, v_WithAffiliation); /* Table 5.3.3.3-18: SIP PUBLISH, Table 5.3.3.3-19: MCPTT-INFO, Table 5.3.3.3-20: PIDF */ f_MCX_Affiliation_Notification(p_ServiceType, v_AffiliationInfo_B, deaffiliating, v_P_id); /* Table 5.3.3.3-34: SIP NOTIFY, Table 5.3.3.3-35: PIDF */ f_MCX_Affiliation_Notification(p_ServiceType, v_AffiliationInfo_B, omit, v_P_id); /* Table 5.3.3.3-46: SIP NOTIFY, Table 5.3.3.3-45: PIDF */ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 21"); // @siclog "Step 24" siclog@ f_MCX_UserCheckPassFail(__FILE__, __LINE__, "Does the UE (" & v_ServiceName & " client) notify the user of the target user (user B) being no longer affiliated with group A?", "Step 24"); … … … // @siclog "Step 34" siclog@ f_UT_MCX_UserAction(MMI, "Make the UE (" & v_ServiceName & " client) request de-affiliation of user A from group A"); // @siclog "Step 35 - 37C" siclog@ v_P_id := f_MCX_Affiliation_Publish(p_ServiceType, v_AffiliationInfo_A, tsc_MCX_Expires0, v_WithAffiliation); /* Table 5.3.3.3-21: SIP PUBLISH, Table 5.3.3.3-22: MCPTT-INFO, Table 5.3.3.3-23: PIDF */ f_MCX_Affiliation_Notification(p_ServiceType, v_AffiliationInfo_A, deaffiliating, v_P_id); /* Table 5.3.3.3-36: SIP NOTIFY, Table 5.3.3.3-37: PIDF */ f_MCX_Affiliation_Notification(p_ServiceType, v_AffiliationInfo_A, omit, v_P_id); /* Table 5.3.3.3-46: SIP NOTIFY, Table 5.3.3.3-45: PIDF */ f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 35"); // @siclog "Step 38" siclog@ f_MCX_UserCheckPassFail(__FILE__, __LINE__, "Does the UE (" & v_ServiceName & " client) notify the user of being no longer affiliated with group A?", "Step 38"); … … … }

# Branches executed

The test case was executed with

- TLSv1.2

- IPSec ipsec-3gpp;alg=hmac-sha-1-96;ealg=aes-cbc

- XML integrity and encrpytion according to  http://www.w3.org/2009/xmlenc11#aes128-gcm

- XML signatures according to  http://www.w3.org/2001/04/xmlenc#sha256

- MIKEY SAKKE/ECCSI (RFC 6507, 6508, 6509)

- LTE FDD Band 5

- Ciphering algorithm eea1 and Integrity algorithm eia1

# Execution Log Files

## Alea - a Leonardo Company, MCXPTT Platform v. 4.2.2 - Android Client v 1.12.1

The Alea - a Leonardo Company, MCXPTT Platform v. 4.2.2 - Android Client v 1.12.1 passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_5_3_MCPTT_LOG.txt

- PICS/PIXIT parameter file:

TC_5_3_MCPTT_PIXIT.xml

# References

| [1] | R5s250420 : Supporting information for Addition of MCPTT test case 5.3 to the MCX ATS with MCX-EUTRA model |
| --- | --- |