3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250430

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3747 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC PEI testcase 8.1.1.1a.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_UE_pow_sav_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-09-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Since Prose CR R5-254147 at RAN5#108 got agreed, the changes proposed in TTCN CR R5s250221 have also been accepted. However, the changes proposed in Change 3, point 2 of R5s250221 have not been implemented in IWD 25wk38 TTCN. “The function f_NR_Paging_Trigger_DCI_2_7 transmitting the DCI 2_7 at Steps 3, 13 and 18 need to be modified to consider the RRC Resume procedure and for the negative check for Step 18, only the PEI should be transmitted to the UE otherwise the Paging following the PEI would be considered as Normal Paging since the noLastCellUpdate-r17 has been set to TRUE in the RRC Release message and the UE responds with a RRC Resume Request failing the check at Step 19. ” The boolean flag p_ResumeProc needs to be set to TRUE |  |  |  |  |  |  |  |  |
| Summary of change: |  | Re-implemented the change 3 point 2 in R5s250221 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.1a.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:		Bhise Parikshit

parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | fl_TC_8_1_1_1a_2_TestBody |
| --- | --- |
| Reason for change | Since Prose CR R5-254147 at RAN5#108 got agreed, the changes proposed in TTCN CR R5s250221 have also been accepted. However, the changes proposed in Change 3, point 2 of R5s250221 have not been implemented in IWD 25wk38 TTCN. “The function f_NR_Paging_Trigger_DCI_2_7 transmitting the DCI 2_7 at Steps 3, 13 and 18 need to be modified to consider the RRC Resume procedure and for the negative check for Step 18, only the PEI should be transmitted to the UE otherwise the Paging following the PEI would be considered as Normal Paging since the noLastCellUpdate-r17 has been set to TRUE in the RRC Release message and the UE responds with a RRC Resume Request failing the check at Step 19. ” The boolean flag p_ResumeProc needs to be set to TRUE |
| Summary of change | Re-implemented the change 3 point 2 in R5s250221 |
| TTCN module | RRC_PEI_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_8_1_1_1a_2_TestBody() runs on NR5GC_PTC { <<SKIPPED CODE>> //@siclog "Step 3" siclog@ //The SS transmits a PEI and the Paging message including a matched identity on the associated PO only. f_NR_Paging_Trigger_DCI_2_7(nr_Cell1, -,'11'B, 22 ); <<SKIPPED CODE>> //@siclog "Step 13" siclog@ //The SS transmits a PEI and the Paging message including a matched identity on the associated PO only on NR Cell 1. f_NR_Paging_Trigger_DCI_2_7(nr_Cell1, -,'11'B, 22 ); <<SKIPPED CODE>> //@siclog "Step 18" siclog@ //The SS transmits a PEI and the Paging message including a matched identity on the associated PO only.. f_NR_Paging_Trigger_DCI_2_7(nr_Cell2, -,'11'B, 22 ); <<SKIPPED CODE>>

After Change

function fl_TC_8_1_1_1a_2_TestBody() runs on NR5GC_PTC { <<SKIPPED CODE>> //@siclog "Step 3" siclog@ //The SS transmits a PEI and the Paging message including a matched identity on the associated PO only. f_NR_Paging_Trigger_DCI_2_7(nr_Cell1, -,'11'B, 22, true ); //WA# <<SKIPPED CODE>> //@siclog "Step 13" siclog@ //The SS transmits a PEI and the Paging message including a matched identity on the associated PO only on NR Cell 1. f_NR_Paging_Trigger_DCI_2_7(nr_Cell2, -,'11'B, 22, true ); //WA# <<SKIPPED CODE>> //@siclog "Step 18" siclog@ //The SS transmits a PEI and the Paging message including a matched identity on the associated PO only.. f_NR_Paging_Trigger_DCI_2_7(nr_Cell2, -,'11'B, 22, -, false); //WA# <<SKIPPED CODE>>