3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250469

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3768 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the function f_IMS_NR5GC_MTSpeechCall |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per TS 38.508-1, Table 4.9.16.2.2-1, step-10(The SS transmits an RRCReconfiguration message and a PDU SESSION MODIFICATION COMMAND message) will be executed between step-5 and step-6 of Annex A.5.1a. This part is not ensured in TTCN code |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the function f_IMS_NR5GC_MTSpeechCall to move f_IMS_IPCAN_WaitForTrigger at correct place. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5.1.NR5GC, 11.5.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Correction in Correction in f_IMS_NR5GC_MTSpeechCall	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Correction in Correction in f_IMS_NR5GC_MTSpeechCall

| Template name | Correction in f_IMS_NR5GC_MTSpeechCall |
| --- | --- |
| Reason for change | As per TS 38.508-1, Table 4.9.16.2.2-1, step-10(The SS transmits an RRCReconfiguration message and a PDU SESSION MODIFICATION COMMAND message) will be executed between step-5 and step-6 of Annex A.5.1a. This part is not ensured in TTCN code. |
| Summary of change | Updated the function f_IMS_NR5GC_MTSpeechCall to move f_IMS_IPCAN_WaitForTrigger at correct place. |
| TTCN module | IMS_NR5GC_Functions.ttcn |

Before Change:

function f_IMS_NR5GC_MTSpeechCall() runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var IMS_DATA_RSP v_IMS_DATA_RSP; f_IMS_IPCAN_WaitForTrigger(IPCAN, "Start MT Speech IMS"); // @sic R5-225362 sic@ v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_X_Step1(pc_Preconditions); // @sic R5-246483: EVS_A2 instead of EVS_B0_A1 sic@ v_ResponseWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_X_Step2_5(v_InviteRequestWithSdp, pc_Preconditions); // @sic R5-246483: EVS_A2 instead of EVS_B0_A1 sic@ f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Add Voice Bearer")); // @sic R5-225362 sic@ if (pc_Preconditions) { f_IMS_MTCallSetup_NR5GC_A_5_1a_Step6_7(v_InviteRequestWithSdp, v_ResponseWithSdp); } a_IMS_MTCallSetup_Receive180Ringing(v_InviteRequestWithSdp, -, v_IMS_DATA_RSP); f_IMS_IPCAN_WaitForTrigger(IPCAN, "Voice Bearer Added"); // @sic R5s250362 sic@ f_IMS_MTCallSetup_AcceptAndAcknowledge(v_InviteRequestWithSdp); // @sic R5s250362 sic@ // Call established f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Voice Call Established")); // @sic R5-225362 sic@ }

After Change:

function f_IMS_NR5GC_MTSpeechCall() runs on IMS_PTC { var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var IMS_DATA_RSP v_IMS_DATA_RSP; f_IMS_IPCAN_WaitForTrigger(IPCAN, "Start MT Speech IMS"); // @sic R5-225362 sic@ v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_X_Step1(pc_Preconditions); // @sic R5-246483: EVS_A2 instead of EVS_B0_A1 sic@ v_ResponseWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_X_Step2_5(v_InviteRequestWithSdp, pc_Preconditions); // @sic R5-246483: EVS_A2 instead of EVS_B0_A1 sic@ f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Add Voice Bearer")); // @sic R5-225362 sic@ f_IMS_IPCAN_WaitForTrigger(IPCAN, "Voice Bearer Added"); // @sic R5s250362 sic@ if (pc_Preconditions) { f_IMS_MTCallSetup_NR5GC_A_5_1a_Step6_7(v_InviteRequestWithSdp, v_ResponseWithSdp); } a_IMS_MTCallSetup_Receive180Ringing(v_InviteRequestWithSdp, -, v_IMS_DATA_RSP); f_IMS_MTCallSetup_AcceptAndAcknowledge(v_InviteRequestWithSdp); // @sic R5s250362 sic@ // Call established f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("Voice Call Established")); // @sic R5-225362 sic@ }