3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250476

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4946 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE EPS mobility test case 9.2.3.3.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-11-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the Table 9.2.3.3.5.3.2-1 [36.523-1] at Step 21a2/21b2 UE is expected to send RAU request before the timer T3323 expires. Since the timer is not stopped immediately UE may unfairly fail the testcase even if it transmits the RAU request before the timer expires. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to stop the timer T3323 immediately once the RAU request is received on the SS side. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	6

2.3	Change 3	9

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:	Narendra Kalahasti
                         Narendra.kalahasti@anritsu.com

- Corrections required

  - Change 1

| Function name | fl_TC_9_2_3_3_5_Body |
| --- | --- |
| Reason for change | As per the Table 9.2.3.3.5.3.2-1 [36.523-1] at Step 21a2/21b2 UE is expected to send RAU request before the timer T3323 expires. Since the timer is not stopped immediately UE may unfairly fail the testcase even if it transmits the RAU request before the timer expires. |
| Summary of change | It is proposed to stop the timer T3323 immediately once the RAU request is received on the SS side. |
| TTCN module | LTE_IRAT\9_2\NAS_TrackingArea_UG.ttcn |

Before Change:

function fl_TC_9_2_3_3_5_Body() runs on EUTRA_PTC { var charstring v_OtherPTC := ""; var template (omit) EPS_BearerContextStatus v_EpsBearerCtxtStatus; var NAS_MSG_Indication_Type v_NasInd; var GutiParameters_Type v_Guti1_Params := f_EUTRA_CellInfo_GetGuti(eutra_CellA); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId(v_Guti1_Params.PLMN_Identity); var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var NAS_Lac v_LAC := f_EUTRA_CellInfo_GetLocationAreaCode(eutra_CellA); var template (omit) LocAreaId v_LAI := f_GetLAI(v_PLMN, v_LAC, NORMAL); // @sic R5s110176 sic@ var NAS_Tac v_Tac1 := '0001'O; var template (omit) MobileIdentity v_MSId := f_GetMSId(NORMAL); // @sic R5s110176 sic@ var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType(); var EPS_UpdateTypeValue v_UpdateType := f_GetEPSTAUType(NORMAL); // @sic R5-145220 sic@ var EPS_UpdateTypeValue v_UpdateResult; var NAS_KsiValue v_KsiValue := f_EUTRA_SecurityKSIasme_Get(); var float v_Delay1Min := f_EUTRA_SetTimerToleranceMax(eutra_CellA, nonProtocolTimer, 60.0); var float v_Delay2Min := f_EUTRA_SetTimerToleranceMax(eutra_CellA, nonProtocolTimer, 120.0); timer t_T3312 := f_EUTRA_SetTimerToleranceMin(eutra_CellA, nasTimer, 7.0 * 60.0); // Periodic Routing Area Update timer on UTRAN timer t_T3323 := f_EUTRA_SetTimerToleranceMax(eutra_CellA, nasTimer, 7.0 * 60.0); // @sic R5-120768, R5s120210 sic@ timer t_Wait := v_Delay1Min; // SKIPPED CODE //@siclog "Step 21" siclog@ select (v_OtherPTC) { // send trigger to other RAT to change its power level too case ("UTRAN") { f_EUTRA_SetCellPower(eutra_CellA, tsc_NonSuitableCellRS_EPRE); f_IRAT_SendCoOrd(UTRAN, cms_IRAT_Trigger); f_IRAT_WaitForCoOrd_Trigger(UTRAN); // wait for trigger from UTRAN side } case ("GERAN") { f_EUTRA_SetCellPower(eutra_CellA, tsc_NonSuitableCellRS_EPRE); f_IRAT_SendCoOrd(GERAN, cms_IRAT_Trigger); f_IRAT_WaitForCoOrd_Trigger(GERAN); // wait for trigger from GERAN side } } t_T3323.stop; // @sic R5-120768 sic@ }// function fl_tc_9_2_3_3_5_Body

After Change:

function fl_TC_9_2_3_3_5_Body() runs on EUTRA_PTC { var charstring v_OtherPTC := ""; var template (omit) EPS_BearerContextStatus v_EpsBearerCtxtStatus; var NAS_MSG_Indication_Type v_NasInd; var GutiParameters_Type v_Guti1_Params := f_EUTRA_CellInfo_GetGuti(eutra_CellA); var NAS_PlmnId v_PLMN := f_Asn2Nas_PlmnId(v_Guti1_Params.PLMN_Identity); var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var NAS_Lac v_LAC := f_EUTRA_CellInfo_GetLocationAreaCode(eutra_CellA); var template (omit) LocAreaId v_LAI := f_GetLAI(v_PLMN, v_LAC, NORMAL); // @sic R5s110176 sic@ var NAS_Tac v_Tac1 := '0001'O; var template (omit) MobileIdentity v_MSId := f_GetMSId(NORMAL); // @sic R5s110176 sic@ var template AdditionalUpdateType v_AdditionalUpdateType := f_GetAdditionalUpdateType(); var EPS_UpdateTypeValue v_UpdateType := f_GetEPSTAUType(NORMAL); // @sic R5-145220 sic@ var EPS_UpdateTypeValue v_UpdateResult; var NAS_KsiValue v_KsiValue := f_EUTRA_SecurityKSIasme_Get(); var float v_Delay1Min := f_EUTRA_SetTimerToleranceMax(eutra_CellA, nonProtocolTimer, 60.0); var float v_Delay2Min := f_EUTRA_SetTimerToleranceMax(eutra_CellA, nonProtocolTimer, 120.0); timer t_T3312 := f_EUTRA_SetTimerToleranceMin(eutra_CellA, nasTimer, 7.0 * 60.0); // Periodic Routing Area Update timer on UTRAN timer t_T3323 := f_EUTRA_SetTimerToleranceMax(eutra_CellA, nasTimer, 7.0 * 60.0); // @sic R5-120768, R5s120210 sic@ timer t_Wait := v_Delay1Min; // SKIPPED CODE //@siclog "Step 21" siclog@ select (v_OtherPTC) { // send trigger to other RAT to change its power level too case ("UTRAN") { f_EUTRA_SetCellPower(eutra_CellA, tsc_NonSuitableCellRS_EPRE); f_IRAT_SendCoOrd(UTRAN, cms_IRAT_Trigger); f_IRAT_WaitForCoOrd_Trigger(UTRAN); // wait for trigger from UTRAN side t_T3323.stop; f_IRAT_SendCoOrd(GERAN, cms_IRAT_Trigger); f_IRAT_WaitForCoOrd_Trigger(GERAN); // wait for trigger from GERAN side } case ("GERAN") { f_EUTRA_SetCellPower(eutra_CellA, tsc_NonSuitableCellRS_EPRE); f_IRAT_SendCoOrd(GERAN, cms_IRAT_Trigger); f_IRAT_WaitForCoOrd_Trigger(GERAN); // wait for trigger from GERAN side t_T3323.stop; f_IRAT_SendCoOrd(GERAN, cms_IRAT_Trigger); f_IRAT_WaitForCoOrd_Trigger(GERAN); // wait for trigger from GERAN side } } }// function fl_tc_9_2_3_3_5_Body

  - Change 2

| Function name | f_TC_9_2_3_3_5_GERAN |
| --- | --- |
| Reason for change | As per the Table 9.2.3.3.5.3.2-1 [36.523-1] at Step 21a2/21b2 UE is expected to send RAU request before the timer T3323 expires. Since the timer is not stopped immediately UE may unfairly fail the testcase even if it transmits the RAU request before the timer expires. |
| Summary of change | It is proposed to stop the timer T3323 immediately once the RAU request is received on the SS side. |
| TTCN module | LTE_IRAT\9_2\NAS_TrackingArea_UG_GERAN.ttcn |

Before Change:

function f_TC_9_2_3_3_5_GERAN () runs on GERAN_PTC { /* Periodic Routing Area Update (multi RAT test) */ // SKIPPED CODE //@siclog "Step 21b1" siclog@ f_GERAN_SetCellPowerLevel (geran_Cell24, tsc_GERAN_ServingCell); //@siclog "Step 21b2" siclog@ v_LLCDataInd := f_GPRS_ReceiveUplinkMsg (geran_Cell24, car_G_LLC_UnitData_IND(cr_G_RAUReq_Ids(cr_GMM_UpdateType(?, v_UpdateType), // @sic R5s120210 sic@ cr_RAI, cr_PTMSI_SignatureAny ifpresent, ?, cdr_MobileIdTMSI_tlv ifpresent, // @sic R5s110607, R5-126068 sic@ cr_MobileIdAny('1A'O) ifpresent, cr_RoutingAreaId2 ifpresent, v_UE_NetworkCap))); // @sic R5s110607, R5s120210 sic@ f_GERAN_PreliminaryPass (__FILE__, __LINE__, "Step 21b2"); // Check ISR support if (v_LLCDataInd.unit.msg.rauReq.msNetworkCap.isrSupport != '1'B) { // @sic R5s120830 sic@ f_GERAN_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21b2"); } //@siclog "Step 21b3" siclog@ v_RAUAccept.rauAccept.updateResult.ttcn_value := v_UpdateResult; v_RAUAccept.rauAccept.pDP_ContextStatus:= cs_PDP_ContextStatusInactive; // @sic R5s180517 sic@ f_GPRS_SendDownlinkMsg(geran_Cell24, 1, cas_G_LLC_UnitData_Req (f_GERAN_TLLI_Get(geran_Cell24), tsc_LLCSapi_GMM, tsc_LLC_PM, px_GSM_CipheringOnOff, v_RAUAccept)); f_GERAN_LLC_XID (geran_Cell24, false); // @sic R5s141137 sic@ //@siclog "Step 21b4 Void" siclog@ //@siclog "Step 21b5" siclog@ f_IRAT_SendCoOrd (EUTRA, cms_IRAT_Trigger); // Send trigger to EUTRA side to release cell // Test case now finished on GERAN side f_GERAN_TestBody_Set(false); f_GERAN_Postamble(geran_Cell24, G1_IDLE); } // f_TC_9_2_3_3_5_GERAN

After Change:

function f_TC_9_2_3_3_5_GERAN () runs on GERAN_PTC { /* Periodic Routing Area Update (multi RAT test) */ // SKIPPED CODE //@siclog "Step 21b1" siclog@ f_GERAN_SetCellPowerLevel (geran_Cell24, tsc_GERAN_ServingCell); //@siclog "Step 21b2" siclog@ v_LLCDataInd := f_GPRS_ReceiveUplinkMsg (geran_Cell24, car_G_LLC_UnitData_IND(cr_G_RAUReq_Ids(cr_GMM_UpdateType(?, v_UpdateType), // @sic R5s120210 sic@ cr_RAI, cr_PTMSI_SignatureAny ifpresent, ?, cdr_MobileIdTMSI_tlv ifpresent, // @sic R5s110607, R5-126068 sic@ cr_MobileIdAny('1A'O) ifpresent, cr_RoutingAreaId2 ifpresent, v_UE_NetworkCap))); // @sic R5s110607, R5s120210 sic@ f_GERAN_PreliminaryPass (__FILE__, __LINE__, "Step 21b2"); f_IRAT_SendCoOrd (EUTRA, cms_IRAT_Trigger); // Send trigger to EUTRA side // Wait for EUTRA to stop timer t_T3323 f_IRAT_WaitForCoOrd_Trigger (EUTRA); // Wait trigger from EUTRAN side // Check ISR support if (v_LLCDataInd.unit.msg.rauReq.msNetworkCap.isrSupport != '1'B) { // @sic R5s120830 sic@ f_GERAN_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21b2"); } //@siclog "Step 21b3" siclog@ v_RAUAccept.rauAccept.updateResult.ttcn_value := v_UpdateResult; v_RAUAccept.rauAccept.pDP_ContextStatus:= cs_PDP_ContextStatusInactive; // @sic R5s180517 sic@ f_GPRS_SendDownlinkMsg(geran_Cell24, 1, cas_G_LLC_UnitData_Req (f_GERAN_TLLI_Get(geran_Cell24), tsc_LLCSapi_GMM, tsc_LLC_PM, px_GSM_CipheringOnOff, v_RAUAccept)); f_GERAN_LLC_XID (geran_Cell24, false); // @sic R5s141137 sic@ //@siclog "Step 21b4 Void" siclog@ //@siclog "Step 21b5" siclog@ f_IRAT_SendCoOrd (EUTRA, cms_IRAT_Trigger); // Send trigger to EUTRA side to release cell // Test case now finished on GERAN side f_GERAN_TestBody_Set(false); f_GERAN_Postamble(geran_Cell24, G1_IDLE); } // f_TC_9_2_3_3_5_GERAN

  - Change 3

| Function name | fl_TC_9_2_3_3_5_Body |
| --- | --- |
| Reason for change | As per the Table 9.2.3.3.5.3.2-1 [36.523-1] at Step 21a2/21b2 UE is expected to send RAU request before the timer T3323 expires. Since the timer is not stopped immediately UE may unfairly fail the testcase even if it transmits the RAU request before the timer expires. |
| Summary of change | It is proposed to stop the timer T3323 immediately once the RAU request is received on the SS side. |
| TTCN module | LTE_IRAT\9_2\NAS_TrackingArea_UG_UTRAN.ttcn |

Before Change:

function f_TC_9_2_3_3_5_UTRAN () runs on UTRAN_IRAT_PTC { /* Periodic Routing Area Update (multi RAT test) */ // SKIPPED CODE //@siclog "Step 21a1" siclog@ f_UTRAN_SetCellPower (utran_Cell5, tsc_AttenuationServingCell); // Routing Area Update on UTRAN side //@siclog "Step 21a2" siclog@ // Prose now says this should occur between timers T3312 and T3323, but these are handled in the LTE PTC // There's no need to start these again here f_UTRAN_RRC_ConnEst(utran_Cell5); U_Dc.receive(car_PS_InitDirectTransfer(utran_CellDedicated, tsc_RB3, cr_U_RA_UpdReq_WithUtranSignatures(cr_GMM_UpdateType(?, v_UpdateType), // @sic R5s120210 sic@ cr_RAI, cr_PTMSI_SignatureAny ifpresent, ?, px_PTMSI_Def, // @sic R5s110607 sic@ cr_MobileIdAny('1A'O) ifpresent, cr_RoutingAreaId2 ifpresent, v_UE_NetworkCap))) // @sic R5s110607, R5s120210 sic@ -> value v_RRC_DataInd; f_UTRAN_PreliminaryPass (__FILE__, __LINE__, "Step 21a2"); v_START_Value := v_RRC_DataInd.ttcn_start; f_UTRAN_SS_SecurityDownloadStart(ps_domain, v_START_Value); // Check ISR support if (v_RRC_DataInd.msg.routingAreaUpdateRequest.msNetworkCap.isrSupport != '1'B) { f_UTRAN_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21a2"); } f_UTRAN_RRC_Security(utran_Cell5, // SECURITY MODE COMMAND, SECURITY MODE COMPLETE v_NewKeyFlag, ps_domain); //@siclog "Step 21a3" siclog@ v_RRC_DataReq.routingAreaUpdateAccept.updateResult.ttcn_value := v_UpdateResult; U_Dc.send (cas_DataReq_PS(utran_CellDedicated, tsc_RB3, v_RRC_DataReq)); //@siclog "Step 21a4 Void" siclog@ f_UTRAN_RRC_ConnRel(utran_Cell5, cell_Dch); //@siclog "Step 21a5" siclog@ f_IRAT_SendCoOrd (EUTRA, cms_IRAT_Trigger); // Send trigger to EUTRA side to release cell // Test case now finished on UTRAN side f_UTRAN_TestBody_Set(false); f_UTRAN_Postamble(utran_Cell5, U1_IDLE); // @sic R5s110607 sic@ } // f_TC_9_2_3_3_5_UTRAN

After Change:

function f_TC_9_2_3_3_5_UTRAN () runs on UTRAN_IRAT_PTC { /* Periodic Routing Area Update (multi RAT test) */ // SKIPPED CODE //@siclog "Step 21a1" siclog@ f_UTRAN_SetCellPower (utran_Cell5, tsc_AttenuationServingCell); // Routing Area Update on UTRAN side //@siclog "Step 21a2" siclog@ // Prose now says this should occur between timers T3312 and T3323, but these are handled in the LTE PTC // There's no need to start these again here f_UTRAN_RRC_ConnEst(utran_Cell5); U_Dc.receive(car_PS_InitDirectTransfer(utran_CellDedicated, tsc_RB3, cr_U_RA_UpdReq_WithUtranSignatures(cr_GMM_UpdateType(?, v_UpdateType), // @sic R5s120210 sic@ cr_RAI, cr_PTMSI_SignatureAny ifpresent, ?, px_PTMSI_Def, // @sic R5s110607 sic@ cr_MobileIdAny('1A'O) ifpresent, cr_RoutingAreaId2 ifpresent, v_UE_NetworkCap))) // @sic R5s110607, R5s120210 sic@ -> value v_RRC_DataInd; f_UTRAN_PreliminaryPass (__FILE__, __LINE__, "Step 21a2"); f_IRAT_SendCoOrd (EUTRA, cms_IRAT_Trigger); // Send trigger to EUTRA side // Wait for EUTRA to stop timer t_T3323 f_IRAT_WaitForCoOrd_Trigger(EUTRA); // Wait trigger from EUTRAN side v_START_Value := v_RRC_DataInd.ttcn_start; f_UTRAN_SS_SecurityDownloadStart(ps_domain, v_START_Value); // Check ISR support if (v_RRC_DataInd.msg.routingAreaUpdateRequest.msNetworkCap.isrSupport != '1'B) { f_UTRAN_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21a2"); } f_UTRAN_RRC_Security(utran_Cell5, // SECURITY MODE COMMAND, SECURITY MODE COMPLETE v_NewKeyFlag, ps_domain); //@siclog "Step 21a3" siclog@ v_RRC_DataReq.routingAreaUpdateAccept.updateResult.ttcn_value := v_UpdateResult; U_Dc.send (cas_DataReq_PS(utran_CellDedicated, tsc_RB3, v_RRC_DataReq)); //@siclog "Step 21a4 Void" siclog@ f_UTRAN_RRC_ConnRel(utran_Cell5, cell_Dch); //@siclog "Step 21a5" siclog@ f_IRAT_SendCoOrd (EUTRA, cms_IRAT_Trigger); // Send trigger to EUTRA side to release cell // Test case now finished on UTRAN side f_UTRAN_TestBody_Set(false); f_UTRAN_Postamble(utran_Cell5, U1_IDLE); // @sic R5s110607 sic@ } // f_TC_9_2_3_3_5_UTRAN