3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250501

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3781 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TCs 7.1.3.4.1, 7.1.3.4.2, 7.1.3.5.3 and 7.1.3.5.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-12-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some distinct SDU numbering indications in the message column of the prose are not implemented in a distinguishable way in the TTCN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Distinct SDUs were generated and applied to make the SDUs distinguishable in the TTCN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Erroneous UE behaviour may not be detected. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.4.1.ENDC, 7.1.3.4.1.NR5GC, 7.1.3.4.2.ENDC, 7.1.3.4.2.NR5GC, 7.1.3.5.3.ENDC, 7.1.3.5.3.NR5GC, 7.1.3.5.4.ENDC and 7.1.3.5.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	16

2.3	Change 3	21

2.4	Change 4	25

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Corrections required

## Change 1

| Function name | f_TC_7_1_3_4_1_NR_Step1to7 |
| --- | --- |
| Reason for change | SDU numbering indications 0 to 7 in the message column of the prose are not implemented in a distinguishable way in the TTCN. |
| Summary of change | Distinct SDUs 0 to 7 were generated and applied to make the SDUs distinguishable in the TTCN Note: The SDUs used throughout complete TC_7_1_3_4_1 are created although not all of them used in this part. |
| TTCN module | PDCP_TC_Common_NR.ttcn |

After Change

function f_TC_7_1_3_4_1_NR_Step1to7(inout NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_PDCP_SDUList_Type v_SduList := f_GenerateSDU (p_NR_PDCP_State, {55,68,81,94,107, 120, 133, 146}); // @sic R5s250501 sic@ var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var NR_AbsoluteCellPower_Type v_CellPowerAtT1Cell2_FR1 := -82; var NR_AbsoluteCellPower_Type v_CellPowerAtT1Cell2_FR2 := -82; var SubFrameTiming_Type v_TimingInfoStart; var integer k; //Step 2 and 43 shall be repeated for k=0 to 1 //@siclog "Step 2-3" siclog@ for (k:= 0; k <= 1; k:= k+1) { //SS transmits a PDCP Data PDU containing one PDCP SDU f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell1, v_IPData v_SduList[k]); //@sic R5s200063 sic@ //Does UE transmit a PDCP Data PDU with SN=K for the first iteration? f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_IPData v_SduList[k], cr_RlcBearerRouting_NR(nr_Cell1)); } //@siclog "Step 3A" siclog@ //@sic R5-201219 sic@ //The SS changes NR Cell 2 parameters according to the row "T1" in table 7.1.3.4.1.3.2.0-1. f_NR_SetCellPower (nr_Cell2, v_CellPowerAtT1Cell2_FR1, v_CellPowerAtT1Cell2_FR2); //@sic R5-195929 sic@ v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, p_NR_PDCP_State.DRB_Id, -, cs_NR_PDCP_RbConfig_TransparentMode(f_NR_38508_SetPDCP_SN_Size()), // @sic R5-227535 sic@, cs_NR_RLC_TestModeConfig_NotAcK, -, -, ms150)};// @sic R5s200690 sic@ v_TimingInfoStart := f_NR_GetNextSendOccasion_DL(nr_Cell1);//@sic R5-201219 sic@ for (k:= 0; k <= 2; k:= k+1) { //@sic R5s190844 sic@ //@siclog "Step 4" siclog@ //Configure the SS to not ACK next RLC PDU @sic R5-190752 ,R5-191296 sic@ f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList); //@siclog "Step 5" siclog@ //The SS sends the PDCP Data PDU f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell1, v_IPData v_SduList[k+2], f_SubFrameTiming_AddMilliSeconds(v_TimingInfoStart, 100*k)); //@sic R5-201219 R5-222511 sic@ //@siclog "Step 6" siclog@ //The UE sends the PDCP Data PDU f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_IPData v_SduList[k+2], cr_RlcBearerRouting_NR(nr_Cell1)); } //@siclog "Step 6A" siclog@ f_NR_ULGrantConfiguration_Stop(nr_Cell1); //@siclog "Step 7" siclog@ Void }

| Function name | f_TC_7_1_3_4_1_NR_Step10to23 |
| --- | --- |
| Reason for change | SDU numbering indications 2 to 7 in the message column of the prose are not implemented in a distinguishable way in the TTCN. |
| Summary of change | Distinct SDUs 2 to 7 were generated and applied to make the SDUs distinguishable in the TTCN Note: The SDUs used throughout complete TC_7_1_3_4_1 are created although not all of them used in this part. |
| TTCN module | PDCP_TC_Common_NR.ttcn |

After Change

function f_TC_7_1_3_4_1_NR_Step10to23(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_PDCP_SDUList_Type v_SduList := f_GenerateSDU (p_NR_PDCP_State, {55,68,81,94,107, 120, 133, 146}); // @sic R5s250501 sic@ var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingCurrentSlot; var SubFrameTiming_Type v_TimingPreviousSlot; // var TimingInfo_Type v_TimingInfo; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu1; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu2; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu3; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu4; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu5; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu6; var NR_DRB_COMMON_IND v_ReceivedPdu1; var NR_DRB_COMMON_IND v_ReceivedPdu2; var NR_DRB_COMMON_IND v_ReceivedPdu3; var NR_DRB_COMMON_IND v_ReceivedPdu4; var NR_DRB_COMMON_IND v_ReceivedPdu5; var NR_DRB_COMMON_IND v_ReceivedPdu6; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell1_FR1 := -82; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell1_FR2 := -82; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell2_FR1 := -88; var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell2_FR2 := -91; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2 := { cs_NR_CellPower(nr_Cell1, v_CellPowerAtT2Cell1_FR1, v_CellPowerAtT2Cell1_FR2), cs_NR_CellPower(nr_Cell2, v_CellPowerAtT2Cell2_FR1, v_CellPowerAtT2Cell2_FR2) }; //@sic R5-195929 sic@ timer t_Wait:= 5.0; // for ENDC RRCReconfigurationComplete is not received, just one UL Grant is assigned (112 bits)for STATUS PDU // For NR5GC, RRCReconfigurationComplete is received, 2 Grants (2x112 bits)are assigned, one for RRCReconfigurationComplete and one for STATUS PDU //@siclog "Step 9" siclog@ // The SS assigns UL grant during the Random Access procedure on NR Cell 2 to allow the UE to send only PDCP status report and // 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header + 3oct BSR or padding = 13 octets v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); //@sic R5-198906 sic@ //@sic R5s201484 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell2); // Timing of DL v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell2, v_TimingDL, 20); if ( p_WaitforRRCConCmp) { // if NR5GC start a periodic grant to receive RRCReconfigurationComplete and PDCP STATUS REPORT @sic R5s210282 sic@ f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell2, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20); } else { // if ENDC - only one grant for PDCP STATUS REPORT f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell2, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, 1); } if ( p_WaitforRRCConCmp) {//@sic R5s210032 sic@ interleave {//@sic R5s201484 sic@ //Receive RRCReconfigurationComplete []SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell2, cr_38508_RRCReconfigurationComplete)){} //@siclog "Step 11" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? []DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(5)})){} } } else{ //@siclog "Step 11" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(5)})); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); if ( p_WaitforRRCConCmp) { // if NR5GC @sic R5s210282 sic@ f_NR_ULGrantConfiguration_Stop(nr_Cell2); } if(p_WaitforRRCConCmp){ //@sic R5-204363 sic@ f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, f_NR_GetActiveDRBs(nr_Cell1)); //@sic R5s200690 sic@ f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit); }; //@siclog "Step 12" siclog@ //The SS generates a PDCP status report message and sends it to UE: D/C field = 0 (PDCP control PDU) //and PDU Type =000, FMC field = 3 DRB.send (cas_NR_DRB_COMMON_REQ_PDCP_PDUList (nr_Cell2, p_NR_PDCP_State.DRB_Id, -, {cs_NR_PDCP_STATUS_REPORT(2)})); //@sic R5-196261 sic@ //@siclog "Step 13" siclog@ // The SS starts the UL default grant transmission. f_NR_ULGrantConfiguration_Start(nr_Cell2); //@siclog "Step 15" siclog@ //@sic R5-213462 sic@ p_NR_PDCP_State.RX_NEXT:= 2;//@sic R5-196261 sic@ v_ExpectedPdu1:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT, v_SduList[2]); // @sic R5s210719 sic@ f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu2:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT, v_SduList[3]); // @sic R5s210719 sic@ f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu3:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT, v_SduList[4]); // @sic R5s210719 sic@ f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu4:= {v_ExpectedPdu1, v_ExpectedPdu2}; v_ExpectedPdu5:= {v_ExpectedPdu2, v_ExpectedPdu3}; v_ExpectedPdu6:= {v_ExpectedPdu1, v_ExpectedPdu2, v_ExpectedPdu3}; alt { // receive PDU1 // @sic R5s210838 sic@ [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, {v_ExpectedPdu1})) -> value v_ReceivedPdu1 // @sic R5s210719 sic@ { v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame; alt { // receive PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, {v_ExpectedPdu2})) -> value v_ReceivedPdu2 // @sic R5s210719 sic@ { v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); v_TimingPreviousSlot:= v_TimingCurrentSlot; } // receive PDU3 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, {v_ExpectedPdu3})) -> value v_ReceivedPdu3 // @sic R5s210719 sic@ { v_TimingCurrentSlot := v_ReceivedPdu3.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } // receive PDU2 and PDU3 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, v_ExpectedPdu5)) -> value v_ReceivedPdu5 { v_TimingCurrentSlot := v_ReceivedPdu5.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } } } // receive PDU1 and PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, v_ExpectedPdu4)) -> value v_ReceivedPdu4 { v_TimingPreviousSlot := v_ReceivedPdu4.Common.TimingInfo.SubFrame; // receive PDU3 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, {v_ExpectedPdu3})) -> value v_ReceivedPdu3; v_TimingCurrentSlot := v_ReceivedPdu3.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } // receive PDU1, PDU2 and PDU3 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, v_ExpectedPdu6)) -> value v_ReceivedPdu6 { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } } //@siclog "Step 16" siclog@ //The SS sends the PDCP Data PDU#5 p_NR_PDCP_State.TX_NEXT:= 5; f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell2, v_IPData v_SduList[5],); //@siclog "Step 17" siclog@ //The UE sends the PDCP Data PDU#5 f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell2, v_IPData v_SduList[5], cr_RlcBearerRouting_NR(nr_Cell2)); //@siclog "Step 20" siclog@ //SS transmits a PDCP Data PDU containing one PDCP SDU#7 p_NR_PDCP_State.TX_NEXT:= 7; p_NR_PDCP_State.RX_NEXT:= 7; //The SS sends the PDCP Data PDU f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell2, v_IPData v_SduList[7]); //@siclog "Step 21" siclog@ //Check: Does the UE transmit a PDCP DATA PDU#7 on NR Cell 2? t_Wait.start; alt { [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList(nr_Cell2, p_NR_PDCP_State.DRB_Id)) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 21"); } [] t_Wait.timeout{} } //@siclog "Step 21A" siclog@ f_NR_ULGrantConfiguration_Stop(nr_Cell1); //@sic R5-190752 sic@ //@siclog "Step 22" siclog@ //The SS changes NR Cell 1 and NR Cell 2 parameters according to row "T2" in Table 7.1.3.4.1.3.1-1. f_NR_SetCellPowerList(v_CellPowerList_AtT2); }

| Function name | f_TC_7_1_3_4_1_NR_Step25to30 |
| --- | --- |
| Reason for change | SDU numbering indications 5 to 7 in the message column of the prose are not implemented in a distinguishable way in the TTCN. |
| Summary of change | Distinct SDUs 5 to 7 were generated and applied to make the SDUs distinguishable in the TTCN Note: The SDUs used throughout complete TC_7_1_3_4_1 are created although not all of them used in this part. |
| TTCN module | PDCP_TC_Common_NR.ttcn |

After Change

function f_TC_7_1_3_4_1_NR_Step25to30(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_PDCP_SDUList_Type v_SduList := f_GenerateSDU (p_NR_PDCP_State, {55,68,81,94,107, 120, 133, 146}); // @sic R5s250501 sic@ var NR_ResourceAllocation_Type v_NR_ResourceAllocation; var SubFrameTiming_Type v_TimingDL; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingCurrentSlot; var SubFrameTiming_Type v_TimingPreviousSlot; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu1; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu2; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu3; var NR_DRB_COMMON_IND v_ReceivedPdu1; var NR_DRB_COMMON_IND v_ReceivedPdu2; var NR_DRB_COMMON_IND v_ReceivedPdu3; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id); timer t_Wait:= 5.0; // for ENDC RRCReconfigurationComplete is not received, jut one UL Grant is assigned (112 bits)for STATUS PDU // For NR5GC, RRCReconfigurationComplete is received, 2 Grants (2x112 bits)are assigned, one for RRCReconfigurationComplete and one for STATUS PDU //@siclog "Step 24" siclog@ // The SS assigns UL grant during the Random Access procedure on NR Cell 1 to allow the UE to send only PDCP status report. // 5 octets PDCP STATUS REPORT + 3 octets RLC header + 2 octets MAC header = 10 octets v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(112, v_NR_UplinkBWP); //@sic R5-198906 sic@ v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1); // Timing of DL v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingDL, 20); if ( p_WaitforRRCConCmp) { // if NR5GC start a periodic grant to receive RRCReconfigurationComplete and PDCP STATUS REPORT @sic R5s210282 sic@ f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20); } else { // if ENDC f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(nr_Cell1, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, 1); } if ( p_WaitforRRCConCmp) {//@sic R5s210032 sic@ //EXCEPTION: Steps 25 and 26 can occur in any order. (Note 7) interleave {//@sic R5s201484 sic@ //@siclog "Step 25" siclog@ //The UE transmits a NR RRCReconfigurationComplete message.(Note 4) []SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)){} //@siclog "Step 26" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? []DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(6,'80'O)})){} } } else{ //@siclog "Step 26" siclog@ //Does the UE send PDCP Control PDUs via RLC-AM RB with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 5? DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), //@sic R5s190844 sic@ -, {cr_NR_PDCP_STATUS_REPORT(6,'80'O)})); } if(p_WaitforRRCConCmp){ //@sic R5-204363 sic@ f_NR_ULGrantConfiguration_Stop(nr_Cell1); // @sic R5s210282 sic@ f_NR_SS_SRBs_DRBs_Release(nr_Cell2, cs_TimingInfo_Now, f_NR_GetActiveDRBs(nr_Cell2)); //@sic R5s200690 sic@ f_NR_SS_SRBs_DRBs_Config(nr_Cell2, cs_TimingInfo_Now, omit); }; //@siclog "Step 27" siclog@ //The SS generates a PDCP status report message and sends it to UE: D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 6 on NR Cell 1. DRB.send (cas_NR_DRB_COMMON_REQ_PDCP_PDUList (nr_Cell1, p_NR_PDCP_State.DRB_Id, -, {cs_NR_PDCP_STATUS_REPORT(6)})); //@siclog "Step 28" siclog@ // The SS starts the UL default grant transmission. f_NR_ULGrantConfiguration_Start(nr_Cell1); //@siclog "Step 28A" siclog@ //The SS sends the PDCP Data PDU#5 p_NR_PDCP_State.TX_NEXT:= 5; f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell1, v_IPData v_SduList[5]); //@sic R5-190752 sic@ //@siclog "Step 28B" siclog@ //Check: Does the UE transmit a PDCP Data PDU via RLC-AM RB with the following content back to the SS? t_Wait.start; //@sic R5-190752 sic@ alt { [] DRB.receive((car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1)))) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 28B"); } [] t_Wait.timeout{} } //@siclog "Step 29" siclog@ //SS transmits a PDCP Data PDU containing one PDCP SDU#6 p_NR_PDCP_State.TX_NEXT:= 6; p_NR_PDCP_State.RX_NEXT:= 6; f_NR_PDCP_Send(p_NR_PDCP_State, nr_Cell1, v_IPData v_SduList[6]); //@siclog "Steps 30 - 31" siclog@ //Check: Does the UE transmit a PDCP Data PDU via RLC-AM RB with the following content back to the SS? D/C field = 1 (PDCP Data PDU) and PDCP SN=6 on NR Cell 1. //Note: Data is previously received packet in PDCP Data PDU#6.(Note 9) //Check: Does the UE transmit PDCP Data PDU via RLC-AM RB with the following content back to the SS? D/C field = 1 (PDCP Data PDU) and PDCP SN=7 on NR Cell 1. //Note: Data is previously received packet in PDCP Data PDU#7(Note 9). //@sic R5-213462 sic@ v_ExpectedPdu1:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT, v_SduList[6]);// @sic R5s210719 sic@ f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu2:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT, v_SduList[7]);// @sic R5s210719 sic@ f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu3:= {v_ExpectedPdu1,v_ExpectedPdu2}; // @sic R5s210719 sic@ alt { // receive PDU1 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -,{v_ExpectedPdu1})) -> value v_ReceivedPdu1 // @sic R5s210719 sic@ { v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame; // receive PDU2 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu2})) -> value v_ReceivedPdu2; // @sic R5s210719 sic@ v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 30-31"); } // receive PDU1 and PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu3))-> value v_ReceivedPdu3 { f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 30-31"); } } }

## Change 2

| Function name | fl_TC_7_1_3_4_2_NR_TestBody |
| --- | --- |
| Reason for change | SDU numbering indications 0 to 2 in the message column of the prose are not implemented in a distinguishable way in the TTCN. |
| Summary of change | Distinct SDUs 0 to 2 were generated and applied to make the SDUs distinguishable in the TTCN |
| TTCN module | PDCP_ENDC_NR.ttcn |

After Change

function fl_TC_7_1_3_4_2_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on ENDC_NR_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_PDCP_SDUList_Type v_SduList := f_GenerateSDU (p_NR_PDCP_State, {65,69,73}); // @sic R5s250501 sic@ var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var integer k; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList := f_NR_ReconfigureRLC_UM(tsc_NR_DRB2, -, -, ms80, true_); var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := f_NR_GetRACHProcedureConfigPSCell( nr_Cell1, -); //@sic R5s190370 sic@ var template (value) CellGroupConfig v_CellGroupConfig := f_NR_GetCellGroupConfigHO (nr_Cell1, v_RLC_BearerConfigList); //@sic R5s1903370 sic@ var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, tsc_NR_DRB2, -, cs_NR_PDCP_RbConfig_TransparentMode(PDCP_SNLength18))}; //@sic R5s1903370 sic@ var EUTRA_NR_Coordination_MSG v_Coordination_MSG_Value; //@sic R5s200126 sic@ p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc; //@siclog "Step 1" All PDCP PDUs use same SDU v_IPData siclog@ // The SS creates 3 PDCP Data PDUs and the TX_NEXT is set to "0". //@siclog "Step 2-3" siclog@ for (k:= 0; k <= 1; k:= k+1) { //SS transmits a PDCP Data PDU containing one PDCP SDU f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData v_SduList[k]); //Does UE transmit a PDCP Data PDU with SN=K for the first iteration? f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_IPData v_SduList[k], cr_RlcBearerRouting_NR(nr_Cell1)); } //@sic R5s200126 sic@ //@siclog "Step 4" siclog@ //SS transmits a RRCConnectionReconfiguration // @sic R5s190370 sic@ v_RadioBearerConfig := cs_NR_RbReestPDCP(cs_38508_PDCP_Config(-,ms80,omit), //@sic R5-195696, R5-213445, R5-227518 sic@ cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), omit), master));//@sic R5-195364 sic@ f_NR_RRC_PSCellChange_IntraCell_Step2To5 (nr_Cell1, -, v_RadioBearerConfig, -, v_CellGroupConfig, {cs_NR_SS_DRB_Release(tsc_NR_DRB2)}, v_SS_Drb_ConfigList, v_RachProcedureConfig ); //@sic R5s200126 sic@ //RRCReconfigurationComplete not yet received from E-UTRA - Wait for it EUTRA.receive(cmr_EUTRA_NR_OctetData ("Complete")) -> value v_Coordination_MSG_Value; f_NR_CheckReceived_RRCReconfigurationComplete(v_Coordination_MSG_Value); //@siclog "Step 6" siclog@ //SS transmits a PDCP Data PDU containing one PDCP SDU // The SS sends the PDCP Data PDU #2 p_NR_PDCP_State.TX_NEXT:= 0; p_NR_PDCP_State.RX_NEXT:= 0; f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1, v_IPData v_SduList[2]); //@siclog "Step 7" siclog@ //Does UE transmit a PDCP Data PDU with SN=K for the first iteration? // Does the UE send the PDCP Data PDU #2? f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_IPData v_SduList[2]), cr_RlcBearerRouting_NR(nr_Cell1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); //Coordination message to trigger the EUTRA Postamble f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); }

| Function name | fl_TC_7_1_3_4_2_NR5GC_TestBody |
| --- | --- |
| Reason for change | SDU numbering indications 0 to 2 in the message column of the prose are not implemented in a distinguishable way in the TTCN. |
| Summary of change | Distinct SDUs 0 to 2 were generated and applied to make the SDUs distinguishable in the TTCN |
| TTCN module | PDCP_NR5GC.ttcn |

After Change

function fl_TC_7_1_3_4_2_NR5GC_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR5GC_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_PDCP_SDUList_Type v_SduList := f_GenerateSDU (p_NR_PDCP_State, {65,69,73}); // @sic R5s250501 sic@ var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var integer k; var template (value) DL_DCCH_Message v_RRCReconfiguration; var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig ; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1); var template (omit) MasterKeyUpdate v_MasterKeyUpdate:= cs_MasterKeyUpdate(false, tsc_NR_38508_NextHopChainingCount); //@sic R5-202618 sic@ var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := f_NR5GC_GetRadioBearerList(f_NR5GC_MobileInfo_GetDRBInfoList());//@sic R5s220503 sic@ v_SS_Drb_ConfigList[0] := f_NR_GetRadioBearerConfig_FullUM(nr_Cell1, p_NR_PDCP_State.DRB_Id, -, cs_NR_PDCP_RbConfig_TransparentMode(f_NR_38508_SetPDCP_SN_Size()), // @sic R5s220124, R5-227518 sic@ cs_SDAP_Configuration_None); v_RachProcedureConfig := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, tsc_C_RNTI_Value3); v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO(nr_Cell1, -, true, -,-,-, v_MasterKeyUpdate, -,-,-,-, cs_38508_PDCP_Config(-, ms80, omit)); ////@sic R5-213445, R5-227518, R5s230856 sic@ p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc; //@siclog "Step 1" All PDCP PDUs use same SDU v_IPData siclog@ // The SS creates 3 PDCP Data PDUs and the TX_NEXT is set to "0". //@siclog "Step 2-3" siclog@ for (k:= 0; k <= 1; k:= k+1) { //SS transmits a PDCP Data PDU containing one PDCP SDU f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1, v_IPData v_SduList[k]); //Does UE transmit a PDCP Data PDU with SN=K for the first iteration? f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_IPData v_SduList[k], cr_RlcBearerRouting_NR(nr_Cell1)); } //@siclog "Step 4" siclog@ //SS transmits a RRCConnectionReconfiguration f_NR5GC_508RRC_IntraNR_HO_IntraCell_Step1_8(nr_Cell1, v_RRCReconfiguration, true,//with Key Change v_SS_Drb_ConfigList, v_RachProcedureConfig, -,-,-,-, //@sic R5s220825 sic@ false); //No PDCP Counts //@siclog "Step 5 siclog@Receive RRCReconfigurationComplete. SRB.receive(car_NR_SRB1_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete)); //@sic R5s191055 sic@ // enable TA transmissions f_NR_ULGrantConfiguration_Common(nr_Cell1, -, -, cs_NR_UplinkTimeAlignment_Start, cs_UL_GrantConfig_OnSR);//@sic R5s220124 sic@ //@siclog "Step 6" siclog@ //SS transmits a PDCP Data PDU containing one PDCP SDU // The SS sends the PDCP Data PDU #2 p_NR_PDCP_State.TX_NEXT:= 0; p_NR_PDCP_State.RX_NEXT:= 0; f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1, v_IPData v_SduList[2]); //@siclog "Step 7" siclog@ //Does UE transmit a PDCP Data PDU with SN=K for the first iteration? // Does the UE send the PDCP Data PDU #2? f_NR_PDCP_Recv(p_NR_PDCP_State, nr_Cell1, v_IPData v_SduList[k], cr_RlcBearerRouting_NR(nr_Cell1)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); }

## Change 3

| Function name | f_TC_7_1_3_5_3_NR_TestBody_Step1to4A |
| --- | --- |
| Reason for change | SDU numbering indications 0 to 2 in the message column of the prose are not implemented in a distinguishable way in the TTCN. |
| Summary of change | Distinct SDUs 0 to 2 were generated and applied to make the SDUs distinguishable in the TTCN |
| TTCN module | PDCP_TC_Common_NR.ttcn |

After Change

function f_TC_7_1_3_5_3_NR_TestBody_Step1to4A(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, NR_CellId_Type p_PSCellId) runs on NR_BASE_PTC { var DRB_Identity v_DrbId := p_NR_PDCP_State.DRB_Id; var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_PDCP_SDUList_Type v_SduList := f_GenerateSDU (p_NR_PDCP_State, {75,79,83}); // @sic R5s250501 sic@ var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var integer k; var NR_CellId_Type v_PSCellId := p_PSCellId; var template (value) NR_RLC_TestModeConfig_Type v_NR_RLC_TestModeConfig := {Info := {NotACK_NextRLC_PDU := Start}}; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList := { f_NR_GetRadioBearerConfig_FullAM(v_PSCellId, v_DrbId, -, cs_NR_PDCP_RbConfig_TransparentMode, v_NR_RLC_TestModeConfig) }; p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc; //@siclog "Step 1" siclog@ // Void //@siclog "Step 2" siclog@ // The SS creates 3 PDCP Data PDUs and the Next_PDCP_TX_SN is set to "0". // PDUs are created in the subsequent steps //EXCEPTION: Steps 2A and 4 shall be repeated for k=0 to 2 (increment=1). //@siclog "Step 2A-4" siclog@ for (k:= 0; k <= 2; k:= k+1) { //The SS is configured on PSCell to not send RLC acknowledgement (RLC ACK) to the UE. f_NR_SS_CommonRadioBearerConfig(v_PSCellId, v_SS_Drb_ConfigList); //The SS sends the PDCP Data PDU #k on SCG DRB on (PSCell): D/C field = 1 (PDCP Data PDU) and PDCP SN = k. f_NR_PDCP_Send(p_NR_PDCP_State,v_PSCellId, v_IPData v_SduList[k]); //The UE sends the PDCP Data PDU #k on the AM RLC entity configured for PSCell: D/C field = 1 (PDCP Data PDU) and PDCP SN = k. f_NR_PDCP_Recv(p_NR_PDCP_State,v_PSCellId, v_IPData v_SduList[k],cr_RlcBearerRouting_NR(v_PSCellId)); } //@siclog "Step 4A" siclog@ // The SS ignores scheduling requests unless otherwise specified and does not allocate any uplink grant. f_NR_ULGrantConfiguration_Stop(v_PSCellId, -, cs_NR_UplinkTimeAlignment_Keep); }

| Function name | f_TC_7_1_3_5_3_NR_TestBody_Step7to9 |
| --- | --- |
| Reason for change | SDU numbering indications 0 to 2 in the message column of the prose are not implemented in a distinguishable way in the TTCN. |
| Summary of change | Distinct SDUs 0 to 2 were generated and applied to make the SDUs distinguishable in the TTCN |
| TTCN module | PDCP_TC_Common_NR.ttcn |

After Change

function f_TC_7_1_3_5_3_NR_TestBody_Step7to9(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, NR_CellId_Type p_PSCellId, NR_CellId_Type p_PCellId, SubFrameTiming_Type p_Timing) runs on NR_BASE_PTC { var DRB_Identity v_DrbId := p_NR_PDCP_State.DRB_Id; var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_PDCP_SDUList_Type v_SduList := f_GenerateSDU (p_NR_PDCP_State, {75,79,83}); // @sic R5s250501 sic@ var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var integer k; var NR_CellId_Type v_PSCellId := p_PSCellId; var NR_CellId_Type v_NRCellId := p_PCellId; // CellId to be used in step 9 var template (value) NR_ResourceAllocation_Type v_NR_ResourceAllocation; var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(v_PSCellId, tsc_NR_BWP_Id); var SubFrameTiming_Type v_TimingUL; p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc; //@siclog "Step 6" siclog@ // The SS assigns 1 UL grant of sufficent size. // For EN-DC, 104-bits of grant size is provided to include 10 bytes of PDCP Status report MAC PDU+ 3 bytes of Short BSR and/or Padding. // For NR-DC, 272-bits of grant size to include 5 bytes of RLC STATUS MAC PDU, 10 bytes of PDCP Status report MAC PDU, 14 bytes of RRCReconfigurationComplete MAC PDU, 2 bytes of Short BSR and 3 bytes of Padding. if (f_GetTestcaseAttrib_ENDC(testcasename())) //@sic R5s230377 sic@ { v_NRCellId := p_PSCellId; v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(104, v_NR_UplinkBWP); } else { v_NRCellId := p_PCellId; v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(272, v_NR_UplinkBWP); // @sic R5s230377 sic@ } f_NR_OneULGrantTransmission(v_NRCellId, cs_TimingInfo_NR(p_Timing), v_NR_ResourceAllocation); //@sic R5s220028 - UL grant with the timing information, R5s220194, R5s230377 sic@ //EXCEPTION: Steps 7 and 8 can occur in any order. //@siclog "Step 8" siclog@ //The UE sends PDCP Control PDUs via RLC-AM RB entity configured for PSCell Connectivity (EN-DC) and PCell for Connectivity (NR-DC)with the following content to the SS: //D/C field = 0 (PDCP control PDU) and PDU Type =000, FMC field = 3. if (f_GetTestcaseAttrib_ENDC(testcasename())) // @sic R5s230377 sic@ { DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(v_NRCellId, v_DrbId, cr_RlcBearerRouting_NR(v_NRCellId), -, {cr_NR_PDCP_STATUS_REPORT(3)})); } else { interleave { //@siclog "Step 7" siclog@ //The UE transmits a NR RRCReconfigurationComplete. []SRB.receive (car_NR_SRB_RrcPdu_IND(v_NRCellId, tsc_NR_RbId_SRB1, cr_38508_RRCReconfigurationComplete)){} []DRB.receive (car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(v_NRCellId, v_DrbId, cr_RlcBearerRouting_NR(v_NRCellId), -, {cr_NR_PDCP_STATUS_REPORT(3)})){} } } //@siclog "Step 8A" siclog@ // After 100 ms the SS allocates 3 UL grants every 20ms of sufficient size to enable the UE to return each received PDCP PDU in one looped back PDCP PDU on PSCell for Connectivity (EN-DC) and PCell for Connectivity (NR-DC). // 44 resp. 64 octets IPData + 3 octets PDCP header + 3 octets RLC header + 2 octets MAC header + 2 octets BSR or padding = 54 resp. 74 octets if (f_IpAddressIsIPv4(f_LoopbackModeB_IP_Address_UE())) { // @sic R5s200153 sic@ v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(432, v_NR_UplinkBWP); // @sic R5s191042, R5-198180 sic@ } else { v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(608, v_NR_UplinkBWP); } v_TimingUL := f_NR_GetNextSendOccasion_UL(v_NRCellId); // @sic R5s230377 sic@ f_NR_ULGrantConfiguration_StartPeriodicCyclicGrant(v_NRCellId, cs_TimingInfo_NR(v_TimingUL), v_NR_ResourceAllocation, 20, 3); //@sic R5-197255, R5s200029, R5s230377 sic@ //EXCEPTION: Step 9 shall be repeated for k=0 to 2 (increment=1). //@siclog "Step 9" siclog@ // Check: Does the UE send the PDCP Data PDU #k via the AM RLC entity configured for PSCell for Connectivity ENDC and PCell for Connectivity (NR-DC). // D/C field = 1 (PDCP Data PDU) and PDCP SN = k. Data is previously received data from PDU #k ? p_NR_PDCP_State.RX_NEXT:= 0; for (k:= 0; k <= 2; k:= k+1) { f_NR_PDCP_Recv(p_NR_PDCP_State,v_NRCellId, v_IPData v_SduList[k],cr_RlcBearerRouting_NR(v_NRCellId)); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9"); }

## Change 4

| Function name | f_TC_7_1_3_5_4_NR_Step1to17 |
| --- | --- |
| Reason for change | SDU numbering indications 0 to 4 in the message column of the prose are not implemented in a distinguishable way in the TTCN. |
| Summary of change | Distinct SDUs 0 to 4 were generated and applied to make the SDUs distinguishable in the TTCN Notes: SDU #X left unchanged as it is not expected to be looped back. SDU #7 and #9 left unchanged as they are already distinguishable. |
| TTCN module | PDCP_TC_Common_NR.ttcn |

After Change

function f_TC_7_1_3_5_4_NR_Step1to17(inout NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_PDCP_SDUList_Type v_SduList := f_GenerateSDU (p_NR_PDCP_State, {64, 164}); var NR_PDCP_SDUList_Type v_SduList1 := f_GenerateSDU (p_NR_PDCP_State, {50, 60, 70, 80,90}); // @sic R5s250501 sic@ var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var SubFrameTiming_Type v_TimingInfo1; var SubFrameTiming_Type v_TimingInfo2; var SubFrameTiming_Type v_TimingInfo3; var SubFrameTiming_Type v_SubFrameTimingT1; var SubFrameTiming_Type v_SubFrameTimingT2; var SubFrameTiming_Type v_TimingCurrentSlot; var SubFrameTiming_Type v_TimingPreviousSlot; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu1; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu2; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu3; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu4; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu5; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu6; var NR_DRB_COMMON_IND v_ReceivedPdu1; var NR_DRB_COMMON_IND v_ReceivedPdu2; var NR_DRB_COMMON_IND v_ReceivedPdu3; var NR_DRB_COMMON_IND v_ReceivedPdu4; var NR_DRB_COMMON_IND v_ReceivedPdu5; var NR_DRB_COMMON_IND v_ReceivedPdu6; var float v_T_reordering; var TimingInfo_Type v_TimingInfo; var integer v_Duration; timer t_Wait := 1.0; p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc ; //@siclog "Step 1" siclog@ //SS transmits a PDCP Data PDU containing one PDCP SDU #131072 resp. #2048 if (pc_supportOfRedCap_r17 == true){ //@sic R5s230199 R5-231533 sic@ p_NR_PDCP_State.TX_NEXT := 2048; } else { p_NR_PDCP_State.TX_NEXT := 131072; //@sic R5s190227 sic@ } f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData); //@siclog "Step 2" siclog@ //Does the UE transmit a PDCP SDU via the AM RLC entity in the next 1s? t_Wait.start; alt { [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList(nr_Cell1, p_NR_PDCP_State.DRB_Id)) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } [] t_Wait.timeout{} } //@siclog "Step 3" siclog@ //The SS sends the PDCP SDU #1 p_NR_PDCP_State.TX_NEXT := 1; v_TimingInfo1 := f_NR_GetNextSendOccasion_DL(nr_Cell1); v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1,20); v_TimingInfo3 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo2,120); f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData v_SduList1[1],v_TimingInfo1); //@siclog "Step 4" siclog@ //The SS sends the PDCP SDU #2 f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData v_SduList1[2],v_TimingInfo2); //@siclog "Step 5" siclog@ Wait for 100ms (< configured t-Reordering) //v_TimingInfo3 - v_TimingInfo2 //@siclog "Step 6" siclog@ The SS sends the PDCP SDU #0 p_NR_PDCP_State.TX_NEXT := 0; f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData v_SduList1[0],v_TimingInfo3); //@sic R5-213462 sic@ //@siclog "Step 7-9" siclog@ //UE transmits the PDCP SDU #0 via the AM RLC entity #0,#1,#2 (Note 4). @sic R5s190227 sic@ v_ExpectedPdu1:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT, v_SduList1[0]); //@sic R5s210673sic@ f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu2:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT, v_SduList1[1]); //@sic R5s210673sic@ f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu3:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT, v_SduList1[2]); //@sic R5s210673sic@ f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); //@sic R5s210673 sic@ v_ExpectedPdu4:= {v_ExpectedPdu1, v_ExpectedPdu2}; v_ExpectedPdu5:= {v_ExpectedPdu2, v_ExpectedPdu3}; v_ExpectedPdu6:= {v_ExpectedPdu1, v_ExpectedPdu2, v_ExpectedPdu3}; alt { // receive PDU1 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu1})) -> value v_ReceivedPdu1 //@sic R5s210673 sic@ { v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame; alt { // receive PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu2})) -> value v_ReceivedPdu2 // @sic R5s210673 sic@ { v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); v_TimingPreviousSlot:= v_TimingCurrentSlot; // receive PDU3 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu3})) -> value v_ReceivedPdu3; // @sic R5s210673 sic@ v_TimingCurrentSlot := v_ReceivedPdu3.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7-9"); } // } // receive PDU2 and PDU3 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu5)) -> value v_ReceivedPdu5 // @sic R5s210673 sic@ { v_TimingCurrentSlot := v_ReceivedPdu5.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7-9"); } } } // receive PDU1 and PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu4)) -> value v_ReceivedPdu4 { v_TimingPreviousSlot := v_ReceivedPdu4.Common.TimingInfo.SubFrame; // receive PDU3 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu3})) -> value v_ReceivedPdu3; v_TimingCurrentSlot := v_ReceivedPdu3.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7-9"); } // receive PDU1, PDU2 and PDU3 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu6)) -> value v_ReceivedPdu6 { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7-9"); } } //@siclog "Step 10" siclog@ //The SS sends the PDCP SDU #4 v_TimingInfo1 := f_NR_GetNextSendOccasion_DL(nr_Cell1); //t-Reordering starts v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1,100); //Next PDU v_TimingInfo3 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1,120); //Wait for PDU v_SubFrameTimingT1 := v_TimingInfo3; p_NR_PDCP_State.TX_NEXT := 4; f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData v_SduList1[4],v_TimingInfo1); //@siclog "Step 11" siclog@ Wait for 100ms (< configured t-Reordering) //v_TimingInfo2 - v_TimingInfo1 //@siclog "Step 12" siclog@ The SS sends the PDCP SDU #7 p_NR_PDCP_State.TX_NEXT := 7; f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_SduList[0],v_TimingInfo2); //@siclog "Step 13" siclog@ The SS sends the PDCP SDU #3 p_NR_PDCP_State.TX_NEXT := 3; f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData v_SduList1[3],v_TimingInfo3); //@siclog "Step 14-15" siclog@ UE transmits a PDCP SDU #3, #4 (Note 5). @sic R5s190227 sic@ //@sic R5-213462 sic@ v_ExpectedPdu1:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT, v_SduList1[3]); // @sic R5s210673 sic@ f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu2:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT, v_SduList1[4]); // @sic R5s210673 sic@ f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu4:= {v_ExpectedPdu1,v_ExpectedPdu2}; // @sic R5s210673 sic@ alt { // receive PDU1 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu1})) -> value v_ReceivedPdu1 // @sic R5s210673 sic@ { v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame; // receive PDU2 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu2})) -> value v_ReceivedPdu2; // @sic R5s210673 sic@ v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 14-15"); } // receive PDU1 and PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu4))-> value v_ReceivedPdu3 // @sic R5s210673 sic@ { f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 14-15"); } } //@siclog "Step 16" siclog@ UE transmits a PDCP SDU #7 p_NR_PDCP_State.RX_NEXT := 5; //@sic R5-194837 sic@ v_TimingInfo := f_NR_PDCP_Recv(p_NR_PDCP_State,nr_Cell1,v_SduList[0], cr_RlcBearerRouting_NR(nr_Cell1)); v_SubFrameTimingT2 := v_TimingInfo.SubFrame; // Check 1: Is (T2 - T1) > t-Reordering? v_Duration := f_SubFrameTiming_Duration(v_SubFrameTimingT1, v_SubFrameTimingT2); v_T_reordering := f_NR_SetTimerToleranceMin(nr_Cell1,l2Timer,300.0); if (v_Duration > float2int(v_T_reordering)){ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); } //@siclog "Step 17" siclog@ The SS sends the PDCP SDU #9 p_NR_PDCP_State.TX_NEXT := 9; f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_SduList[1]); }