3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250518

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3787 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RedCap emergency call test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-12-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Emergency call shall allow maximum flexibility for the UE to find all available networks and RAN5 shall not enforce capability restrictions without core spec justification - List of emergency call test cases need to be defined for use in Change 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Skip check of pc_supportOfRedCap_r17 during emergency call test cases. List of such test cases to be defined in Change 2. - Define emergency call test cases under TestcaseProperties |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.2.NR5GC, 11.3.7.NR5GC, 11.4.x.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:  Kalahasti, Narendra
Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | function fl_NR_CheckRedCapUEIndicator() |
| --- | --- |
| Reason for change | Emergency call shall allow maximum flexibility for the UE to find all available networks and RAN5 shall not enforce capability restrictions without core spec justification |
| Summary of change | Skip check of pc_supportOfRedCap_r17 during emergency call test cases. List of such test cases to be defined in Change 2. |
| TTCN module | NR5GC_RRCSteps.ttcn |

Before Change:

function fl_NR_CheckRedCapUEIndicator(UE_NR_Capability p_NRCapability) { // @sic R5s240258 sic@ var UE_NR_Capability v_Received_NRCapabilityMsg; var template (omit) integer v_UE_Release := omit; v_Received_NRCapabilityMsg := p_NRCapability; if (pc_supportOfRedCap_r17) { // Checks done in case support of RedCap PICS is set if (isvalue(v_Received_NRCapabilityMsg.accessStratumRelease)) { v_UE_Release := f_NR_ConvertAccessStratumReleaseToInteger(v_Received_NRCapabilityMsg.accessStratumRelease); // Check the UE release } if (valueof(v_UE_Release) < 17 ) //valueof is OK as it is checked in v_Received_NRCapabilityMsg.accessStratumRelease { f_SetVerdictInconc(__FILE__, __LINE__, "UE did not report rel-17 or higher with pc_supportOfRedCap_r17 enabled"); } else { if (not (ispresent(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.redCapParameters_r17.supportOfRedCap_r17))) { f_SetVerdictInconc(__FILE__, __LINE__, "pc_supportOfRedCap_r17 is set but UE did not signal capability for supportOfRedCap_r17"); } } } }

After Change:

function fl_NR_CheckRedCapUEIndicator(UE_NR_Capability p_NRCapability) { // @sic R5s240258 sic@ var UE_NR_Capability v_Received_NRCapabilityMsg; var template (omit) integer v_UE_Release := omit; v_Received_NRCapabilityMsg := p_NRCapability; if (pc_supportOfRedCap_r17) { // Checks done in case support of RedCap PICS is set if (isvalue(v_Received_NRCapabilityMsg.accessStratumRelease)) { v_UE_Release := f_NR_ConvertAccessStratumReleaseToInteger(v_Received_NRCapabilityMsg.accessStratumRelease); // Check the UE release } if (valueof(v_UE_Release) < 17 ) //valueof is OK as it is checked in v_Received_NRCapabilityMsg.accessStratumRelease { f_SetVerdictInconc(__FILE__, __LINE__, "UE did not report rel-17 or higher with pc_supportOfRedCap_r17 enabled"); } else if (not f_GetTestcaseAttrib_EmergencyCall(testcasename())) // relax RedCap check for emergency call { if (not (ispresent(v_Received_NRCapabilityMsg.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.nonCriticalExtension.redCapParameters_r17.supportOfRedCap_r17))) { f_SetVerdictInconc(__FILE__, __LINE__, "pc_supportOfRedCap_r17 is set but UE did not signal capability for supportOfRedCap_r17"); } } } }

## Change 2

| Function name | function f_GetTestcaseAttrib_EmergencyCall() |
| --- | --- |
| Reason for change | List of emergency call test cases need to be defined for use in Change 1. |
| Summary of change | Define emergency call test cases under TestcaseProperties |
| TTCN module | NR5GC/TestcaseProperties.ttcn |

New Function:

function f_GetTestcaseAttrib_EmergencyCall(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_2_NR5GC") { return true; } case ("TC_11_3_7_NR5GC") { return true; } case ("TC_11_4_1_NR5GC") { return true; } case ("TC_11_4_1a_NR5GC") { return true; } case ("TC_11_4_2_NR5GC") { return true; } case ("TC_11_4_3_NR5GC") { return true; } case ("TC_11_4_4_NR5GC") { return true; } case ("TC_11_4_5_NR5GC") { return true; } case ("TC_11_4_6_NR5GC") { return true; } case ("TC_11_4_7_NR5GC") { return true; } case ("TC_11_4_8_NR5GC") { return true; } case ("TC_11_4_9_NR5GC") { return true; } } return false; }