3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250039

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4877 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NB-IoT NTN testcase 22.3.2.7a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-01-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to current implementation of TTCN, for the UE, which supports TN and NTN (pc_NB_ntn_only_Connectivity_EPC=false), the preconfigured UE location is not configured. To take this situation into account, change the condition of judgement from supporting TN only to the type of NTN satellite. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace the judgement of pc_NB_ntn_only_Connectivity_EPC with “f_NBIOT_MobileInfo_GetNTNSatelliteType()!= ntn_None”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant shall fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.3.2.7a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact:	Parikshit Bhise
	Parikshit.bhise@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_NBIOT_Preamble () |
| --- | --- |
| Reason for change | According to current implementation of TTCN, for the UE, which supports TN and NTN (pc_NB_ntn_only_Connectivity_EPC=false), the preconfigured UE location is not configured. To take this situation into account, change the condition of judgement from supporting TN only to the type of NTN satellite. |
| Summary of change | Replace the judgement of pc_NB_ntn_only_Connectivity_EPC with “f_NBIOT_MobileInfo_GetNTNSatelliteType()!= ntn_None”. |
| TTCN module | NBIOT_CommonProcedures.ttcn |
| MCC160 Comment | Accepted, but different implementation is proposed. The f_NBIOT_Preamble is used in the majority of the legacy NBIOT test cases, and pc_NB_ntn_only_Connectivity_EPC setting controls the execution of the legacy test case over NTN for the UEs that do not support TN connectivity. However, 22.3.2.7a is a pure NTN test case and also must be run UEs that support both. To achieve this and to keep UE capability-driven test case execution control it is proposed to do a pre-configuration for UE with pc_NB_ntn_Connectivity_EPC in the test case function f_TC_22_3_2_7a_NBIOT(), see proposed implementation Additional comments from MCC160: Taking into consideration that NTN-only capable UE must also set pc_NB_ntn_Connectivity_EPC to True, to ensure that pre-configuration won’t be done twice, the capability check within the test case must be extended with a negative check on pc_NB_ntn_only_Connectivity_EPC |

Before Change

function f_NBIOT_Preamble(NBIOT_CellId_Type p_CellId, IOT_STATE_Type p_IOT_State, NBIOT_IDLEUPDATED_STATE_Type p_State := STATE2_NB_CONNECTED_UPDATED, template (omit) ExtdDRXParams p_ExtdDRXParams := omit //@sic R5-212889 sic@ ) runs on NBIOT_PTC { timer t_GuardTimer := tsc_GuardTimePreamble_12min; // @sic R5s200494 sic@ t_GuardTimer.start; // First switch cell back on f_NBIOT_SetCellPower(p_CellId, tsc_ServingCellRS_EPRE); // Switch on UE f_UT_SwitchOnUE(UT, false); //Preconfigure UE supporting NTN only access in NB-IoT with UE position if(pc_NB_ntn_only_Connectivity_EPC) {//@sic R5s240281, R5-237126, R5s240092 sic@ //The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13. f_NBIOT_PreconfigureNTN_UE(); // @sic R5w240103 sic@ } // Now continue with the procedure f_NBIOT_InitialRegistration(p_CellId, p_IOT_State, PREAMBLE, p_State, -, p_ExtdDRXParams); t_GuardTimer.stop; }

After Change

function f_NBIOT_Preamble(NBIOT_CellId_Type p_CellId, IOT_STATE_Type p_IOT_State, NBIOT_IDLEUPDATED_STATE_Type p_State := STATE2_NB_CONNECTED_UPDATED, template (omit) ExtdDRXParams p_ExtdDRXParams := omit //@sic R5-212889 sic@ ) runs on NBIOT_PTC { timer t_GuardTimer := tsc_GuardTimePreamble_12min; // @sic R5s200494 sic@ t_GuardTimer.start; // First switch cell back on f_NBIOT_SetCellPower(p_CellId, tsc_ServingCellRS_EPRE); // Switch on UE f_UT_SwitchOnUE(UT, false); //Preconfigure UE supporting NTN only access in NB-IoT with UE position if(f_NBIOT_MobileInfo_GetNTNSatelliteType()!= ntn_None) //WI=1356999 {//@sic R5s240281, R5-237126, R5s240092 sic@ //The preconfigured UE location is defined in TS 36.508 [18] Clause 4.13. f_NBIOT_PreconfigureNTN_UE(); // @sic R5w240103 sic@ } // Now continue with the procedure f_NBIOT_InitialRegistration(p_CellId, p_IOT_State, PREAMBLE, p_State, -, p_ExtdDRXParams); t_GuardTimer.stop; }

MCC160 proposed implementation

function f_TC_22_3_2_7a_NBIOT() runs on NBIOT_PTC { /* NB-IoT / NTN / AM RLC / Receiver status triggers / extended t-Reordering configured */ var RLC_SS_State v_RLC_Rec; var template (value) DL_CCCH_Message_NB v_RrcConnSetup; var T_PollRetransmit_NB_r13 v_T_PollRetransmit_NB_r13; var RLC_Config_NB_v1430.t_Reordering_r14 v_T_Reordering_r14; var RLC_Config_NB_v1700.t_ReorderingExt_r17.setup v_T_Reordering_r17; var UL_AM_RLC_NB_r13.maxRetxThreshold_r13 v_MaxRetxThreshold_r13; var template (value) SRB_ToAddModList_NB_r13 v_SRB_ToAddModList_NB_r13; var integer v_HARQ_ProcessNumber := 0; //RLC global initialisation f_InitRLC_Record(v_RLC_Rec, AM_Mode); //Initialise acc. Table 22.3.2.7a.3.3-1 v_T_PollRetransmit_NB_r13 := ms4000; v_RLC_Rec.t_PollRetransmit_r13 := 4.0; // t_PollRetransmit_r13 set to 4000 ms v_MaxRetxThreshold_r13 := t2; v_RLC_Rec.maxRetxThreshold_r13 := 2; // maxRetxThreshold_r13 set to 2 (=t2) v_RLC_Rec.enableStatusReportSN_Gap_r13 := true; // enableStatusReportSN-Gap-r13 set to true v_T_Reordering_r14 := ms1600_v1310; v_RLC_Rec.t_Reordering_r14 := 1.6; // t_Reordering_r14 set to 1600 ms v_T_Reordering_r17 := ms3200; v_RLC_Rec.t_Reordering_r17 :=3.2; //t_Reordering_r17 set to 3200 ms //UE RLC settings are as initialized v_SRB_ToAddModList_NB_r13 := {cs_SRB_ToAddMod_NB_RLC_Config (cs_UL_AM_RLC_NB_r13 (v_T_PollRetransmit_NB_r13, v_MaxRetxThreshold_r13), cs_DL_AM_RLC_NB_r13_GapReport, cs_RLC_Config_NB_v1430(v_T_Reordering_r14))}; v_SRB_ToAddModList_NB_r13[0].rlc_Config_v1700.t_ReorderingExt_r17.setup := v_T_Reordering_r17; //acc to Table 22.3.2.7a.3.3-1 // RRCConnectionSetup-NB with twoHARQ-ProcessesConfig-r14 configured v_RrcConnSetup := f_NBIOT_508_RRCConnectionSetup(-, v_SRB_ToAddModList_NB_r13, -, -, true_); if(pc_NB_ntn_Connectivity_EPC and not pc_NB_ntn_only_Connectivity_EPC) {// @sic R5s240065, R5w240103, R5s240429, R5s250039 sic@ f_NBIOT_PreconfigureNTN_UE(); } // perform NBIOT specific preamble for RLC f_NBIOT_RLC_Preamble_State2B_NB(v_RLC_Rec, nbiot_Cell1, STATE2A_NB_TESTLOOP_ModeG, // @sic R5-174405 sic@ v_RrcConnSetup, v_HARQ_ProcessNumber); f_NBIOT_TestBody_Set(true, layer2); f_NBIOT_HarqErrorHandling_Set(raiseInconc); fl_TC_22_3_2_7a_TestBody (v_RLC_Rec); f_NBIOT_HarqErrorHandling_Set(ignore); f_NBIOT_TestBody_Set(false); f_NBIOT_L2_Postamble_State2B_NB(nbiot_Cell1, v_RLC_Rec.AM_VTS, v_RLC_Rec.AM_VRR); }