3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250063

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0964 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR5GC IMS emergency call test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | IMS emergency test cases will not support by default IMS geolocation usage, in prose it is specified that in 10.1.NR5GC UE must support it and 10.4.NR5GC UE may support it (depending on UE implementation). IMS Geolocation support is verified with A8 condition in Annex A.2.1 in TS 34.229-1, requiring UE support of the feature and also, by different means, Location Information being provided by SS. Current implementation presents misaligments in different test cases, where some of them will handle UE INVITE including Geolocation information and in some others, SS providing the Location. This CR is raised to only accept and handle geolocation services for test cases defined for this purpose, 10.1.NR5GC and 10.4.NR5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modified test cases to not provide Location information as well as not expect any geolocation information provided by UE if not specficied in prose. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A Conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.NR5GC, 10.6.NR5GC, 10.9.NR5GC, 10.10.NR5GC, 10.12.NR5GC, and 10.13.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Correction to f_TC_10_3_NR5GC_IMS1	2

2.2	Correction to f_TC_10_6_NR5GC_IMS1	4

2.3	Correction to f_TC_10_9_NR5GC_IMS1	6

2.4	Correction to f_TC_10_9_NR5GC_IMS2	8

2.5	Correction to f_TC_10_3_NR5GC_IMS1	10

2.6	Correction to f_TC_10_3_NR5GC_IMS1	10

# Overview

This document lists all the essential changes needed to correct problems in the ATS iwd-TTCN3-B2024-06_D24wk50 related to the title of this CR.

Contact: 	Juan Garcia Martin

Juan.GarciaMartin@rohde-schwarz.com

# Corrections required

## Correction to f_TC_10_3_NR5GC_IMS1

| Function name | f_TC_10_3_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Current implementation presents misaligments in different test cases, where some of them will handle UE INVITE including Geolocation information and in some others, SS providing the Location. |
| Summary of change | Modified test case to not support IMS geolocation if not specficied in prose. |
| TTCN module | /IMS/NR5GC_10/IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_10_3_NR5GC_IMS1() runs on IMS_PTC { // Emergency call with emergency registration / Emergency SIP signalling and media in parallel with another ongoing IM CN subsystem signalling and media / 5GS var INVITE_Request v_InviteRequest; var IMS_CC_MO_CallInfo_Type v_CallInfo; var boolean v_GeolocationUsage := pc_IMS_Geolocation; f_IMS_CC_Preamble(IPCAN_SpeechAndEmergencyCall, IMS_REGISTERED); // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { // @sic R5s220215 add. change sic@ f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); v_CallInfo := f_IMS_MOCallSetup_NR5GC_A_4_1a(); /* @sic R5-246477: f_IMS_MOCallSetup_NR5GC_A_4_1a instead of f_IMS_MOCallSetup_NR5GC_A_4_1 sic@ */ v_InviteRequest := v_CallInfo.InviteRequestWithSdp.Invite; f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ // UE is made to hold the ongoing call f_UT_CallHold(MMI); <<SKIPPED CODE>>

After Change:

function f_TC_10_3_NR5GC_IMS1() runs on IMS_PTC { // Emergency call with emergency registration / Emergency SIP signalling and media in parallel with another ongoing IM CN subsystem signalling and media / 5GS var INVITE_Request v_InviteRequest; var IMS_CC_MO_CallInfo_Type v_CallInfo; <<REMOVEED CODE>> var boolean v_GeolocationUsage := pc_IMS_Geolocation; <<REMOVEED CODE>> f_IMS_CC_Preamble(IPCAN_SpeechAndEmergencyCall, IMS_REGISTERED); <<REMOVEED CODE>> // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { // @sic R5s220215 add. change sic@ f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } <<REMOVEED CODE>> f_IMS_CC_StartCall(IPCAN_MO_SpeechCall); v_CallInfo := f_IMS_MOCallSetup_NR5GC_A_4_1a(); /* @sic R5-246477: f_IMS_MOCallSetup_NR5GC_A_4_1a instead of f_IMS_MOCallSetup_NR5GC_A_4_1 sic@ */ v_InviteRequest := v_CallInfo.InviteRequestWithSdp.Invite; f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ // UE is made to hold the ongoing call f_UT_CallHold(MMI); <<SKIPPED CODE>>

## Correction to f_TC_10_6_NR5GC_IMS1

| Function name | f_TC_10_6_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Current implementation presents misaligments in different test cases, where some of them will handle UE INVITE including Geolocation information and in some others, SS providing the Location. |
| Summary of change | Modified test case to not support IMS geolocation if not specficied in prose. |
| TTCN module | /IMS/NR5GC_10/IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_10_6_NR5GC_IMS1() runs on IMS_PTC { // Non-UE detectable emergency call / IM CN sends 380 with an Alternative Service / Previous emergency IMS registration not expired / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var template (omit) MessageBody v_MessageBody := omit; var template (present) MessageHeader v_MessageHeader_Ack; var template (value) MessageHeader v_MessageHeader_AltService_380; var INVITE_Request v_InviteRequest; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var charstring v_CalleeUri := px_IMS_CalleeUri; var template (present) SipUrl v_CalleeSipUrl := f_SIP_BuildSipUri_RX(px_IMS_CalleeUri); var boolean v_GeolocationUsage := pc_IMS_Geolocation; f_IMS_CC_Preamble(IPCAN_SpeechAndEmergencyCall, IMS_REGISTERED); //@sic R5-225426 sic@ // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); // Steps 3b1-3b3 of generic procedure specified in Table 4.9.12B.2.2-1 of TS 38.508-1 [21] are performed to release emergency speech bearer and Keep emergency PDU session. //@siclog "Step 19" siclog@ // UE is made to attempt an IMS voice call f_UT_RequestIMSCall(MMI, v_CalleeUri); <<SKIPPED CODE>>

After Change:

function f_TC_10_6_NR5GC_IMS1() runs on IMS_PTC { // Non-UE detectable emergency call / IM CN sends 380 with an Alternative Service / Previous emergency IMS registration not expired / 5GS var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var template (omit) MessageBody v_MessageBody := omit; var template (present) MessageHeader v_MessageHeader_Ack; var template (value) MessageHeader v_MessageHeader_AltService_380; var INVITE_Request v_InviteRequest; var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var charstring v_CalleeUri := px_IMS_CalleeUri; var template (present) SipUrl v_CalleeSipUrl := f_SIP_BuildSipUri_RX(px_IMS_CalleeUri); <<REMOVED CODE>> var boolean v_GeolocationUsage := pc_IMS_Geolocation; <<REMOVED CODE>> f_IMS_CC_Preamble(IPCAN_SpeechAndEmergencyCall, IMS_REGISTERED); //@sic R5-225426 sic@ <<REMOVED CODE>> // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } <<REMOVED CODE>> f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); // Steps 3b1-3b3 of generic procedure specified in Table 4.9.12B.2.2-1 of TS 38.508-1 [21] are performed to release emergency speech bearer and Keep emergency PDU session. //@siclog "Step 19" siclog@ // UE is made to attempt an IMS voice call f_UT_RequestIMSCall(MMI, v_CalleeUri); <<SKIPPED CODE>>

## Correction to f_TC_10_9_NR5GC_IMS1

| Function name | f_TC_10_9_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Current implementation presents misaligments in different test cases, where some of them will handle UE INVITE including Geolocation information and in some others, SS providing the Location. |
| Summary of change | Modified test case to not support IMS geolocation if not specficied in prose. |
| TTCN module | /IMS/NR5GC_10/IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_10_9_NR5GC_IMS1() runs on IMS_PTC { // Emergency call without emergency registration / UE credentials are not accepted / 5GS var boolean v_GeolocationUsage := pc_IMS_Geolocation; f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED); // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); // @sic R5s230677 sic@ f_IMS_CC_Postamble(); // @sic R5s230677 sic@ no test procedure as connection is already released in IMS2 }

After Change:

function f_TC_10_9_NR5GC_IMS1() runs on IMS_PTC { // Emergency call without emergency registration / UE credentials are not accepted / 5GS <<SKIPPED CODE>> var boolean v_GeolocationUsage := pc_IMS_Geolocation; <<SKIPPED CODE>> f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED); <<SKIPPED CODE>> // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } <<SKIPPED CODE>> f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); // @sic R5s230677 sic@ f_IMS_CC_Postamble(); // @sic R5s230677 sic@ no test procedure as connection is already released in IMS2 }

## Correction to f_TC_10_9_NR5GC_IMS2

| Function name | f_TC_10_9_NR5GC_IMS2 |
| --- | --- |
| Reason for change | Current implementation presents misaligments in different test cases, where some of them will handle UE INVITE including Geolocation information and in some others, SS providing the Location. |
| Summary of change | Modified test case to not support IMS geolocation if not specficied in prose. |
| TTCN module | /IMS/NR5GC_10/IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_10_9_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var SipUrl v_ContactUrlUE; var template (value) MessageHeader v_MessageHeader; var template (value) MessageBody v_MessageBody; var IMS_DATA_REQ v_IMS_DATA_REQ; var REGISTER_Request v_RegisterRequest; var boolean v_CondA7 := true; var boolean v_GeolocationUsage := pc_IMS_Geolocation; /* the test case intends to use GeoLocationInformation if supported by the UE (PICS) @sic R5s150846 sic@ INVITE (step 1 of Annex C.22) refers to condition A8 of A.2.1 */ f_IMS_PTC_Init(PDN_2); // Test Case Body f_IMS_TestBody_Set(true); <<SKIPPED CODE>> // EXCEPTION: In parallel to steps 16-20 below, steps 16-18 of generic procedure specified in Table 4.9.12.2.2-1 of TS 38.508-1 [21] takes place. //@siclog "Steps 16 - 20" siclog@ //Check: Does the UE send a correctly composed INVITE request?(Step 1 of annex A.6) v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(A_2_1_A6, v_GeolocationUsage); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Steps 16"); // Emergency call setup: 34.229-5 Annex A.6 f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20"); <<SKIPPED CODE>>

After Change:

function f_TC_10_9_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var SipUrl v_ContactUrlUE; var template (value) MessageHeader v_MessageHeader; var template (value) MessageBody v_MessageBody; var IMS_DATA_REQ v_IMS_DATA_REQ; var REGISTER_Request v_RegisterRequest; var boolean v_CondA7 := true; <<REMOVED CODE >> //var boolean v_GeolocationUsage := pc_IMS_Geolocation; /* the test case intends to use GeoLocationInformation if supported by the UE (PICS) @sic R5s150846 sic@ // INVITE (step 1 of Annex C.22) refers to condition A8 of A.2.1 */ <<REMOVED CODE>> f_IMS_PTC_Init(PDN_2); // Test Case Body f_IMS_TestBody_Set(true); <<SKIPPED CODE>> // EXCEPTION: In parallel to steps 16-20 below, steps 16-18 of generic procedure specified in Table 4.9.12.2.2-1 of TS 38.508-1 [21] takes place. //@siclog "Steps 16 - 20" siclog@ //Check: Does the UE send a correctly composed INVITE request?(Step 1 of annex A.6) v_IMS_DATA_REQ := f_IMS_EmergencyCallSetup_NR5GC_A_6_Step1(A_2_1_A6); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Steps 16"); // Emergency call setup: 34.229-5 Annex A.6 f_IMS_EmergencyCallSetup_NR5GC_A_6_Step2ToEnd(v_IMS_DATA_REQ); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 20");<<SKIPPED CODE>>

## Correction to f_TC_10_10_NR5GC_IMS1

| Function name | f_TC_10_10_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Current implementation presents misaligments in different test cases, where some of them will handle UE INVITE including Geolocation information and in some others, SS providing the Location. |
| Summary of change | Modified test case to not support IMS geolocation if not specficied in prose. |
| TTCN module | /IMS/NR5GC_10/IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_10_10_NR5GC_IMS1() runs on IMS_PTC { // Emergency call without emergency registration / Failure of registration / Rejected by 403(Forbidden) var boolean v_GeolocationUsage := pc_IMS_Geolocation; //The UE is in test state 1N-A (TS 38.508-1) and registered to IMS. f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED, -, -, -, -, IPCAN_UseVPLMN); //@sic R5s220611 R5-223465 sic@ // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); // @sic R5s230677 sic@ f_IMS_CC_Postamble(); // @sic R5s230677 sic@ no test procedure as connection is already released in IMS2 }

After Change:

function f_TC_10_10_NR5GC_IMS1() runs on IMS_PTC { // Emergency call without emergency registration / Failure of registration / Rejected by 403(Forbidden) <<REMOVED CODE>> var boolean v_GeolocationUsage := pc_IMS_Geolocation; <<REMOVED CODE>> //The UE is in test state 1N-A (TS 38.508-1) and registered to IMS. f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED, -, -, -, -, IPCAN_UseVPLMN); //@sic R5s220611 R5-223465 sic@ <<REMOVED CODE>> // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } <<REMOVED CODE>> f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); // @sic R5s230677 sic@ f_IMS_CC_Postamble(); // @sic R5s230677 sic@ no test procedure as connection is already released in IMS2 }

## Correction to f_TC_10_12_NR5GC_IMS1

| Function name | f_TC_10_12_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Current implementation presents misaligments in different test cases, where some of them will handle UE INVITE including Geolocation information and in some others, SS providing the Location. |
| Summary of change | Modified test case to not support IMS geolocation if not specficied in prose. |
| TTCN module | /IMS/NR5GC_10/IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_10_12_NR5GC_IMS1() runs on IMS_PTC { /* User-initiated emergency reregistration / UE has emergency related ongoing dialog / 5GS */ var boolean v_GeolocationUsage := pc_IMS_Geolocation; //The UE is in test state 1N-A (TS 38.508-1) and registered to IMS. f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED); // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); // @sic R5s230677 sic@ f_IMS_CC_Postamble(); // @sic R5s230677 sic@ no test procedure as connection is already released in IMS2 }

After Change:

function f_TC_10_12_NR5GC_IMS1() runs on IMS_PTC { /* User-initiated emergency reregistration / UE has emergency related ongoing dialog / 5GS */ <<REMOVED CODE>> var boolean v_GeolocationUsage := pc_IMS_Geolocation; <<REMOVED CODE>> //The UE is in test state 1N-A (TS 38.508-1) and registered to IMS. f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED); <<REMOVED CODE>> // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } <<REMOVED CODE>> f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); // @sic R5s230677 sic@ f_IMS_CC_Postamble(); // @sic R5s230677 sic@ no test procedure as connection is already released in IMS2 }

## Correction to f_TC_10_12_NR5GC_IMS2

| Function name | f_TC_10_12_NR5GC_IMS2 |
| --- | --- |
| Reason for change | Current implementation presents misaligments in different test cases, where some of them will handle UE INVITE including Geolocation information and in some others, SS providing the Location. |
| Summary of change | Modified test case to not support IMS geolocation if not specficied in prose. |
| TTCN module | /IMS/NR5GC_10/IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_10_12_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var SipUrl v_ContactUrlUE; var boolean v_GeolocationUsage := pc_IMS_Geolocation; var integer v_Expiry := 120; var float v_ReRegistrationTime := int2float(v_Expiry / 2); timer t_RegExpireMin := f_IMS_SetTimerToleranceMin(v_ReRegistrationTime); timer t_RegExpireMax := f_IMS_SetTimerToleranceMax(v_ReRegistrationTime); f_IMS_PTC_Init(PDN_2); // Test Case Body f_IMS_TestBody_Set(true); f_IMS_IMS_WaitForTrigger(OtherPDN); //@siclog "Step 1" siclog@ //UE is made to initiate an emergency call f_IMS_CC_StartCall(IPCAN_EmergencyCall_OptionalPDNRelease); //@sic R5-225362 sic@ //@siclog "Steps 2-5" siclog@ //Check: Does the UE correctly initiate emergency registration ? (Steps 1-3 of Annex A.3) //SS responds 200 OK to finish the registration, setting the expiration time to 120 seconds. (Steps 4 of Annex A.3) f_IMS_EmergencyRegistration(-, int2str(v_Expiry)); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // start re-registration timer t_RegExpireMin.start; t_RegExpireMax.start; //@siclog "Steps 6-10" siclog@ // Check: Does the UE correctly initiate and complete the establishment of an emergency voice call? (Steps 1-5 of Annex A.6) f_IMS_EmergencyCallSetup_NR5GC_A_6(-, v_GeolocationUsage); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10");

After Change:

function f_TC_10_12_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var SipUrl v_ContactUrlUE; <<REMOVED CODE>> var boolean v_GeolocationUsage := pc_IMS_Geolocation; <<REMOVED CODE>> var integer v_Expiry := 120; var float v_ReRegistrationTime := int2float(v_Expiry / 2); timer t_RegExpireMin := f_IMS_SetTimerToleranceMin(v_ReRegistrationTime); timer t_RegExpireMax := f_IMS_SetTimerToleranceMax(v_ReRegistrationTime); f_IMS_PTC_Init(PDN_2); // Test Case Body f_IMS_TestBody_Set(true); f_IMS_IMS_WaitForTrigger(OtherPDN); //@siclog "Step 1" siclog@ //UE is made to initiate an emergency call f_IMS_CC_StartCall(IPCAN_EmergencyCall_OptionalPDNRelease); //@sic R5-225362 sic@ //@siclog "Steps 2-5" siclog@ //Check: Does the UE correctly initiate emergency registration ? (Steps 1-3 of Annex A.3) //SS responds 200 OK to finish the registration, setting the expiration time to 120 seconds. (Steps 4 of Annex A.3) f_IMS_EmergencyRegistration(-, int2str(v_Expiry)); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // start re-registration timer t_RegExpireMin.start; t_RegExpireMax.start; //@siclog "Steps 6-10" siclog@ // Check: Does the UE correctly initiate and complete the establishment of an emergency voice call? (Steps 1-5 of Annex A.6) f_IMS_EmergencyCallSetup_NR5GC_A_6(); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10");

## Correction to f_TC_10_13_NR5GC_IMS1

| Function name | f_TC_10_13_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Current implementation presents misaligments in different test cases, where some of them will handle UE INVITE including Geolocation information and in some others, SS providing the Location. |
| Summary of change | Modified test case to not support IMS geolocation if not specficied in prose. |
| TTCN module | /IMS/NR5GC_10/IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_10_13_NR5GC_IMS1() runs on IMS_PTC { /* User-initiated emergency reregistration / User initiates an emergency call / 5GS */ var boolean v_GeolocationUsage := pc_IMS_Geolocation; //The UE is in test state 1N-A (TS 38.508-1) and registered to IMS. f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED); // Trigger SS to execute location related functions to allow UE to get its position if (v_GeolocationUsage) { f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); // @sic R5s230677 sic@ f_IMS_CC_Postamble(); // @sic R5s230677 sic@ no test procedure as connection is already released in IMS2 }

After Change:

function f_TC_10_13_NR5GC_IMS1() runs on IMS_PTC { /* User-initiated emergency reregistration / User initiates an emergency call / 5GS */ <<REMOVED CODE>> var boolean v_GeolocationUsage := pc_IMS_Geolocation; <<REMOVED CODE>> //The UE is in test state 1N-A (TS 38.508-1) and registered to IMS. f_IMS_CC_Preamble(IPCAN_EmergencyCall, IMS_REGISTERED); // Trigger SS to execute location related functions to allow UE to get its position <<REMOVED CODE>> if (v_GeolocationUsage) { f_IMS_IPCAN_UpdateUELocationInformation(IPCAN); } <<REMOVED CODE>> f_IMS_IMS_SendCoOrdMsg(OtherPDN); f_IMS_IMS_WaitForTrigger(OtherPDN); // @sic R5s230677 sic@ f_IMS_CC_Postamble(); // @sic R5s230677 sic@ no test procedure as connection is already released in IssMS2 }

## Correction to f_TC_10_13_NR5GC_IMS2

| Function name | f_TC_10_13_NR5GC_IMS2 |
| --- | --- |
| Reason for change | Current implementation presents misaligments in different test cases, where some of them will handle UE INVITE including Geolocation information and in some others, SS providing the Location. |
| Summary of change | Modified test case to not support IMS geolocation if not specficied in prose. |
| TTCN module | /IMS/NR5GC_10/IMS_NR5GC_EmergencyCallTestcases.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_10_13_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var IMS_DATA_REQ v_IMS_DATA_REQ; var INVITE_Request v_InviteRequest; var SipUrl v_ContactUrlUE; var boolean v_GeolocationUsage := pc_IMS_Geolocation; var integer v_Expiry := 30; var float v_ReRegistrationTime := int2float(v_Expiry / 2); var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (present) SDP_Message v_SDP_Message_Step1; var SDP_Message v_SDP_Message; var IMS_CallType_Type v_EmergencyType := REGULAR_EMERGENCY; /* @sic R5s150692 change 25 sic@ */ var IMS_SDP_AudioCodecInfo_Type v_AudioCodecInfo; var template (present) SipUrl v_CalleeContactSipUri; var template (value) MessageHeader v_MessageHeader_InviteResponse_200; var template (present) MessageHeader v_MessageHeader_Ack; var template (value) SDP_Message v_SDP_Message_Step4; timer t_RegExpireMax := v_ReRegistrationTime * 2.0; // to allow for a period between reregistration and reregistration expiry @sic R5s170876 sic@ f_IMS_PTC_Init(PDN_2); // Test Case Body f_IMS_TestBody_Set(true); f_IMS_IMS_WaitForTrigger(OtherPDN); //@siclog "Step 1" siclog@ //UE is made to initiate an emergency call f_IMS_CC_StartCall(IPCAN_EmergencyCall_OptionalPDNRelease); //@sic R5-225362 sic@ //@siclog "Steps 2-5" siclog@ //Check: Does the UE correctly initiate emergency registration ? (Steps 1-3 of Annex A.3) //SS responds 200 OK to finish the registration. (Note: the SS sets the expiration time to 30 seconds.) (Step 4 of Annex A.3) f_IMS_EmergencyRegistration(-, int2str(v_Expiry)); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); t_RegExpireMax.start; //@siclog "Step 6-8" siclog@ // Check: Does the UE correctly initiate and process the establishment of an emergency voice call? (Steps 1-3 of Annex A.6) // Receive INVITE for an Emergency call v_SDP_Message_Step1 := f_IMS_BuildSDP_EmergencyCallOffer_RX(); v_IMS_DATA_REQ := f_IMS_INVITE_ReceiveRequest(A_2_1_A7, -, -, -, v_GeolocationUsage); v_InviteRequest := v_IMS_DATA_REQ.Request.Invite; v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo); v_SDP_Message := f_IMS_MessageBody_CheckSdpMessageAndGeolocation(v_InviteRequest.messageBody, v_SDP_Message_Step1, v_InviteRequest.msgHeader.geolocation); v_AudioCodecInfo := f_IMS_SDP_GetAudioCodecInfo(v_SDP_Message, { AMR_16000 });

After Change:

function f_TC_10_13_NR5GC_IMS2() runs on IMS_PTC { /* IMS implementation for PDN2 - Emergency call */ var IMS_DATA_REQ v_IMS_DATA_REQ; var INVITE_Request v_InviteRequest; var SipUrl v_ContactUrlUE; <<REMOVED CODE>> var boolean v_GeolocationUsage := pc_IMS_Geolocation; <<REMOVED CODE>> var integer v_Expiry := 30; var float v_ReRegistrationTime := int2float(v_Expiry / 2); var template (value) IMS_RoutingInfo_Type v_RoutingInfo_DL; var template (present) SDP_Message v_SDP_Message_Step1; var SDP_Message v_SDP_Message; var IMS_CallType_Type v_EmergencyType := REGULAR_EMERGENCY; /* @sic R5s150692 change 25 sic@ */ var IMS_SDP_AudioCodecInfo_Type v_AudioCodecInfo; var template (present) SipUrl v_CalleeContactSipUri; var template (value) MessageHeader v_MessageHeader_InviteResponse_200; var template (present) MessageHeader v_MessageHeader_Ack; var template (value) SDP_Message v_SDP_Message_Step4; timer t_RegExpireMax := v_ReRegistrationTime * 2.0; // to allow for a period between reregistration and reregistration expiry @sic R5s170876 sic@ f_IMS_PTC_Init(PDN_2); // Test Case Body f_IMS_TestBody_Set(true); f_IMS_IMS_WaitForTrigger(OtherPDN); //@siclog "Step 1" siclog@ //UE is made to initiate an emergency call f_IMS_CC_StartCall(IPCAN_EmergencyCall_OptionalPDNRelease); //@sic R5-225362 sic@ //@siclog "Steps 2-5" siclog@ //Check: Does the UE correctly initiate emergency registration ? (Steps 1-3 of Annex A.3) //SS responds 200 OK to finish the registration. (Note: the SS sets the expiration time to 30 seconds.) (Step 4 of Annex A.3) f_IMS_EmergencyRegistration(-, int2str(v_Expiry)); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); t_RegExpireMax.start; //@siclog "Step 6-8" siclog@ // Check: Does the UE correctly initiate and process the establishment of an emergency voice call? (Steps 1-3 of Annex A.6) // Receive INVITE for an Emergency call v_SDP_Message_Step1 := f_IMS_BuildSDP_EmergencyCallOffer_RX(); v_IMS_DATA_REQ := f_IMS_INVITE_ReceiveRequest(A_2_1_A7); v_InviteRequest := v_IMS_DATA_REQ.Request.Invite; v_RoutingInfo_DL := f_IMS_RoutingInfo_ULtoDL(v_IMS_DATA_REQ.RoutingInfo); v_SDP_Message := f_IMS_MessageBody_CheckSdpMessageAndGeolocation(v_InviteRequest.messageBody, v_SDP_Message_Step1, v_InviteRequest.msgHeader.geolocation); v_AudioCodecInfo := f_IMS_SDP_GetAudioCodecInfo(v_SDP_Message, { AMR_16000 });