3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250155

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3660 | rev | - | Current version: | 18.2.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC SOR testcase 6.3.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint, TDIA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, eCPSOR_CON-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1 Timer p_SorcmValue should be adjusted for the 10% tolerance, SS may receive DEREGISTRATION REQUEST message before Tsor-cm timer expires. 2 The iel is not correct if pass omit to p_CriterionValue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1 Called function f_NR_SetTimerToleranceMin on timer p_SorcmValue. 2 Get iel according to the value of p_CriterionValue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	5

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk11 related to the title of this CR.

Contact: Ting Gao
gaoting@starpointcomm.com

- Corrections required

  - Change 1

| Function name | f_NR5GC_TransportSorCmCheckfunction |
| --- | --- |
| Reason for change | Timer p_SorcmValue should be adjusted for the 10% tolerance, SS may receive DEREGISTRATION REQUEST message before Tsor-cm timer expires. |
| Summary of change | Called function f_NR_SetTimerToleranceMin on timer p_SorcmValue. |
| TTCN module | SOR_CMCI_NR5GC.ttcn |
| MCC160 Comment | Rejected. It will not align with the prose if updating the Tsor-cm and the tese case should be verdicted to fail if SS receive DEREGISTRATION REQUEST message before Tsor-cm timer expires. |

Before Change:

function f_NR5GC_TransportSorCmCheckfunction(NR_CellId_Type p_NR_CellId, NR_CellId_Type p_NR_TargetCellId, template (value) NG_NAS_DL_Pdu_Type p_PduToSend, template (present) PayloadContainerType p_PayloadContainerType:=cr_PayloadContainerType(tsc_PayloadContainerSORMsg),//LYLY template (omit) charstring p_VerdictMsg := "Step 26 PASS", template (omit) charstring p_SecondVerdictMsg := "Step 28 PASS", float p_TmaxValue:=660.0, float p_SorcmValue:=60.0) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; timer t_Tmax; timer t_Sorcm; //@siclog "Step n+1" siclog@ //The SS transmits an DLInformationTransfer message containing steering of roaming information SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB2, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,p_PduToSend.Msg))); //@siclog "Step n+2" siclog@ //The SS starts timer of Tsor-cm timer. t_Sorcm.start(p_SorcmValue); //@siclog "Step n+3" siclog@ //The SS starts timer of tmax =(6 minutes + cell selection time) //@siclog "Step n+4" siclog@ //The UE transmits an ULInformationTransfer message … }

After Change:

function f_NR5GC_TransportSorCmCheckfunction(NR_CellId_Type p_NR_CellId, NR_CellId_Type p_NR_TargetCellId, template (value) NG_NAS_DL_Pdu_Type p_PduToSend, template (present) PayloadContainerType p_PayloadContainerType:=cr_PayloadContainerType(tsc_PayloadContainerSORMsg),//LYLY template (omit) charstring p_VerdictMsg := "Step 26 PASS", template (omit) charstring p_SecondVerdictMsg := "Step 28 PASS", float p_TmaxValue:=660.0, float p_SorcmValue:=60.0) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_MSG_Indication_Type v_ReceivedMsg; timer t_Tmax; timer t_Sorcm; //@siclog "Step n+1" siclog@ //The SS transmits an DLInformationTransfer message containing steering of roaming information SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB2, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,p_PduToSend.Msg))); //@siclog "Step n+2" siclog@ //The SS starts timer of Tsor-cm timer. t_Sorcm.start(f_NR_SetTimerToleranceMin(p_NR_CellId, nonProtocolTimer, p_SorcmValue)); //@siclog "Step n+3" siclog@ //The SS starts timer of tmax =(6 minutes + cell selection time) //@siclog "Step n+4" siclog@ //The UE transmits an ULInformationTransfer message … }

  - Change 2

| Template name | cs_SOR_CMCIRule |
| --- | --- |
| Reason for change | The iel is not correct if pass omit to p_CriterionValue |
| Summary of change | Get iel according to the value of p_CriterionValue. |
| TTCN module | NG_NAS_Templates.ttcn |
| MCC160 Comment | Rejected. v_Length is already assigned in test case 6.3.2.3 so it is not needed to do this change in common template. Also, this template is used in several cases and it will cause failure if the passed para p_Len is removed. |

Before Change:

template (value) SOR_CMCIRule cs_SOR_CMCIRule (O1_Type p_TsorcmTimer, O1_Type p_CriterionType, template (omit) octetstring p_CriterionValue, octetstring p_Len := '00'O):= // @sic R5s240580 sic@ { /* @status APPROVED (NR5GC) */ /* 24.501 cl. 9.11.3.51 */ iel := p_Len, tSORcmTimer := p_TsorcmTimer, criterionType := p_CriterionType, criterionValue := p_CriterionValue };

After Change:

template (value) SOR_CMCIRule cs_SOR_CMCIRule (O1_Type p_TsorcmTimer, O1_Type p_CriterionType, template (omit) octetstring p_CriterionValue):= { /* @status APPROVED (NR5GC) */ /* 24.501 cl. 9.11.3.51 */ iel := f_get_CMCIRule_length(p_CriterionValue), tSORcmTimer := p_TsorcmTimer, criterionType := p_CriterionType, criterionValue := p_CriterionValue };

  - Change 3

| Function name | f_get_CMCIRule_length |
| --- | --- |
| Reason for change | For change 2. |
| Summary of change | Add a new function |
| TTCN module | NG_NAS_Common.ttcn |
| MCC160 Comment | Not Needed |

New function:

function f_get_CMCIRule_length(template (omit) octetstring p_CriterionValue) return Type6Length_Type { if(ispresent(p_CriterionValue)) { return int2oct(lengthof (p_CriterionValue) + 2, 2); } else { return int2oct(2, 2); } }