3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250184

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4903 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of LTE Access control test case 13.5.6a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2025-04-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To verify LTE RRC testcase 13.5.6a using iWD D25wk11 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to LTE RRC testcase 13.5.6a required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.5.6a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Correction to function f_TC_13_5_6a_EUTRA	3

3	Branches executed	5

4	Execution Log Files	5

5	References	5

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 13.5.6a which is part of the LTE_A_R12 test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Narendra Kalahasti
                         Narendra.kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	13.5.6a

ATS Version:	iwd-TTCN3-B2024-06_D25wk11

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Mediatek MT6897 UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_13_5_6a_EUTRA |
| --- | --- |
| Reason for change | SIB modifications done during the TC initialization will not take into effect since function f_EUTRA_CellConfig_Def is called before the functions that does SIB modifications. |
| Summary of change | Move function f_EUTRA_CellConfig_Def after the SIB modifications are done |
| TTCN module | \LTE_A_R12\8_1\RRC_ConnEst_R12.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_TC_13_5_6a_EUTRA() runs on EUTRA_PTC { // MTSI MO SMS / SCM / 0% access probability skip for MTSI MO SMS over IP / AC-Barring per PLMN f_EUTRA_Init(c1, HANDLE_UM_DATA, USE_BIG_GRANTS); /* @sic R5s170597 - Cleanup of UL grant Configuration: HANDLE_UM_DATA, USE_BIG_GRANTS sic@ */ // @sic R5s180047 sic@ f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_CellInfo_SetPLMN_2Entries (eutra_Cell1, cs_HPLMN_Def, //PLMNid 1 (index 0) cs_HPLMN_002_11, //PLMNid 2 (index 1) -, true, 1); f_EUTRA_Preamble(eutra_Cell1, STATE2_IDLEUPDATE); f_EUTRA_TestBody_Set(true); // @siclog "Step 1-2 Void" siclog@ // @siclog "Step 3" siclog@ f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12 (eutra_Cell1, 1, false, // voice video sig data CSFB omit, omit, omit, cs_AC_BarringConfig(p00, s512, '11111'B), omit, -, -, true_); f_EUTRA_ModifySysinfo(eutra_Cell1); f_TC_13_5_6x_Step4ToEnd_EUTRA(); }

After Change

function f_TC_13_5_6a_EUTRA() runs on EUTRA_PTC { // MTSI MO SMS / SCM / 0% access probability skip for MTSI MO SMS over IP / AC-Barring per PLMN f_EUTRA_Init(c1, HANDLE_UM_DATA, USE_BIG_GRANTS); /* @sic R5s170597 - Cleanup of UL grant Configuration: HANDLE_UM_DATA, USE_BIG_GRANTS sic@ */ // @sic R5s180047 sic@ f_EUTRA_CellInfo_SetPLMN_2Entries (eutra_Cell1, cs_HPLMN_Def, //PLMNid 1 (index 0) cs_HPLMN_002_11, //PLMNid 2 (index 1) -, true, 1); f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_Preamble(eutra_Cell1, STATE2_IDLEUPDATE); f_EUTRA_TestBody_Set(true); // @siclog "Step 1-2 Void" siclog@ // @siclog "Step 3" siclog@ f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12 (eutra_Cell1, 1, false, // voice video sig data CSFB omit, omit, omit, cs_AC_BarringConfig(p00, s512, '11111'B), omit, -, -, true_); f_EUTRA_ModifySysinfo(eutra_Cell1); f_TC_13_5_6x_Step4ToEnd_EUTRA(); }

# Branches executed

The test case was executed on LTE primary band 1 using LTE ciphering eea1 and integrity algorithm eia1.

# Execution Log Files

  - Mediatek MT6897 UE

Mediatek MT6897 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_13_5_6a_LOG.xml

- PICS/PIXIT parameter file:

TC_13_5_6a_PIXIT.txt

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s250185: Supporting information for agreement of TC 13.5.6a |
| --- | --- |