3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250241

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3703 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 10.1.3.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2025-06-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current TTCN implementation, the function v_IMS_DATA_REQ := f_IMS_Register_NonGIBA(IPCAN_E, omit, tsc_IMS_RegisterExpiration, -, true); is called. This sets parameter p_IsEmergency to TRUE, which is not needed for this testcase. This needs to be corrected. Before transmitting the PDU Session Establishment Accept at Step 3, the EPS Bearer Identity needs to be updated. The current TTCN is not doing this which needs to be fixed. For a Multi-PDU scenario, the EPS bearer Identity needs to be assigned in such a way that an earlier identity is not assigned. This also needs to be implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of calling v_IMS_DATA_REQ := f_IMS_Register_NonGIBA(IPCAN_E, omit, tsc_IMS_RegisterExpiration, -, true); called v_IMS_DATA_REQ := f_IMS_Register_NonGIBA(); Implemented the TTCN code for assigning the EPS Bearer Identity before transmitting the PDU Session Establishment Accept. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.3.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1 Overview	3

2 Corrections required	3

2.1 Change 1	3

# 1 Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk24 related to the title of this CR

Contact:		Bhise Parikshit

parikshit.bhise@rohde-schwarz.com

# 2 Corrections required

## 2.1 Change 1

| Function name | f_TC_10_1_3_3_NR5GC_IMS1 |
| --- | --- |
| Reason for change | In the current TTCN implementation, the function v_IMS_DATA_REQ := f_IMS_Register_NonGIBA(IPCAN_E, omit, tsc_IMS_RegisterExpiration, -, true); is called. This sets parameter p_IsEmergency to TRUE, which is not needed for this testcase. This needs to be corrected. |
| Summary of change | Instead of calling v_IMS_DATA_REQ := f_IMS_Register_NonGIBA(IPCAN_E, omit, tsc_IMS_RegisterExpiration, -, true);, called v_IMS_DATA_REQ := f_IMS_Register_NonGIBA(); |
| TTCN module | GSM_NR5GC_IMS.ttcn |
| MCC160 comment | Accepted |

Before Change:

function f_TC_10_1_3_3_NR5GC_IMS1()runs on IMS_PTC { //Network-requested PDU session release / reactivation requested / new P-CSCF address / Initial IMS registration var IMS_DATA_REQ v_IMS_DATA_REQ; f_IMS_PTC_Init(PDN_1); // @sic R5s250189 sic@ f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); // @sic R5s250189 sic@ f_IMS_NewPCSCFaddr(IPCAN); v_IMS_DATA_REQ := f_IMS_Register_NonGIBA(IPCAN_E, omit, tsc_IMS_RegisterExpiration, -, true); f_IMS_Register_SubscribeNotify(v_IMS_DATA_REQ, true); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Steps 4-11"); // Inform NR: IMS registration done f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("IMS registered on new address")); }

After Change:

function f_TC_10_1_3_3_NR5GC_IMS1()runs on IMS_PTC { //Network-requested PDU session release / reactivation requested / new P-CSCF address / Initial IMS registration var IMS_DATA_REQ v_IMS_DATA_REQ; f_IMS_PTC_Init(PDN_1); // @sic R5s250189 sic@ f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); // @sic R5s250189 sic@ f_IMS_NewPCSCFaddr(IPCAN); //WA#WI1427284 WA#10_1_3_3 COMMENTED OUT v_IMS_DATA_REQ := f_IMS_Register_NonGIBA(IPCAN_E, omit, tsc_IMS_RegisterExpiration, -, true); v_IMS_DATA_REQ := f_IMS_Register_NonGIBA(); //WA#WI1427284 WA#10_1_3_3 f_IMS_Register_SubscribeNotify(v_IMS_DATA_REQ, true); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Steps 4-11"); // Inform NR: IMS registration done f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("IMS registered on new address")); } }

## 2.2 Change 2

| Function name | fl_TC_10_1_3_3_Body |
| --- | --- |
| Reason for change | Before transmitting the PDU Session Establishment Accept at Step 3, the EPS Bearer Identity needs to be updated. The current TTCN is not doing this which needs to be fixed. For a Multi-PDU scenario, the EPS bearer Identity needs to be assigned in such a way that an earlier identity is not assigned. This also needs to be implemented. |
| Summary of change | Implemented the TTCN code for assigning the EPS Bearer Identity before transmitting the PDU Session Establishment Accept. Please see below. |
| TTCN module | GSM_NR5GC.ttcn |
| MCC160 comment | Alternative solution implemented – re-used the EPS Bearer Id from the released session information |

Before Change:

function fl_TC_10_1_3_3_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var O1_Type v_PDUSessionIdToRelease; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var PDUSessionInfoList_Type v_PDUSessionInfoList; var NR_SRB_COMMON_IND v_ReceivedAsp; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var integer v_NoOfSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); var template (omit)S_NSSAI_Type v_RequestedNSSAI:= omit; var template (omit) DNN v_RequestedDNN:= omit; var template (omit) ExtdProtocolConfigOptions v_ExtdPCO := omit; var IntegerList_Type v_ListOfDRBs; var NR_DRBInfo_Type v_DRBInfo; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo_IMS; timer t_Wait; // Get GSM info for existing PDU session v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_GSM_MobilityInfo_IMS := f_Get_PDUSessionForDNNType(v_PDUSessionInfoList, IMS_DNN); v_GSM_MobilityInfo := valueof(v_GSM_MobilityInfo_IMS); v_PDUSessionIdToRelease := v_GSM_MobilityInfo.SessionId; v_RequestedDNN := f_Get_DNNForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease); v_RequestedNSSAI:= f_Get_NSSAIForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease); // Get DRB info for existing PDU session v_ListOfDRBs := f_NR_GetDRBList_ForGivenPDUSession(oct2int(v_PDUSessionIdToRelease)); v_DRBInfo := valueof(f_NR_GetDRBInfo_FromId(v_ListOfDRBs[0])); // only 1 DRB in list as haven't modified PDU session //@siclog "Steps 1" siclog@ //The generic test procedure in TS 38.508-1[4] clause 4.9.21 for PDU Session Release is performed with PDU SESSION RELEASE COMMAND message includes 5GSM cause #39 "reactivation requested" to release IMS PDU session. f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00100111'B)); //5GSM cause Reactivation Request // @sic R5s250189 sic@ // Check new PCSCF addresses are unique and send them to IMS v_ExtdPCO := f_NR5GC_GetPCO_NewPCSCFaddr(IMS[tsc_Index_IMS1], v_GSM_MobilityInfo.PCO, PDN_5, PDN_6); if (not isvalue(v_ExtdPCO)) { FatalError(__FILE__, __LINE__, "PCSCF addresses shall be different"); } //@siclog "Steps 2" siclog@ //The UE transmits a PDU SESSION ESTABLISHMENT REQUEST message and the S-NSSAI and DNN in UL NAS TRANSPORT message are the same values as released (Note 1). --> 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST 1 P t_Wait.start (8.0); //@sic R5-252163 sic@ alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { t_Wait.stop; } []t_Wait.timeout { f_UT_RequestPDUSession(UT, v_NoOfSessions +1); SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; } } if (f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, ?, -, -, -, -, -, -, -, -, -, -, -, v_RequestedNSSAI, v_RequestedDNN)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } // Don't use the generic PDU establishment function as need to re-use PDN and DRB info //@siclog "Steps 3" siclog@ //The SS transmits a PDU SESSION ESTABLISHMENT ACCEPT message containing new P-CSCF addresses. f_SendPDUSessionAcceptOverReconfig (v_NGC_NASCellA, v_GSM_MobilityInfo, v_RequestedDNN, cs_38508_RadioBearerConfigDef(omit, {cs_38508_NRDRB_ToAddMod(v_DRBInfo.SDAP_Config, v_DRBInfo.DRB_Id, cs_38508_PDCP_Config)}), f_NR_GetCellgroupConfig_DRB(v_NGC_NASCellA, {v_DRBInfo.DRB_Id}, false), {v_DRBInfo.SS_Config}, -, -, -, -, -, v_RequestedNSSAI, -, -, omit, omit, v_ExtdPCO); //@siclog "Steps 4-11" siclog@ //Check: Does the UE correctly initiate and complete initial IMS registration using the first address from received new P-CSCF addresses? //(Steps 1-8 of Annex A.2 in TS 34.229-5 [41]) f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "IMS registered on new address"); //@siclog "Step 12" siclog@ //SS transmits an RRCRelease message. f_NR_RRCRelease(v_NGC_NASCellA); } //End of test body

After Change:

function fl_TC_10_1_3_3_Body() runs on NR5GC_PTC { // @sic R5s250189 sic@ var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var O1_Type v_PDUSessionIdToRelease; var GSM_MobilityInfo_Type v_GSM_MobilityInfoToRelease; var GSM_MobilityInfo_Type v_GSM_MobilityInfo_New; var PDUSessionInfoList_Type v_PDUSessionInfoList; var NR_SRB_COMMON_IND v_ReceivedAsp; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2; var integer v_NoOfSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); var template (omit)S_NSSAI_Type v_RequestedNSSAI:= omit; var template (omit) DNN v_RequestedDNN:= omit; var template (omit) ExtdProtocolConfigOptions v_ExtdPCO := omit; var IntegerList_Type v_ListOfDRBs; var NR_DRBInfo_Type v_DRBInfo; var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo_IMS; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var default v_MyDefaultVar := null; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; //WA#WI1427284 WA#10_1_3_3 var IntegerList_Type v_ExistingIds; //WA#WI1427284 WA#10_1_3_3 var integer v_NoOfIds := 0; //WA#WI1427284 WA#10_1_3_3 var integer i; //WA#WI1427284 WA#10_1_3_3 timer t_Wait; // Get GSM info for existing PDU session v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_GSM_MobilityInfo_IMS := f_Get_PDUSessionForDNNType(v_PDUSessionInfoList, IMS_DNN); v_GSM_MobilityInfoToRelease := valueof(v_GSM_MobilityInfo_IMS); v_PDUSessionIdToRelease := v_GSM_MobilityInfoToRelease.SessionId; v_RequestedDNN := f_Get_DNNForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease); v_RequestedNSSAI:= f_Get_NSSAIForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease); // Get DRB info for existing PDU session v_ListOfDRBs := f_NR_GetDRBList_ForGivenPDUSession(oct2int(v_PDUSessionIdToRelease)); v_DRBInfo := valueof(f_NR_GetDRBInfo_FromId(v_ListOfDRBs[0])); // only 1 DRB in list as haven't modified PDU session // Don't use the generic functions as need to re-use PDN and DRB info //@siclog "Steps 1" siclog@ //The generic test procedure in TS 38.508-1[4] clause 4.9.21 for PDU Session Release is performed with PDU SESSION RELEASE COMMAND message includes 5GSM cause #39 "reactivation requested" to release IMS PDU session. v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionReleaseCommand(v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00100111'B)); //5GSM cause Reactivation Request // The SS transmits an RRCReconfiguration message with a DRB ToReleaseList and PDU SESSION RELEASE COMMAND v_ReceivedAsp := f_NR_RRCReconfigReleaseDRBs(v_NGC_NASCellA, v_ListOfDRBs, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend)); // @sic R5s220595 sic@ if (not f_Check_NG_PDUSessionReleaseComplete (v_PDUSessionInfoList, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToRelease)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Complete failed"); } // Clear store of msgs received in parallel f_NR5GC_MsgInDefault_ResetStoreOfNASMessageReq(); // Then activate store in case a PDUSessionEstablishmentReq comes before we can process it v_MyDefaultVar := activate(a_NR5GC_AdditionalPDUSessionEstablishmentReq(v_NGC_NASCellA)); // Check new PCSCF addresses are unique and send them to IMS v_ExtdPCO := f_NR5GC_GetPCO_NewPCSCFaddr(IMS[tsc_Index_IMS1], v_GSM_MobilityInfoToRelease.PCO, PDN_5, PDN_6); if (not isvalue(v_ExtdPCO)) { FatalError(__FILE__, __LINE__, "PCSCF addresses shall be different"); } deactivate(v_MyDefaultVar); //@siclog "Steps 2" siclog@ //The UE transmits a PDU SESSION ESTABLISHMENT REQUEST message and the S-NSSAI and DNN in UL NAS TRANSPORT message are the same values as released (Note 1). --> 5GMM: UL NAS TRANSPORT 5GSM: PDU SESSION ESTABLISHMENT REQUEST 1 P if (f_NR5GC_MsgInDefault_GetNumOfNASMessageReq() > 0) { v_ReceivedAsp := f_NR5GC_MsgInDefault_GetOneNASMessageReq(1); } else { t_Wait.start (8.0); //@sic R5-252163 sic@ alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { t_Wait.stop; } []t_Wait.timeout { f_UT_RequestPDUSession(UT, v_NoOfSessions +1); SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; } } } if (f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_New, v_ReceivedAsp.Signalling.Nas[0].Pdu, ?, -, -, -, -, -, -, -, -, -, -, -, v_RequestedNSSAI, v_RequestedDNN)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); } // Update the routing table with the new session id f_IP_ChangeDrbMappingNR(IP, v_NGC_NASCellA, cs_IP_QosFlowId(oct2int(v_GSM_MobilityInfo_New.SessionId), v_DRBInfo.SDAP_Config.mappedQoS_FlowsToAdd[0]), v_GSM_MobilityInfoToRelease.PdnIndex); v_ExistingIds := f_NR5GC_MobileInfo_GetEPS_BearerIdList(); //WA#WI1427284 WA#10_1_3_3 v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo(); //WA#WI1427284 WA#10_1_3_3 if (isvalue(v_GMM_MobilityInfo.GMMCap)) { //WA#WI1427284 WA#10_1_3_3 if (v_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) { v_GSM_MobilityInfo_New.EPS_Bearer := valueof(f_GetNextEPSBearerId (v_GSM_MobilityInfo_New, v_GMM_MobilityInfo)); } } /*This is for MultiPDU case. For example, in case 2 PDU Sessions are established in preamble : 1 for IMS, and 1 for Internet and if IMS PDU Session is released with reactivation requested In such a scenario the IMS PDU session that is established again should not get an EPS Bearer allocated that is already assigned to Internet PDU Session established earlier. */ v_ExistingIds := f_NR5GC_MobileInfo_GetEPS_BearerIdList(); //WA#WI1427284 WA#10_1_3_3 v_NoOfIds := lengthof(v_ExistingIds); //WA#WI1427284 WA#10_1_3_3 for (i:= 0; i < v_NoOfIds; i :=i+1) { //WA#WI1382276 WA#10_1_3_3 if (match (v_ExistingIds[i], v_GSM_MobilityInfo_New.EPS_Bearer)) { v_GSM_MobilityInfo_New.EPS_Bearer := v_GSM_MobilityInfo_New.EPS_Bearer + 1; // allocate the next one break; } } //@siclog "Steps 3" siclog@ //The SS transmits a PDU SESSION ESTABLISHMENT ACCEPT message containing new P-CSCF addresses. f_SendPDUSessionAcceptOverReconfig (v_NGC_NASCellA, v_GSM_MobilityInfo_New, v_RequestedDNN, cs_38508_RadioBearerConfigDef(omit, {cs_38508_NRDRB_ToAddMod(v_DRBInfo.SDAP_Config, v_DRBInfo.DRB_Id, cs_38508_PDCP_Config)}), f_NR_GetCellgroupConfig_DRB(v_NGC_NASCellA, {v_DRBInfo.DRB_Id}, false), {v_DRBInfo.SS_Config}, -, -, -, -, -, v_RequestedNSSAI, -, -, omit, omit, v_ExtdPCO); //@siclog "Steps 4-11" siclog@ //Check: Does the UE correctly initiate and complete initial IMS registration using the first address from received new P-CSCF addresses? //(Steps 1-8 of Annex A.2 in TS 34.229-5 [41]) f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "IMS registered on new address"); //@siclog "Step 12" siclog@ //SS transmits an RRCRelease message. f_NR_RRCRelease(v_NGC_NASCellA); } //End of test body

## 2.3 Additional Change requested by ROHDE & SCHWARZ

| Function name | f_TC_10_1_3_3_NR5GC_IMS1 |
| --- | --- |
| Reason for change | The current TTCN is not handling the IMS deregistration for preamble |
| Summary of change | The function f_IMS_IpcanReleaseWithOptionalDeregistration needs to be called |
| TTCN module | GSM_NR5GC_IMS.ttcn |
| MCC160 comment | Accepted |

After Change:

function f_TC_10_1_2_3_NR5GC_IMS1()runs on IMS_PTC

{ //Network-requested PDU session modification / new P-CSCF address / Initial IMS registration

var IMS_DATA_REQ v_IMS_DATA_REQ;

f_IMS_PTC_Init(); // @sic R5s250098 sic@

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

f_IMS_Registration();

f_IMS_IPCAN_SendCoOrdMsg(IPCAN);

// @sic R5s250098 sic@

f_IMS_NewPCSCFaddr(IPCAN);

//WA#10_1_2_3 WA#WI1426599 v_IMS_DATA_REQ := f_IMS_Register_NonGIBA(IPCAN_E, omit, tsc_IMS_RegisterExpiration, -, true);

v_IMS_DATA_REQ := f_IMS_Register_NonGIBA(); //WA#10_1_2_3 WA#WI1426599

f_IMS_Register_SubscribeNotify(v_IMS_DATA_REQ, true);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 10");

// Inform NR: IMS registration done

f_IMS_IPCAN_SendCoOrdMsg(IPCAN, cms_IMS_IPCAN_Trigger("IMS registered on new address"));

f_IMS_IpcanReleaseWithOptionalDeregistration(); //WA#PCSCF

}

## 2.4 Additional Change

| Function name | f_GetNewPCOPCSCFaddr_Common |
| --- | --- |
| Reason for change | Only one new IPv4 address is included in the PDU Session Establishment Accept message instead of 2 |
| Summary of change | Increment the count Please see below. |
| TTCN module | NAS_AuxiliaryDefsAndFunctions.ttcn |

function f_GetNewPCOPCSCFaddr_Common(ProtocolContainerList p_PCOListFromUE,

CharStringList_Type p_AddrList_2,

CharStringList_Type p_AddrList_3 := {}) return template (value) ProtocolContainerList

{

var template (value) ProtocolContainerList v_PCOList;

var octetstring v_Contents;

var integer i;

var integer v_Count := 0;

for (i := 0; i < lengthof(p_PCOListFromUE); i := i +1) {

if (p_PCOListFromUE[i].protocolID == '0001'O) { // IPv6

v_Contents := f_Convert_IPv6Addr2OctString(p_AddrList_2[1]);

v_PCOList[v_Count] := cs_ProtocolContainer('0001'O, v_Contents);

v_Count := v_Count + 1;

if (lengthof(p_AddrList_3) > 0) {

v_Contents := f_Convert_IPv6Addr2OctString(p_AddrList_3[1]);

v_PCOList[v_Count] := cs_ProtocolContainer('0001'O, v_Contents);

v_Count := v_Count + 1;

}

} else if(p_PCOListFromUE[i].protocolID == '000C'O) { // IPv4

v_Contents := f_Convert_IPv4Addr2OctString(p_AddrList_2[0]);

v_PCOList[v_Count] := cs_ProtocolContainer('000C'O, v_Contents);

v_Count := v_Count + 1;

if (lengthof(p_AddrList_3) > 0) {

v_Contents := f_Convert_IPv4Addr2OctString(p_AddrList_3[0]);

v_PCOList[v_Count] := cs_ProtocolContainer('000C'O, v_Contents);

v_Count := v_Count + 1;

}

}

}

return v_PCOList;

}

#### MCC160 Implementation

function fl_TC_10_1_3_3_Body() runs on NR5GC_PTC

{ // @sic R5s250189 sic@

var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN);

var O1_Type v_PDUSessionIdToRelease;

var GSM_MobilityInfo_Type v_GSM_MobilityInfoToRelease;

var GSM_MobilityInfo_Type v_GSM_MobilityInfo_New;

var PDUSessionInfoList_Type v_PDUSessionInfoList;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB2;

var integer v_NoOfSessions := f_NR5GC_MobileInfo_GetNoOfSessions();

var template (omit)S_NSSAI_Type v_RequestedNSSAI:= omit;

var template (omit) DNN v_RequestedDNN:= omit;

var template (omit) ExtdProtocolConfigOptions v_ExtdPCO := omit;

var IntegerList_Type v_ListOfDRBs;

var NR_DRBInfo_Type v_DRBInfo;

var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo_IMS;

var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend;

var default v_MyDefaultVar := null;

timer t_Wait;

// Get GSM info for existing PDU session

v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

v_GSM_MobilityInfo_IMS := f_Get_PDUSessionForDNNType(v_PDUSessionInfoList, IMS_DNN);

v_GSM_MobilityInfoToRelease := valueof(v_GSM_MobilityInfo_IMS);

v_PDUSessionIdToRelease := v_GSM_MobilityInfoToRelease.SessionId;

v_RequestedDNN := f_Get_DNNForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease);

v_RequestedNSSAI:= f_Get_NSSAIForPDUSessionId(v_PDUSessionInfoList, v_PDUSessionIdToRelease);

// Get DRB info for existing PDU session

v_ListOfDRBs := f_NR_GetDRBList_ForGivenPDUSession(oct2int(v_PDUSessionIdToRelease));

v_DRBInfo := valueof(f_NR_GetDRBInfo_FromId(v_ListOfDRBs[0])); // only 1 DRB in list as haven't modified PDU session

// Don't use the generic functions as need to re-use PDN and DRB info

//@siclog "Steps 1" siclog@

//The generic test procedure in TS 38.508-1[4] clause 4.9.21 for PDU Session Release is performed with PDU SESSION RELEASE COMMAND message includes 5GSM cause #39 "reactivation requested" to release IMS PDU session.

v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionReleaseCommand(v_PDUSessionIdToRelease,

cs_GMM_GSM_Cause (omit, '00100111'B)); //5GSM cause Reactivation Request

.

.

.

// Update the routing table with the new session id

f_IP_ChangeDrbMappingNR(IP, v_NGC_NASCellA, cs_IP_QosFlowId(oct2int(v_GSM_MobilityInfo_New.SessionId), v_DRBInfo.SDAP_Config.mappedQoS_FlowsToAdd[0]), v_GSM_MobilityInfoToRelease.PdnIndex);

// re-use the existing EPS Bearer Id

v_GSM_MobilityInfo_New.EPS_Bearer := v_GSM_MobilityInfoToRelease.EPS_Bearer;

//@siclog "Steps 3" siclog@

//The SS transmits a PDU SESSION ESTABLISHMENT ACCEPT message containing new P-CSCF addresses.

f_SendPDUSessionAcceptOverReconfig (v_NGC_NASCellA,

v_GSM_MobilityInfo_New,

v_RequestedDNN,

cs_38508_RadioBearerConfigDef(omit, {cs_38508_NRDRB_ToAddMod(v_DRBInfo.SDAP_Config, v_DRBInfo.DRB_Id, cs_38508_PDCP_Config)}),

f_NR_GetCellgroupConfig_DRB(v_NGC_NASCellA, {v_DRBInfo.DRB_Id}, false),

{v_DRBInfo.SS_Config},

-, -, -, -, -,

v_RequestedNSSAI,

-, -,

omit, omit, v_ExtdPCO);

//@siclog "Steps 4-11" siclog@

//Check: Does the UE correctly initiate and complete initial IMS registration using the first address from received new P-CSCF addresses?

//(Steps 1-8 of Annex A.2 in TS 34.229-5 [41])

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1], "IMS registered on new address");

//@siclog "Step 12" siclog@

//SS transmits an RRCRelease message.

f_NR_RRCRelease(v_NGC_NASCellA);

}