3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250499

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3780 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on NR NTN test case 6.7.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | UE which has PICs setting with pc_noOf_PDUsSameConnection=1, will fail this case at step 7-19, as UE will request for PDU sessin establishment request. #draft proseCR would be raised during RAN5#110 meeting |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add f_NR5GC_RRC_Idle_Steps16_20 after f_NR5GC_RRC_Idle_Steps5_15 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail abnormaly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required to testcase 6.7.1.3	4

2.1	Change 1	5

5	References	10

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 6.7.1.2 which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk38.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Narendra Kalahasti

Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TC_6_7_1_3_NR5GC_TestBody () |
| --- | --- |
| Reason for change | UE which has PICs setting with pc_noOf_PDUsSameConnection=1, will fail this case at step 7-19, as UE will request for PDU sessin establishment request. #draft proseCR would be raised during RAN5#110 meeting |
| Summary of change | Add f_NR5GC_RRC_Idle_Steps16_20 after f_NR5GC_RRC_Idle_Steps5_15 |
| TTCN module | NTN_Idle_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#110. See implementation below. |

Before Change:

function fl_TC_6_7_1_3_NR5GC_TestBody() runs on NR5GC_PTC { var GMM_MobilityInfo_Type v_GMM_MobilityInfo; ---- skip code----- // @siclog "Steps 7-19" siclog@ // Steps 3 to 15 of the registration procedure described in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 1. f_NR_RRCSetup_Def (nr_Cell1); // send RRC Connection Setup on SRB0 (CCCH) and switch on UL grant assignments v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete( tsc_NR_RRC_TI_Def), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_15(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, v_PLMN, TESTMode_OFF); //added here // @siclog "Step 20" siclog@ // The SS transmits an RRCRelease message to move to RRC_IDLE state. f_NR_RRCRelease(nr_Cell1); ----skip code--- } // end of fl_TC_6_7_1_3_NR5GC_TestBody

After Change:

function fl_TC_6_7_1_3_NR5GC_TestBody() runs on NR5GC_PTC { var GMM_MobilityInfo_Type v_GMM_MobilityInfo; ---- skip code----- // @siclog "Steps 7-19" siclog@ // Steps 3 to 15 of the registration procedure described in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 1. f_NR_RRCSetup_Def (nr_Cell1); // send RRC Connection Setup on SRB0 (CCCH) and switch on UL grant assignments v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete( tsc_NR_RRC_TI_Def), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_RRC_Idle_Steps5_15(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, v_PLMN, TESTMode_OFF); f_NR5GC_RRC_Idle_Steps16_20(nr_Cell1, -, -, noRrcConnectionRelease); // @siclog "Step 20" siclog@ // The SS transmits an RRCRelease message to move to RRC_IDLE state. f_NR_RRCRelease(nr_Cell1); ----skip code--- } // end of fl_TC_6_7_1_3_NR5GC_TestBody

MCC160 Implementation:

function fl_TC_6_7_1_3_NR5GC_TestBody() runs on NR5GC_PTC { var GMM_MobilityInfo_Type v_GMM_MobilityInfo; ---- skip code----- // @siclog "Steps 7-20Da1" siclog@ // Steps 3 to 20a1 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 1. f_NR5GC_RRC_Idle_Steps3_20(nr_Cell1,-,noRrcConnectionRelease); ----skip code--- } // end of fl_TC_6_7_1_3_NR5GC_TestBody