3GPP TSG SA WG2 Meeting #160	S2-2313413

# 13 - 17 November, 2023, Chicago, USA		(revision of S2-2312398)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0987 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Location accuracy analytics. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI, TOYOTA MOTOR CORPORATION |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to clause 6.17.1, Location Accuracy Analytics provides the confidence of location accuracy prediction. However, there is no definition for the confidence IE in Output Analytics. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to - Add the confidence IE to the Location Accuracy predictions table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.17.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Changes ***

6.17.3	Output Analytics

The Location Accuracy analytics are shown in table 6.17.3-1.

Table 6.17.3-1: Location Accuracy statistics

| Information | Description |
| --- | --- |
| List of Accuracy sustainability Analytics (1..max) |  |
| >Applicable Area | A list of TAIs or Cell IDs or below cell level location information that the analytics applies to |
| >Applicable Time Period | The time period within the Analytics target period that the statistics applies to. |
| > Positioning Methods (1…max) (NOTE) | The positioning methods used to measure location as defined in clause 4.2b of TS 23.273 [39] |
| >> average accuracy of positioning method in applicable area (NOTE) | Statistics about the horizontal and optionally vertical location average accuracy of applicable positioning method at the applicable area |
| >> Percentage of NLOS/LOS measurements of positioning method in applicable area | Provides ratio of LOS to NLOS measurements of positioning method in the applicable area |
| > Percentage of UE in indoor/outdoor in applicable area (optional) (NOTE) | Provides ratio of UEs that are indoor/outdoor in the applicable area |
| > Applicable Location | A location (i.e. Geographical location or location in local coordinate (as defined in TS 23.032 [34])) that the analytics applies to |
| >Applicable Time Period | The time period within the Analytics target period that the prediction applies to. |
| > Positioning Methods (1…max) (NOTE) | The positioning methods used to measure location as defined in clause 4.2b of TS 23.273 [39] |
| >>accuracy of positioning method in applicable location (NOTE) | statistics about the horizontal and optionally vertical location accuracy of applicable positioning method at the applicable location |
| >>Indication whether the location is measured NLOS/LOS with the positioning method | Indicates whether this applicable location is measured with NLOS or LOS with the positioning method |
| > Indication whether location is indoor/outdoor (optional) (NOTE) | Indicates whether this applicable location is measured to be indoor or outdoor |
| NOTE: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". |  |

Table 6.17.3-2: Location Accuracy predictions

| Information | Description |
| --- | --- |
| List of Accuracy sustainability Analytics (1..max) |  |
| >Applicable Area | A list of TAIs or Cell IDs or below cell level location information that the analytics applies to |
| >Applicable Time Period | The time period within the Analytics target period that the prediction applies to |
| > Positioning Methods (1…max) (NOTE) | The positioning methods used to measure location as defined in clause 4.2b of TS 23.273 [39] |
| >> average accuracy of positioning method in applicable area (NOTE) | Prediction about the horizontal and optionally vertical location average accuracy of applicable positioning method at the applicable area |
| >> Percentage of NLOS/LOS measurements of positioning method in applicable area | Provides predicted ratio of LOS to NLOS measurements of positioning method in the applicable area |
| > Percentage of UE in indoor/outdoor in applicable area (optional) (NOTE) | Provides predicted ratio of UEs that are indoor/outdoor in the applicable area |
| > Applicable Location | A location (i.e. Geographical location or location in local coordinate (as defined in TS 23.032 [34])) that the analytics applies to |
| >Applicable Time Period | The time period within the Analytics target period that the prediction applies to. |
| > Positioning Methods (1…max) (NOTE) | The positioning methods used to measure location as defined in clause 4.2b of TS 23.273 [39] |
| >>accuracy of positioning method in applicable location (NOTE) | Prediction about the horizontal and optionally vertical location accuracy of applicable positioning method at the applicable location |
| >>Indication whether the location is measured NLOS/LOS with the positioning method | Predicts whether this applicable location is measured with NLOS or LOS with the positioning method |
| > Indication whether location is indoor/outdoor (optional) (NOTE) | Predicts whether this applicable location is measured to be indoor or outdoor |
| > Confidence | Confidence of the prediction. |
| NOTE: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". |  |

# *** End of Changes ***