3GPP TSG-SA WG6 Meeting #57	S6-233280

Xiamen, China 9th – 13th October 2023	(revision of S6-232377,S6-233062, S6-233240)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0019 | rev | 3 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve issues related to PIN modification after local PEMC failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital, Vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The PIN Modification procedure (clause 8.5.10.2.2) has several issues. Observation 1 - in step 1, there is no subscription/notification procedures defined for the PEGC to send a notification for PEMC failure. Observation 2 - detection of PEMC failure may also happen via the Hearbeat procedure, or may be detected directly via a communication timeout. Observation 3 - in step 6, the procedure is implemented by the PIN status notification procedure but is not indicated as such. Observation 4 - steps 1, 2, 3 and 7 should be optional dependent on the PIN server detecting PEMC failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - PIN modification after local PEMC failure is updated - The EN of clause 8.5.10.2.2 is resolved - PEGC PIN connectivity subscription/notification is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PIN modification after local PEMC failure cannot be realized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.10.2.2, 8.5.14 (new), 8.5.14.1 (new), 8.5.14.2 (new), 8.5.14.2.1 (new), 8.5.14.2.2 (new), 8.5.14.2.3 (new), 8.5.14.2.4 (new), 8.5.14.3 (new), 8.5.14.3.1 (new), 8.5.14.3.2 (new), 8.5.14.3.3 (new), 8.5.14.3.4 (new), 8.5.14.3.5 (new), 8.5.14.3.6 (new), 8.5.14.3.7 (new), 8.5.14.3.8 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.5.10.2.2	PIN modification after local PEMC failure

Figure 8.5.10.2.2-1 describes the PIN modification procedure to perform a PEMC role change due to the failure of the PEMC. An authorised administrator is the owner of the PIN and accesses PIN configuration using an application on a UE, which is one of the PEMC for the PIN. The authorised administrator can manage the PIN locally or through the 5G network. This procedure describes a PEMC (e.g., an authorized administrator on a UE) managing the PIN remotely via the 5G network.

The procedure may be used e.g. when a PEMC is available on a UE for PIN management by an authorised administrator. When there is a local PEMC failure, the authorized administrator can be enabled to manage the PIN remotely, via the 5G network, using the following steps.

Pre-conditions:

1.	The PIN server has authorized the creation of the PIN.

2.	The authorized administrator is the owner of the PIN and has created the PIN.

3.	The authorized administrator configures the active PEMC to provide PIN management for the PIN. UE/PEMC1 is the inactive PEMC.

4.	The active PEMC, PEGC, PINE-1, PINE-2, and the UE/PEMC1 are members of the PIN. PINE-1 has PEMC capability.

5.  The authorized administrator leaves the local area of the PIN (e.g., in a home) and is able to access the PIN remotely through the 5G network. As a result, the authorized administrator is able to manage the PIN through the 5G network.

6.	The UE/PEMC1 is subscribed to the PEGC for PIN connectivity notifications

Figure 8.5.10.2.2-1: PIN Modification after local PEMC failure

1.	The UE/PEMC1 may detect that PEMC2 is unavailable or failed. In step 1a, the UE/PEMC1 may receive a PIN connectivity notification from PEGC indicating that there is a communication failure with PEMC2. Otherwise, in step 1b, the UE/PEMC1 may detect that PEMC2 is unavailable by using the PIN heartbeat mechanism or if a communication timeout happens with PEMC2.

2.	If UE/PEMC1 has detected that PEMC2 is unavailable or failed in step 1, an authorized administrator on UE/PEMC1 sends a PIN configuration request to the PIN server through the 5G network. The request includes the security credentials of the authorized administrator, the UE ID, the PIN ID, PIN member ID, authorization type indicating the role change (e.g., the request may indicate that PINE-1 be assigned the new PEMC).

3.	If the PIN Server has received a PIN Configuration request in step 2, the PIN server processes the modification request and checks if the authorized administrator is allowed to modify the PIN. The PIN server verifies that PINE-1 has the capability to serve as a PEMC using information in the PIN profile.

4. 	If the authorized administrator is allowed to perform PIN modification or if the PIN Server has detected that PEMC2 is unavailable or failed and has determined that PINE-1 can be reassigned as the new PEMC, the PIN server sends a PIN management request to PINE-1 to assign PINE-1 as the new PEMC and provides PIN profile information to PINE-1.

5.	PINE-1 processes the PIN management request indicating the PEMC assignment and returns a PIN management response to the PIN server indicating if the assignment was successful.

6.	If the PIN management response in step 5 was successful, the PIN server notifies the other members of the PIN that PINE-1 will be the new PEMC for the PIN and updates PIN profile information.

7.	If the PIN Server has received a PIN Configuration request in step 2, the PIN server sends a PIN configuration response to the UE/PEMC1, the response includes the updated PIN profile information if the PEGC assignment was successful; otherwise, the response indicates that the assignment failed..

8.	If the PEGC assignment was successful, the PIN communications resume with PINE-1 serving as the new PEMC.

* * * Next Change * * * *

### 8.5.14	PIN connectivity subscription

#### 8.5.14.1	General

The PIN connectivity subscription is used by the PINE/PEMC/PIN Server to be notified of PIN connectivity events by the PEGC.

PIN connectivity notification includes PIN connectivity information.

#### 8.5.14.2	Procedure

##### 8.5.14.2.1	PIN connectivity subscribe

Figure 8.5.14.2.1-1 illustrates the PIN connectivity subscribe procedure.

Pre-conditions:

1.	The PINE/PEMC/PIN Server (e.g., subscriber) has already received the address of PEGC;

2.	The subscriber has been authorized to communicate with PEGC;

Figure 8.5.14.2.1-1: PIN connectivity subscribe

1.	The subscriber sends the PIN connectivity subscribe request to the PEGC. The PIN connectivity subscribe request includes the subscriber identifier along with the security credentials and the event identifier.

2.	Upon receiving the request from the subscriber, the PEGC checks if the subscriber is authorized to subscribe for PIN connectivity information. The authorization check may apply to an individual PIN. If the request is authorized, the PEGC creates and stores the subscription for PIN.

3.	If the processing of the request is successful, the PEGC sends a PIN connectivity subscribe response to the subscriber, which includes the subscription identifier and may include the expiration time, indicating when the subscription will automatically expire. To maintain the subscription, the subscriber shall send a PIN connectivity update request prior to the expiration time. If a new PIN connectivity update request is not received prior to the expiration time, the PEGC shall treat the subscriber as implicitly unsubscribed.

##### 8.5.14.2.2	PIN connectivity notify

Figure 8.5.14.2.2-1 illustrates the PIN connectivity notify procedure.

Pre-conditions:

1.	The PINE/PEMC/PIN Server (e.g., subscriber) has successfully subscribed with the PEGC for the PIN connectivity as specified in clause 8.5.14.2.1.

Figure 8.5.14.2.2-1: PIN connectivity notify procedure

1.	When an event occurs at the PEGC that satisfies triggering conditions, the PEGC notifies the subscriber(s).

2.	The PEGC sends an PIN connectivity notification to the subscribers related to the connectivity event and includes the PIN connectivity information.

The PIN connectivity information includes the PEGC identifier, the PIN identifier, the PIN client identifier, and the event type (e.g., connectivity).

3.	Upon receiving the notification, the subscriber processes the connectivity changes according to the information in the PIN connectivity notification.

##### 8.5.14.2.3	PIN connectivity update

Figure 8.5.14.2.3-1 illustrates the PIN connectivity update procedure.

Pre-conditions:

1.	The PINE/PEMC/PIN Server (e.g., subscriber) has subscribed with the PEGC for PIN connectivity information as specified in 8.5.14.2.1;

Figure 8.5.14.2.3-1: PIN connectivity update

1.	The subscriber sends a PIN connectivity update request to the PEGC. The PIN connectivity update request includes the security credentials and the subscription identifier. It may also include notification target address and proposed expiration time.

2.	Upon receiving the request from the requestor, the PEGC checks if the subscriber is authorized to update the subscription information. If the request is authorized, the PEGC updates the stored subscription for PIN connectivity information.

3.	If the processing of the request is successful, the PEGC sends a PIN connectivity update response to the subscriber, which may include the expiration time, indicating when the subscription will automatically expire. To maintain the subscription, the subscriber shall send an PIN connectivity update request prior to the expiration time. If a PIN connectivity update request is not received prior to the expiration time, the PEGC shall treat the subscriber as implicitly unsubscribed.

##### 8.5.14.2.4	PIN connectivity unsubscribe

Figure 8.5.14.2.4-1 illustrates the PIN connectivity unsubscribe procedure.

Pre-conditions:

1.	The PINE/PEMC/PIN Server (e.g., subscriber) has subscribed with the PEGC for PIN connectivity information as specified in 8.5.14.2.1;

Figure 8.5.14.2.4-1: PIN connectivity update

1.	The subscriber sends a PIN connectivity unsubscribe request to the PEGC. The PIN connectivity unsubscribe request includes the security credentials and the subscription identifier.

2.	Upon receiving the request from the subscriber, the PEGC checks if the requestor is authorized to unsubscribe. If the request is authorized, the PEGC cancels the subscription for PIN connectivity information.

3.	If the processing of the request is successful, the PEGC sends a PIN connectivity unsubscribe response to the subscriber.

#### 8.5.14.3	Information flows

##### 8.5.14.3.1	General

The following information flows are specified for PIN connectivity subscription:

-	PIN connectivity subscribe

-	PIN connectivity notify

-	PIN connectivity update

-	PIN connectivity unsubscribe

##### 8.5.14.3.2	PIN connectivity subscribe request

Table 8.5.14.3.2-1: PIN connectivity subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Subscriber identifier | M | Unique identifier of the PINE/PEMC/PIN server. |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| PIN ID | M | The identifier of PIN. |
| Notification Target Address | M | The Notification target address (e.g. URL) where the notifications destined for the subscriber should be sent to. |
| Proposed expiration time | O | Proposed expiration time for the subscription |

##### 8.5.14.3.3	PIN connectivity subscribe response

Table 8.5.14.3.3-1: PIN connectivity subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of subscription request failure |

##### 8.5.14.3.4	PIN connectivity notify

Table 8.5.14.3.4-1 describes information elements in the PIN connectivity notification from the PEGC.

Table 8.5.14.3.4-1: PIN connectivity notification

| Information element | Status | Description |
| --- | --- | --- |
| PEGC identifier | M | Identifier of the PEGC |
| PIN identifier | M | Identifier of the PIN. |
| PIN client identifier | M | Unique identifier of the PINE/PEMC/PIN server related to the connectivity change |
| Event type | M | Type of event (e.g., connectivity) |

##### 8.5.14.3.5	PIN connectivity update request

Table 8.5.14.3.5-1: PIN connectivity update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| Notification Target Address | O | The Notification target address (e.g. URL) where the notifications destined for the subscriber should be sent to. |
| Proposed expiration time | O | Proposed expiration time for the subscription |

##### 8.5.14.3.6	PIN connectivity update response

Table 8.5.14.3.6-1: PIN connectivity update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (see NOTE) | O | Indicates that the subscription update request was successful. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response (see NOTE) | O | Indicates that the subscription update request failed. |
| > Cause | M | Indicates the cause of subscription update request failure |
| NOTE: One IE is included in the response. |  |  |

##### 8.5.14.3.7	PIN connectivity unsubscribe request

Table 8.5.14.3.7-1: PIN connectivity unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |

##### 8.5.14.3.8	PIN connectivity unsubscribe response

Table 8.5.14.3.8-1: PIN connectivity unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (see NOTE) | O | Indicates that the unsubscribe request was successful. |
| Failure response (see NOTE) | O | Indicates that the unsubscribe request failed. |
| > Cause | M | Indicates the cause of unsubscribe request failure |
| NOTE: One IE is included in the response. |  |  |

* * * End of Change(s) * * * *