SA WG6 Meeting #58	S6-233966

Chicago, USA, 13th – 17th November 2023	(revision of S6-233459)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0021 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of PIN profile and dynamic PIN profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For now, some of the parameters in PIN profile and dynamic PIN profile are the same. And also, it seems unclear that where is the difference between PIN profile and dynamic PIN profile. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification of the details of PIN profile and dynamic PIN profile. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear where is the difference between PIN profile and dynamic PIN profile. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.0(new), 8.2.2.1, 8.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1, all new text * * *

#### 8.2.2.0	PIN Profile in a PIN

PIN profile information include static and semi-static data and default configuration that are needed for configuration of Personal IoT Networks, such as the PEMC, PEGC and PIN server end point addresses. Information in a PIN profile is infrequently changed.

Dynamic PIN profile information includes data that are updated more frequently due to operations of a PIN, e.g. caused by PINE join or PINE leave operations. Example information include PINE lists and the services that PINEs provide.

* * * Start of Change 2 * * *

#### 8.2.2.1	PIN Profile in a PIN

PIN profile includes information about the static data needed for configuration for the Personal IoT networks.

Table 8.2.2.1-1: PIN Profile

| Parameter Name | Parameter Description | PIN Server | PEMC | PEGC |
| --- | --- | --- | --- | --- |
| PIN ID | The identifier of the PIN | Y | Y | Y |
| PIN Description | Human-readable description of the PIN, for example, the company name, location or the type of service. | Y | Y | Y |
| Duration | Specifies the time period of how long the PIN can be active | Y | Y | Y |
| Maximum number of PIN elements | Maximum number of PINEs allowed to join the PIN | Y | Y | N |
| Allowed PIN service | List of service that can be offered within a PIN: > PIN service Provider Identifier > PIN service type > PIN service Feature | Y | Y | N |
| Allowed PEMC list | The list of PINE static information for PINE(s) that can be allowed to take the role of PEMC: > PINE ID > GPSI > Role (e.g., primary, secondary or both) > Port number (see NOTE) | Y | Y | Y |
| Allowed PEGC list | The list of PINE static information for PINE(s) that can be allowed to take the role as PEGC: > PINE identifier > GPSI > Role (e.g., primary, backup or both) > Port number (see NOTE) | Y | Y | Y |
| PIN Server ID | The identifier of the PIN server that serves the PIN | N | Y | Y |
| PIN server Endpoint | The endpoint information (e.g., URI, FQDN) or a static IP address used to communicate with the PIN server. | N | Y | Y |
| Allowed PIN elements list | List of PINEs which can be allowed to join the PIN > PINE ID | Y | Y | Y |

Editor's Note: The NAT related issues if any in PIN is FFS.

* * * Start of Change 3 * * *

#### 8.2.2.2	Dynamic profile information of a PIN

Dynamic profile information of a PIN contains the PIN dynamic data needed for management of the Personal IoT networks.

Table 8.2.2.2-1 describes the list of parameters that are classified as dynamic profile information and which are maintained at the PIN server, PEMC and PEGC. PIN dynamic profile information maintained at these entities are updated based on the events occurring in the PIN. Below are some of the events (not exhaustive):

-	PINE joins or leaves the PIN;

-	Role of PEMC or PEGC changes;

-	When the services offered by the PIN changes;

-	When a PINE updates the services it offers;

-	When a PINE joins or leaves the PIN;

Table 8.2.2.2-1: Dynamic profile information of a PIN

| Parameter Name | Parameter Description | PIN Server | PEMC | PEGC |
| --- | --- | --- | --- | --- |
| PIN ID | The identifier of the PIN | Y | Y | Y |
| Current PIN services | List of services that are currently offered within a PIN | Y | Y | N |
| PIN state | Indicates the current state of the PIN (activated or de-activated). When the PIN is in deactivated state services offered by the PIN are inaccessible and no PIN elements can join the PIN. Also PEGC closes all the communication channel it has created for the flow of application traffic from PIN elements via 5GS to the application server | Y | Y | Y |
| Current PEMC list | The list of PINE dynamic information for PINE(s) that currently have the role of PEMC: > PINE identifier > Role (e.g., primary or secondary) > PEMC current IP address > Duration or time period allowed as PEMC > Heartbeat timer value (see NOTE 2). | Y | Y | Y |
| Current PEGC list | The list of PINE dynamic information for PINE(s) that currently have the role of PEGC: > PINE identifier > Role (e.g., primary or backup) > PEGC current IP address > Duration or time period allowed as PEGC > Maximum number of served PINE(s) > List of currently served PINE(s) information > Heartbeat timer value (see NOTE 3). | Y | Y | Y |
| PIN Elements List | List of PIN elements currently registered/joined the PIN and their details |  |  |  |
|  | > PINE ID | Y | Y | Y |
|  | > Endpoint information of each PINE (e.g. URI, FQDN, IP address, port number) used to communicate with the PINE (NOTE 1). | Y | Y | Y |
|  | > Services offered by the PIN element | Y | Y | N |
|  | > Reachability information of the PIN element NOTE: Reachability information of the PIN element may include the IP address which is internally routable via PEGC (like LAN or Layer 2 gateway) or routable via 5GS. | Y | Y | Y |
|  | > List of application clients for this PIN element: | Y | Y | Y |
|  | >> Minimum KPIs required by each application client to operate effectively within the PIN (e.g., PIN bandwidth, PIN request rate, PIN response time) | Y | Y | Y |
|  | >> Operational schedules of each application client (e.g., time windows) | Y | Y | Y |
|  | > Identifier of the default PEGC authorized to service this PIN element. The PIN element will use this PEGC as the primary PEGC to relay PIN communications. Location and/or schedule information for the default PEGC may also be included such that the default PEGC may be selected by the PIN element based on its current location and proximity to the default PEGC and/or the availability schedule of the default PEGC. | Y | Y | Y |
|  | > Identifiers of backup PEGCs authorized to service this PIN element. The list is in prioritized order (the first PEGC listed will serve as the first backup PEGC). If the default PEGC is not available, the PIN element will use this prioritized list of PEGCs to relay PIN communications. Location and/or schedule information for each of the backup PEGCs may also be included such that a backup PEGC may be selected by the PIN element based on its current location and proximity to a backup PEGC and/or the availability schedule of the PEGC. | Y | Y | Y |
|  | > Heartbeat timer value defining the periodicity of heartbeat requests this PIN element sends to the PEMC to indicate this PIN element is still available within the PIN. | Y | Y | Y |
| NOTE 1: The port number is the port used by a PINE to expose a service within the PIN. NOTE 2: The heartbeat timer value defining the periodicity of heartbeat requests that the PEMC sends to the PIN server to indicate the PEMC is still available and serving as a PEMC. NOTE 3: The heartbeat timer value defining the periodicity of heartbeat requests that the PEGC sends to the PEMC to indicate the PEGC is still available and serving as a PEGC. |  |  |  |  |

* * * End of Change * * *