3GPP TSG-SA6 Meeting #58	S6-233969

Chicago, USA, November 13 – 17, 2023	(revision of S6-233462)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0032 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PIN-Removal-Behaviour |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V., vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | After a PINE is removed from a PIN, a PIN status notify is sent to the PEGC and PIN elements acting as a relay, as well as the PIN elements that have subscribed to notifications. However, some PIN elements might not be subscribed to those notifications and thus might attempt to send data to a PINE that is not part of the PIN anymore. In that case, there is no specification for the behaviour of the PEGC and PIN elements with relay capabilities when receiving messages that are directed to a PINE that has been removed from the PIN. In Rev 2: Due to the relay capability in PIN element is not introduced in current specification, so only the PEGC keeps this relay capability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating clauses 8.5.8.2.2, 8.5.8.2.3 and 8.5.8.2.5 to specify the behaviour of the PEGC and PIN elements with relay capabilities in case a PINE attempts to communicate with a removed PINE. In Rev 2, only keep the relay capability in PEGC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is unclear and ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.8.2.2, 8.5.8.2.3, 8.5.8.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.5.8.2.2	Procedure of PIN elements decides to leave the PIN

The following procedure defines the PIN elements decides to leave the PIN.

Pre-conditions:

1.	The PIN client has already been added into a PIN;

2.	The UE Identifier or PIN client Identifier is available;

3.	The PIN client has been authorized to communicate with the PEMC and already has the application layer connection with the PEMC;

Figure 8.5.8.2.2-1: PIN client decides to leave a PIN

1.	The PINE decides to leave a PIN, and sends the PIN Management PINE leave from PIN request to PEMC to leave the PIN. The request includes the security credentials of the UE or PIN client received during authorization procedure and may include the UE identifier such as GPSI, PIN client ID, UE location and PIN ID.

Editor's note: The authorization procedure between PIN client and PEMC should be captured in SA3 scope.

2.	The PEMC authorizes the request, and decides to remove a PIN client from a PIN which indicated by PIN client ID or UE GPSI.

3.	The PEMC sends the PIN Management PINE leave from PIN response to PINE to notify that the PIN client is not the member of the PIN anymore.

4-5.	The PEMC sends the PIN status notify Request to the PEGC/PIN server containing the details of the PIN client that requested to leave the PIN. The details of the PINE include PIN client ID, GPSI and etc.

6-8.	The PEMC/PEGC/PIN server updates the dynamic profile information of the PIN to remove the details of the PIN client that requested to leave the PIN. The PEGC disables the access control information for this PINE. The PEGC stops relaying traffic to the PINE.

* * * Next Change * * * *

8.5.8.2.3	Procedure of PEMC removes the PIN elements from a PIN

The following procedure defines the PIN elements decides to leave the PIN.

Pre-conditions:

1.	The PIN client has already been added into a PIN;

2.	The UE Identifier or PIN client Identifier is available;

3.	The PIN client has been authorized to communicate with the PEMC and already has the application layer connection with PEMC;

Figure 8.5.8.2.3-1: Remove a PIN element from a PIN by PEMC

1.	The PEMC decides to remove the PINE (identified by GPSI, PIN client ID).

2.	The PEMC sends the PIN status notify to the PINE that this PINE has been removed from the PIN.

3-4.	The PEMC sends the PIN status notify to the PEGC and PIN server containing the details of the PIN client that is removed from the PIN. The details of the PINE include PIN client ID, GPSI and etc.

5-7.	The PEMC/PEGC/PIN server updates the dynamic profile information of the PIN to remove the details of the PIN client that is removed from the PIN. The PEGC disables the access control information for this PINE. The PEGC stops relaying traffic to the PINE.

* * * Next Change * * * *

8.5.8.2.5	PINE leave via PEGC

The PINE leaves the PIN via the PEGC is depicted in figure 8.5.8.2.5-1.

Pre-conditions:

1.	The PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PEMC or PIN server;

2.	The PINE already establishes the connection with PEGC;

3.	The PEGC acts as the application layer relay as defined in 6.3.4

Figure 8.5.8.2.5-1: PINE leave via the PEGC

1.	The PINE determines to leave the PIN and sends PIN Management PINE leave from PIN request to the PEGC. The PIN Management PINE leave from PIN request contains the PIN ID which identifies the PIN to leave, PINE client ID and credentials if available, PEMC identify/PIN server address[optional].

2.	The PEGC identifies the received message is the PIN Management PINE leave from PIN request and perform the authorization. If authorized, the PEGC determines to forward the PIN leave request to the PEMC or the PIN server.

If authorization in PEGC is failed, directly skip to step 6.

The PEGC can decide to perform either Option 1 (from step 3a to step 3b) or Option 2 (from step 4a to step 4d). If direct communication between PEGC and PEMC always available, the PEGC performs Option 1.

3.	(Option 1, step 3a and step 3b) If PEGC determine to forward the PIN Management PINE leave from PIN request to the PEMC in step 2, PEGC forwards the PIN Management PINE leave from PIN request to the PEMC based on the PEMC identity in step 1 or by resolving the PIN ID.

The PEMC authorized the PINE to leave the PIN, and returns the PIN Management PINE leave from PIN response to the PEGC.

4.	(Option 2, step 4a and step 4d) If PEGC determine to forward the PIN Management PINE leave from PIN request to the PIN Sever in step2, the PEGC forwards the PIN Management PINE leave from PIN request to the PIN server based on the PIN server address in step 1 or by resolving the PIN ID. The PIN server forwards the PIN Management PINE leave from PIN request to the PEMC, and the PEMC authorized the PINE to leave the PIN, and returns the PIN Management PINE leave from PIN response to the PIN server. Further the PIN server return the PIN Management PINE leave from PIN response to the PEGC.

5.	Further, the PEMC updates the PIN and may notify other entities (e.g., existing joined members, PIN server).

6.	The PEGC returns the PIN Management PINE leave from PIN response to the PINE. The PEGC disables the access control information for this PINE. The PEGC stops relaying traffic to the PINE.

If authorization in PEGC is failed, PEGC generates the PIN Management PINE leave from PIN response to the PINE for the authorization failure.

* * * END of Change * * * *