SA WG6 Meeting #58	S6-233968

Chicago, USA, 13th – 17th November 2023	(revision of S6-233461)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0036 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of PEMC represents multiple PINEs or PEGCs to register |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-09-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As indicated in the agreed CR S6-233241 in SA6 57 meeting, the PEMC represents the PINE to perform the registration procedure. The information flow is extended. While for the network side, the PEMC and PIN server in the procedure also needs to have more actions to check the list of PINEs whether to allowed to be included in PIN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add clarification of the details of PIN profile and dynamic PIN profile. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how to PEMC and PIN server behave when receives the request that including list of PINEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

8.4.2.2.2	PINE registration indirectly to PIN server

The following procedure defines the PEMC can substitute the PINE/PEGC to register into PIN server.

Pre-conditions:

1.	The PEMC has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

2.	The UE Identifier or PIN client Identifier is available;

3.	The PEMC has been authorized to communicate with the PIN server;

4.	The PINE/PEGC has already received the IP address of PEMC.

Figure 8.4.2.2.2-1: PINE registration indirectly to PIN server

1.	The PINE/PEGC sends PINE Registration Request to the PEMC. The request includes the security credentials of the PINE received during authorization procedure and also the request may include the GPSI, MAC address, vendor name, device description, PINE Address.

2.	If the PEMC receives the PINE Registration Request from PINE, the PEMC may represent the PINE to register into PIN server by sending the PINE registration request to PIN server.

If the PEMC represents the PINEs to perform registration procedure, it should send a PINE Registration Request to PIN server including an Indication of representation registration and Lists of PINEs/PEGCs.

3.	The PIN server responds to the PEMC with PINE Registration Response with allocated PIN client ID in successful response.

When the PINE Registration Request that containing Indication of representation registration, the PIN server shall verify whether PINE(s) and PEGC(s) included in the List of PINEs/PEGCs are allowed to fullfill the requested role or not. The PIN server responses to PEMC with a successful response and may include a list of accepted registration information and a list of rejected registration information (if any). If all the PINEs in the Lists of PINEs/PEGCs are rejected, the PIN server sends a failure response including the failure cause.

4.	The PEMC sends the response received in step 3 to the PINE/PEGC.

When the PEMC receiving the PINE Registration response from PIN server, the PEMC should extract the accepted registration information and rejected registration information (if any) for all the PINEs, and send the result to each PINE in the Lists of PINEs/PEGCs.

* * * End of Change * * *