3GPP TSG-SA WG6 Meeting #58	S6-233713

Chicago, USA, 13th – 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.542 | CR | 0043 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to cardinality from UE perspective |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | PINAPP |  |  |  |  |  | Date: |  |  | 2023-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The cardinality from UE perspective is missing and essential for the architecture formalization. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the cardinality description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity may affect Stage 3 understanding and implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.2, 6.5.3, 6.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.5.2	PEMC

The following cardinality rules apply for PEMC:

a)	one or more PEMCs per PIN.

b)	Only one PEMC in certain PIN is assigned as primary role and other PEMCs if any are assigned with secondary role.

c)	One PEMC can act as PEMC for multiple PINs.

d)	One PEMC per UE.

### 6.5.3	PEGC

The following cardinality rules apply for PEGC:

a)	one or more PEGCs per PIN.

a)	For a certain PINE in PIN, one PEGC acts as default PEGC and other PEGCs act as backup.

c)	One PEGC can act as PEGC for multiple PINs.

d)	One PEGC per UE.

* * * Next Change * * * *

### 6.5.5	PIN client

The following cardinality rules apply for PIN client:

a)	one or more PIN clients per PIN.

b)	one PIN client per PIN element.

c)	one PIN client per UE.