3GPP TSG-SA WG6 Meeting #57	S6-233248

Xiamen, China 9th – 13th October 2023	(revision of S6-233007)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0238 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update subscription service operation for SS_LocationMonitoring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2023-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, subscriptions for Monitor Location events can be created and deleted. The issue with this approach is that when a VAL server wants to update a subscription (e.g. Area of Interest or Notify Interval or Notification Target URI), it has to delete the old subscription and create a new one. Depending on the order these operations are invoked, the VAL server may either receive duplicated notifications (create-new followed by delete-old) or may miss notifications (delete-old followed by create-new). Also, the CRUD (Create-Read-Update-Delete) paradigm used in the RESTful mapping in stage 3 does require to allow an update operation. Additionally, the unsubscribe procedure should target a specific Subscription Identifier (a VAL server could have several subscriptions). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enable a Monitor Location subscription update procedure triggered by a VAL server using a Subscription ID (obtained during the subscription creation). Additionally, the unsubscribe request targets a specific subscription ID (in case the requestor has several active subscriptions). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with stage 3 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.11, 9.3.2.12, 9.3.2.12a (New), 9.4.1, 9.4.6.5 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: capitalization problems, complete messages names in the figure, correction of the name of the service api operation, reuse of the subscription request and response messages to update a subscription |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

#### 9.3.2.11	Monitor Location Subscription Request

Table 9.3.2.11-1 describes the information flow from the VAL server to the LM server for monitoring the location deviation of the VAL UE(s) or for updating an existing monitor location subscription.

Table 9.3.2.11-1: Monitor Location Subscription Request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identifier of the VAL users or VAL UE whose location monitoring is requested to be monitored in a given location (NOTE). |
| Area of Interest | M | Geographic area location information where the VAL server wishes to monitor the VAL UE's location adherence. This could be geographical co-ordinates or VAL service area identified by the VAL service area ID. |
| Notify Interval | M | Periodic time interval in which the LM server needs to notify the VAL UE's location information to the VAL server. |
| Notification Target URI | M | Target URI where the VAL server wishes to receive the notifications about VAL UE's location. |
| Timeout | O | A timeout period when subscription response is not received. |
| NOTE: This information element shall not be updated via monitor location subscription update procedure in clause 9.3.2.12a. |  |  |

#### 9.3.2.12	Monitor Location Subscription Response

Table 9.3.2.12-1 describes the information flow from LM server to the VAL server for Monitor Location Subscription Response or monitor location subscription update response.

Table 9.3.2.12-1: Monitor Location Subscription Response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result |

* * * * Next Change * * * *

#### 9.3.2.12a	Monitor location subscription update procedure

Figure 9.3.2.12a -1 illustrates the high level procedure for allowing a VAL server update an existing monitor location subscription at the location management server.

Pre-conditions:

-	the VAL server has an active monitor location subscription at the location management server.

Figure 9.3.2.12a -1: Monitor location subscription update procedure

1.	When the VAL server decides to update an active monitor location subscription, it sends a monitor location subscription update request to the location management server including the information specified in table 9.3.2.11.

2.	The location management server shall check if the VAL server is authorized to update the subscription.

3.	If the VAL server is authorized to update the subscription, the location management server updates the monitor location subscription.

4.	The location management server sends a monitor location subscription update response to the VAL server including the information as specified in table 9.3.2.12.

* * * * Next Change * * * *

## 9.4	SEAL APIs for location management

### 9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting | VAL server | Request /Response |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Info | VAL server |  |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Unsubscribe_Location_Monitoring | VAL server |  |
|  | Update_Location_Monitoring_Subscription | VAL server |  |
|  | Notify_Location_Monitoring_Events | VAL server |  |
|  | Subscribe_Location_Area_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Area_Monitoring_Events |  |  |
|  | Update_Location_Area_Monitoring_Subscribe |  |  |
|  | Unsubscribe_Location_Area_Monitoring |  |  |
| SS_VALServiceAreaConfiguration | Configure_VAL_Service_Area | VAL server | Request/Response |
|  | Obtain_VAL_Service_Area | VAL server |  |
|  | Update_VAL_Service_Area | VAL server |  |
|  | Delete_VAL_Service_Area | VAL server |  |
|  | Subscribe_VAL_Service_Area_Change_Event | SEAL server | Subscribe/Notify |
|  | Notify_VAL_Service_Area_Change_Event | SEAL server |  |
|  | Unsubscribe_VAL_Service_Area_Change_Event | SEAL server |  |

* * * * Next Change * * * *

#### 9.4.6.5	Update_Location_Monitoring_Subscription

API operation name: Update_Location_Monitoring_Subscription

Description: Updating an active monitor location subscription.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.11

Outputs: See subclause 9.3.2.12

See subclause 9.3.2.12a for the details of usage of this API operation.

* * * * End of Changes * * * *