3GPP TSG-SA WG6 Meeting #58	S6-233874

Chicago, USA, 13th – 17th November 2023	(revision of S6-233677)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0271 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding the information flow and AP for NRM reliable transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the AF influence URSP procedure for reliable transmission defined in clause 14.3.10, there is no related information flow and API introduced, and some typo error exists. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper is proposed to complete the information flow and API for NRM reliable transmission procedure, and correct the typo errors. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For current specification, the information flow and API for reliable transmission is incomplete, and some typo error exsits. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.2.x (new), 14.3.2.y (new), 14.4.2.z (new), 14.3.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

14.3.2.x	Reliable transmission request

Table 14.3.2.x-1 describes the information flow for reliable transmission request from SEALDD server (or VAL server) to the NRM server.

Table 14.3.2.x-1: Reliable transmission request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the SEALDD server (or VAL server) performing the request. |
| Application descriptors | O | A pair of applic traffic descriptors (e.g. address, port, transport layer protocol) of the SEALDD server (or VAL server), used to establish redundant transmission paths. |
| UE ID | O | The identity of VAL UE. |
| UE address | O | The address of VAL UE |

14.3.2.y	Reliable transmission response

Table 14.3.2.y-1 describes the information flow for the reliable transmission response from NRM server to SEALDD server (or VAL server).

Table 14.3.2.y-1: Reliable transmission response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the reliable transmission request operation. |

* * * Next Change * * * *

14.4.2.z	Reliable_Transmission_request

API operation name: Reliable_Transmission_request

Description: Requesting Reliable Transmission service

Known Consumers: SEALDD server or VAL server

Inputs: See subclause 14.3.2.x.

Outputs: See subclause 14.3.2.y.

See subclause 14.3.10.2 for the details of usage of this API operation.

* * * Next Change * * * *

14.3.10	AF influence URSP procedure for reliable transmission

14.3.10.1	General

The procedures related to the AF influence URSP for reliable transmission are described in the following subclauses.

14.3.10.2	AF influence URSP procedure for reliable transmission

Pre-conditions:

1.	The SEALDD server (or VAL server) has decided to use reliable transmission for a specific SEALDD client (or VAL client) and has got the UE address or UE ID.

Figure 14.3.10.2-1: AF influence URSP procedure for reliable transmission

1.	The NRM server receives from the SEALDD server (or VAL server) about the request for reliable transmission service with the application descriptors for the two redundant transmission paths. The SEALDD client’s current UE address or UE ID is also provided to identify the affected UE.

2.	After receiving the request from SEALDD server (or VAL server), the NRM server associates the available DNN and S-NSSAI information for the two application descriptors. NRM server triggers via N5/N33 with the AF guidance for URSP procedure as described in 3GPP TS 23.502 [11] clause 4.15.6.10. The AF request includes the application traffic descriptors and the associated DNN, S-NSSAI, and also includes UE address or UE ID received in step 1.

3.	NRM server sends response about reliable transmission service.

* * * Next Change * * * *

14.4.1	General

Table 14.4.1-1 illustrates the SEAL APIs for network resource management.

Table 14.4.1-1: List of SEAL APIs for network resource management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_NetworkResourceAdaptation | Reserve_Network_Resource | VAL server | Request /Response |
|  | Request_Unicast_Resource | VAL server | Request /Response |
|  | Update_Unicast_Resource | VAL server | Request /Response |
|  | Request_Multicast_Resource | VAL server | Request /Response |
|  | Notify_UP_Delivery_Mode | VAL server | Subscribe/Notify |
|  | TSC_Stream_Availability_Discovery | VAL server | Request /Response |
|  | TSC_Stream_Creation | VAL server | Request /Response |
|  | TSC_Stream_Deletion | VAL server | Request /Response |
|  | Reliable_Transmission | VAL server, SEALDD server | Request /Response |
| SS_EventsMonitoring | Subscribe_Monitoring_Events | VAL server | Subscribe/Notify |
|  | Notify_Monitoring_Events | VAL server |  |
| SS_NetworkResourceMonitoring | Subscribe_Unicast_QoS_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Unicast_QoS_Monitoring | VAL server |  |
|  | Unsubscribe_Unicast_QoS_Monitoring | VAL server |  |
|  | Obtain_Unicast_QoS_Monitoring_Data | VAL server | Request /Response |