3GPP TSG-SA WG6 Meeting #58	S6-233867

Chicago, USA, 13th – 17th November 2023	(revision of S6-233720)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0277 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to add missing API operations for BDT configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2023-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The operations for get, update and delete are missing in the BDT configuration API. The operations are in incorrect API of Network Resource Adapations. New API is to be created for BDT Configuration |  |  |  |  |  |  |  |  |
| Summary of change: |  | The new API is created and operations are added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The stage 3 implementation will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.4.1, 14.4.2.x (new) , 14.4.2.y (new) , 14.4.2.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 14.4.1	General

Table 14.4.1-1 illustrates the SEAL APIs for network resource management.

Table 14.4.1-1: List of SEAL APIs for network resource management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_NetworkResourceAdaptation | Reserve_Network_Resource | VAL server | Request /Response |
|  | Request_Unicast_Resource | VAL server | Request /Response |
|  | Update_Unicast_Resource | VAL server | Request /Response |
|  | Request_Multicast_Resource | VAL server | Request /Response |
|  | Notify_UP_Delivery_Mode | VAL server | Subscribe/Notify |
|  | TSC_Stream_Availability_Discovery | VAL server | Request /Response |
|  | TSC_Stream_Creation | VAL server | Request /Response |
|  | TSC_Stream_Deletion | VAL server | Request /Response |
|  | BDT_Configuration_Request | VAL server | Subscribe/Notify |
|  | BDT_Negotiation_Notification | VAL server | Subscribe/Notify |
|  | BDT_Configuration_Get | VAL server | Request /Response |
|  | BDT_Configuration_Update | VAL server | Request /Response |
|  | BDT_Configuration_Delete | VAL server | Request /Response |
| SS_EventsMonitoring | Subscribe_Monitoring_Events | VAL server | Subscribe/Notify |
|  | Notify_Monitoring_Events | VAL server |  |
| SS_NetworkResourceMonitoring | Subscribe_Unicast_QoS_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Unicast_QoS_Monitoring | VAL server |  |
|  | Unsubscribe_Unicast_QoS_Monitoring | VAL server |  |
|  | Obtain_Unicast_QoS_Monitoring_Data | VAL server | Request /Response |
|  | Update_Unicast_QoS_Monitoring_Subscription | VAL server |  |

* * * Next Change * * * *

#### 14.4.2.x	BDT_Configuration_Get_request

API operation name: BDT_Configuration_Get_request

Description: Retrieving Background data transfer configuration.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.c

Outputs: See subclause 14.3.2.d

See subclause 14.3.13.z for the details of usage of this API operation.

#### 14.4.2.y	BDT_Configuration_Update_request

API operation name: BDT_Configuration_Update_request

Description: Updating Background data transfer configuration.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.a

Outputs: See subclause 14.3.2.b

See subclause 14.3.13.y for the details of usage of this API operation.

#### 14.4.2.z	BDT_Configuration_Delete_request

API operation name: BDT_Configuration_Delete_request

Description: Deleting Background data transfer configuration.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.x

Outputs: See subclause 14.3.2.y

See subclause 14.3.13.x for the details of usage of this API operation.