3GPP TSG-SA3 Meeting #116	S3-241941

Jeju, South Korea,  20th - 24th May 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.533 | CR | 0068 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Implementing CR to TS 33.533 agreed in SA plenary in SP-240498 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Huawei, HiSilicon, Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL_Sec |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA plenary has agreed a CR to TS 33.533 in SP-240498 as indicated in LS in SP-240497. The CR in SP-240498 was updating the same sentence in clause 6.3.5 as another agreed CR S3-240948 and MCC was not able to implement both of the CR’s as they impacted the same clause. The changes in clause 6.3.5 introduced by SP-240498 was then not implemented. Also due to this collition, the text “for the UEs belonging to the same PLMN” of following change in clause 6.3.5 from S3-240948 was not implemented. When receiving the Ranging/SL Pos or LCS client itioning service request from the AF,/5GC NF or LCS client, the GMLC (i.e. anchor GMLC) interacts with the UDM to check the UE privacy profile for the UEs belonging to the same PLMN. If any of n UEs belong to different PLMNs, then the anchor GMLC sends a request to the Home GMLC of each of those UEs to check the Ranging/SL positioning privacy profiles of the UEs. Therefore this CR implements the missing part from the CR to TS 33.533 in SP-240498 and the CR S3-240948. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR implements the missing part from the agreed CR to TS 33.533 at SA plenary in SP-240498 and the agreed CR S3-240948. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with stage 2 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.5.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the 1st Change ****************

### 6.3.5	Procedures for authorization of AF/5GC NF/LCS Client for Ranging/SL positioning service exposure

#### 6.3.5.0	General

For the authorization of the AF, 5GC NF or LCS client for Ranging/SL Positioning service exposure, the SL-MT-LR procedure specified in TS 23.273 [3] is taken as the baseline. The authorization shall be performed towards all the n UEs (n ≥ 2), i.e. UE1, UE2, ..., UEn in the request message. If all of the UEs grant permission for Ranging/SL Positioning exposure, the GMLC shall forward the service request from the AF,5GC NF or LCS client to the AMF. If none of the UEs grants permission for Ranging/SL Positioning exposure, the GMLC shall reject the service request. If part of the UEs grant and part of the UEs don't grant permission for Ranging/SL Positioning exposure, the GMLC shall decide to proceed with or reject the service request from the AF, 5GC NF or LCS client based on the privacy check results of the n UEs and a criterion up to implementation, e.g. a local rule configured by the network operator. If the GMLC decides to accept the service request, it shall only include the identities of the UEs granting permission in the service request forwarded to the AMF.

When receiving the Ranging/SL Positioning service request from the AF,5GC NF or LCS client, the GMLC (i.e. anchor GMLC) interacts with the UDM to check the UE privacy profile for Ranging/SL Positioning service as specified in Annex B for the UEs belonging to the same PLMN. If any of n UEs belong to different PLMNs, then the anchor GMLC sends a request to the Home GMLC of each of those UEs to check the Ranging/SL positioning privacy profiles of the UEs.

NOTE:	The address of the Home GMLC of the UE(s) in a different PLMN is determined by the anchor GMLC based on local configuration or by the NRF query.

*************** End of the Changes ****************