3GPP TSG-SA Meeting #104	SP-240885

Shanghai, China, 18th–21st June 2024

Title:	Summary for the work on the security aspect of home network triggered primary authentication in SA3

Type:	Work Item Summary

Agenda Item:	21.5

Source:	He Li

Contact:	lihe2@huawei.com

Summary based on the input provided by Huawei in SP-240885.

| Unique ID | Name | Acronym | WG | WID | WI Rapporteur name |
| --- | --- | --- | --- | --- | --- |
| 950020 | Study of Security aspect of home network triggered primary authentication | FS_HN_Auth | S3 | SP-220198 | He Li, Huawei |
| 980024 | Security aspects of home network triggered primary authentication | HN_Auth | S3 | SP-221147 | He Li, Huawei |
| 1010001 | CT aspects of home network triggered primary authentication | HN_Auth | C4 | CP-232028 | Jing, Hao, Huawei |

Introduction

The requirements for the support of a Home Network triggered primary authentication are captured in TS 33.501[1] and TS 29.503[2] based on the outcome of the study documented in TR 33.741 [3]. This new feature enables home network to trigger the primary authentication. In the past, it has been always the case that this procedure is only triggered by the visited network.

Description

In this feature, the UDM is the central point that can trigger the primary authentication procedure. The UDM takes the decision based on local policy or a request from another NF (AAnF or OAM). The procedure is initiated by the UDM notifying the AMF. The AMF then based on its local policy and/or the status of the UE determines whether or not to trigger the primary authentication procedure. The procedure has been specified in TS 33.501 [1].

References

List of related CRs:

https://portal.3gpp.org/ChangeRequests.aspx?q=1&workitem=980024

[1]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[2]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[3]	3GPP TR 33.741: "Study of Security aspect of home network triggered primary authentication"