TSG-SA/CT/RAN Meeting #104	SP-240907

Shanghai, China, June 18 – 21, 2024

Title:	Summary for Ranging-based Service and sidelink positioning

Type:	Work Item Summary

Agenda Item:	21.5 (SA)

Source:	Xiaomi

Contact:	Sherry (Yang) Shen

shenyang6@xiaomi.com

Summary based on the input provided by Xiaomi in SP-240907.

9.6	Ranging-based Service and sidelink positioning

| 910040 | Ranging-based Service and sidelink positioning | Ranging |  | SP-210212 | Xiaowei jiang, Xiaomi |
| --- | --- | --- | --- | --- | --- |
| 880039 | Study on Ranging | FS_Ranging | S1 | SP-200575 | Xiaowei jiang |
| 910034 | Stage 1 for Ranging | Ranging | S1 | SP-210212 | Xiaowei jiang, Xiaomi |
| 940069 | Study on Ranging based services and sidelink positioning | FS_Ranging_SL | S2 | SP-211647 | Shen, Sherry (Yang), Xiaomi, |
| 980015 | Stage 2 of Ranging_SL | Ranging_SL | S2 | SP-230106 | Shen, Sherry (Yang), Xiaomi |
| 960036 | Study on Security Aspects of Ranging Based Services and Sidelink Positioning | FS_Ranging_SL_Sec | S3 | SP-220534 | Wei Lu, Xiaomi, |
| 990046 | Security Aspects of Ranging_SL | Ranging_SL_Sec | S3 | SP-230769 | Lu, Wei, Xiaomi |
| 990014 | CT1 aspects of Ranging_SL | Ranging_SL | C1 | CP-230331 | Tingfang Tang, Xiaomi |
| 990066 | CT3 aspects of Ranging_SL | Ranging_SL | C3 | CP-230331 | Tingfang Tang, Xiaomi |
| 990067 | CT4 aspects of Ranging_SL | Ranging_SL | C4 | CP-230331 | Tingfang Tang, Xiaomi |
| 1000009 | Charging Aspects of Ranging and Sidelink Positioning | FS_Ranging_SL_CH | S5 | SP-230622 | Zhiwei Mo, China Telecom, |

Introduction

Ranging based service provides the distance between two UEs or more UEs and/or the direction of one UE (i.e. Target UE) from another UE (i.e. SL Reference UE) via PC5 operations.

Sidelink Positioning provides absolute location, relative position, or Ranging information of a UE by using PC5 for the positioning. A Located UE can be used to determine the absolute location of a Target UE using Sidelink Positioning.

The operation of Ranging/Sidelink Positioning can be performed with Network-assisted Operation, Network-based Operation or UE-only Operation:

-	In the Network-based Operation, 5GC NF(s) is involved for the service request handling and result calculation.

-	In the UE-only Operation, the service request handling and result calculation are performed by UE.

-	In the Network-assisted Operation, 5GC NF(s) is involved for the service request handling and assist UE for the result calculation.

The Ranging/Sidelink Positioning service request can be initiated by a UE (i.e. SL Positioning Client UE, Target UE, SL Reference UE), a 5GC NF, an LCS Client or an AF.

Description

Ranging based services and sidelink positioning in 5G System are applied for commercial, V2X and public safety use cases in in-coverage, partial coverage, and out-of-coverage of 5G network using 5G NR PC5 RAT, which includes the following functionality and features:

- Authorization and Provisioning for Ranging/SL positioning service

- Ranging/Sidelink Positioning UE Discovery & Selection

- Ranging/SL Positioning control

- UE Positioning assisted by Sidelink Positioning and involving 5GC

- UE-only Operation for SL Positioning using Located UE

- Ranging/SL Positioning service exposure

- QoS Handling

- Subscription to Ranging/SL Positioning

- Support of Concurrent Ranging/Sidelink Positioning Requests

- Security

- Charging

References

[1]	3GPP TS 23.586: "Architectural Enhancements to support Ranging based services and Sidelink Positioning".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[6]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[7]		3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[8]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[9]	3GPP TS 38.355: "NR; Sidelink Positioning Protocol (SLPP); Protocol Specification".

[10]	3GPP TS 33.533: "Security aspects of ranging based services and Sidelink positioning".

[11]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[12]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[13]	3GPP TS 24.080: "Supplementary services specification; Formats and coding".

[14]	3GPP TS 24.514: "Ranging based services and sidelink positioning in 5G system (5GS); Stage 3".