3GPP TSG-SA WG6 Meeting #65	S6-250428

Athens, Greece, 17th – 21st February 2025	(revision of S6-250206)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0248 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solving ENs on CAPIF interconnection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | S6-244551 agreed in SA6#63 developed the CAPIF interconnection related functionality. However, ENs to remember that the security solution was under study in SA3 were included. 8.25.3.7 API invoker obtaining authorization for service API access in CAPIF interconnection Editor's note: The information exchanged between CCF-B and CCF-A in the interconnection obtain API authorization procedure will be addressed by SA3. 8.25.3.10 Procedure for obtaining security information in CAPIF interconnection Editor's note: The security information obtained in CAPIF interconnection procedures will be addressed by SA3. However, the specification of the information exchanged in the procedures need to be covered in the information flows section. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The Editor’s Notes are removed in the procedures - Clauses to introduce flow information are created in the information flows section. - Bullet 3 is rephrased to make it more open. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The remaining work is not properly captured in the TS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.25.2.x (new), 8.25.2.y (new), 8.25.3.7, 8.25.3.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.25.2.x	Interconnection API invoker obtaining authorization to access service API

Editor's note: The information exchanged between CCF-B and CCF-A in the interconnection obtain API authorization procedure will be addressed by SA3.

* * * Next Change * * * *

#### 8.25.2.y	Interconnection Authentication and authorization between the API invoker and the AEF

Editor's note: The information exchanged between CCF-B and CCF-A in the interconnection to obtain API invoker authentication and authorization procedure will be addressed by SA3.

* * * Next Change * * * *

#### 8.25.3.7	API invoker obtaining authorization for service API access in CAPIF interconnection

Pre-condition:

1.	The API invoker has discovered service APIs provided by an AEF via procedure defined in step 1 and 2 of clause 8.25.3.2.

2.	The API invoker and the CCF-B are in the same trusted domain.

3.	The AEF and the CCF-A are in the same trusted domain.

4.	The CCF-A and the CCF-B are connected to each other, and they have business agreement for service API authorization.

5.	The CCF-A is the authorization function for service API access on the AEF.

Figure 8.25.3.7-1: Procedure for the API invoker obtaining authorization for service API access

1.	The API invoker sends an obtain service API authorization request to the CCF-B for obtaining permission to access the service API by including the API invoker identity information and any information required for authentication of the API invoker.

2.	After successful authentication validation of the API invoker, if the CCF-B determines that the service API access authorization cannot be done by itself alone, then the CCF-B sends an interconnection obtain API authorization from the CCF-A.

3.	The authorization information to access the service APIs is sent to the API invoker in the obtain service API authorization response.

* * * Next Change * * * *

#### 8.25.3.10	Procedure for obtaining security information in CAPIF interconnection

Pre-condition:

1.	The AEF has no security information available for authentication and/or authorization.

2.	The CCF-B has security information available for authentication and/or authorization corresponding to one or more service APIs.

3.	The AEF and the CCF-A are in the same trusted domain.

Figure 8.25.3.10-1: Procedure for obtaining security information in CAPIF interconnection

1.	The AEF obtains the API invoker information required for authentication and/or authorization by the AEF from the CCF-A by including the details of the API invoker and hosted service API. After successful authentication validation of AEF, if the CCF-A determines that it has no security information, then the CCF-A obtains security information request from the CCF-B. Then the security information required for service API invocations is sent to the AEF.

NOTE 1:	If the CCF-A has security information available, it executes the procedure defined in clause 8.14.3, clause 8.15.3 or clause 8.16.3 without interaction with the CCF-B.

NOTE 2:	The CCF-A can subscribe to API invoker onboarding event at the CCF-B according to the procedure in clause 8.8.3 and receive notifications about any onboarded API invoker and its security related onboarding information at the CCF-B according to the procedure in clause 8.8.4.

* * * END of Change * * * *