3GPP TSG-SA WG6 Meeting #65	S6-250575

Athens, Greece 17th – 21st February 2025	(revision of S6-250554, S6-250470, S6-250246)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0257 | rev | 3 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on resource owner consent upon service API invocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Proposes updates as per conclusion of Key Issue#1 relating to Solution #20 in TR 23.700-22. SA6 agreed in SA6#64 meeting, to update the service API invocation in clause 8.16.2.2 with failure messge indicating the missing consent information required for authorizing the service API invocation. However, the related procedure in 8.16.3 is not aligned to solution #20. This CR proposes the udpate to clause 8.16.3 clarify the behviour of AEF when resource owner consent information is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In TR 23.700-22, conclusions of Key Issue #1 states: Solution#20 can be considered as candidate for normative work for enhancing the procedure to obtain resource owner consent upon service API invocation at AEF.Solution#25 is an enhancement to solution#22, whose specific aspects can be considered together with solution#22 during normative phase. The CR proposed is using the approach proposed by solution#20 i.e. to clarify the procedure in clause 8.16.3. Additionally, minor correction in table 8.16.2.2-1, consent term is missing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support for necessary failure handling will be missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.16.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************ First change ******************************/

### 8.16.3	Procedure

Figure 8.16.3-1 illustrates the procedure for API invoker authorization to access service APIs.

Pre-conditions:

1.	The API invoker has been authenticated.

2.	The API invoker associated authorization information is available at AEF.

Figure 8.16.3-1: Procedure for API invoker authorization to access service APIs

1.	The API invoker triggers service API invocation request to the AEF, including the service API to be invoked.

NOTE 1:	Authentication can also be performed if not authenticated previously.

NOTE 2:	The API invoker can trigger several service API invocations asynchronously.

2.	Upon receiving the service API invocation request, the AEF checks whether the API invoker is authorized to invoke that service API, based on the authorization information.

2a.	If the AEF does not have information required to authorize service API invocation, the AEF obtains the authorization information from the CAPIF core function.

3.	The AEF executes the service logic for the invoked service API.

4.	The API invoker receives the service API invocation response as a result of the service API invocation. If in step 2, the AEF determines that the required resource owner authorization information is not available for the requested service API, then the AEF responds to API Invoker with failure status message indicating that the authorization information with resource owner is not available, as specified in Table 8.16.2.2-1.

# /************************ End of changes ******************************/