3GPP TSG-SA WG6 Meeting #65	S6-250439

# Athens, Greece, 17th – 21st February 2025	                                 (revision of S6-250132)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0134 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add service operation of Crossflow measurement and delay difference for XR application |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2025-2-7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For XR application, how to measure the key parameters Crossflow measurement and delay difference is not clear. To clarify the method of Crossflow measurement and delay difference measurement, add service operation in clause 9.12.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add service operation of Crossflow measurement and delay difference measurement for XR application. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to measure the key parameters Crossflow measurement and delay difference is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.12.2.1.2, 9.12.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.12.2.1	SEALDD enabled XR data transmission service for XR application

9.12.2.1.1	General

The following clauses specify procedures, information flows and APIs about SEALDD enabled data transmission for XR application, including the SEALDD facilate PDU set handling.

9.12.2.1.2	SEALDD enabled XR data transmission establishment

Figure 9.12.2.1.2-1 illustrate the procedure for establishing XR data transmission connection, and the SEALDD facilitates the XR application to transmit its data between the VAL client and VAL server with RTP packetilization and PDU set inclusion.

Pre-condition:

-	The VAL server can discover and select the SEALDD server by CAPIF functions.

Figure 9.12.2.1.2-1: SEALDD enabled XR data transmission connection establishment procedure

1.	The VAL server decides to use SEALDD service for XR traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_XRTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, SEALDD-S Data transmission connection information of the VAL server side, and optionally, the protocol description, the QoS information for the application traffic, e.g. QoS requirements. As one of key parameters for XR application, Crossflow measurement can be measured as procedure in clause 9.7.2.2 step 1.

2.	Same as step 2 of clause 9.2.2.2. For application multi-modal service, the SEALDD server derives PDU Set related assistance information based on received VAL service ID and/or VAL server ID, and protocol description for interacting with NEF/PCF.

3-5.	Same as step 3-5 of clause 9.2.2.2.

6.	The SEALDD client establishes XR transmission connection with the SEALDD server. The request includes the SEALDD client ID, VAL user/UE ID, VAL server ID, VAL service ID, SEALDD-UU flow IDs, and traffic descriptors for the multiple flows from the SEALDD client side. The SEALDD server sends the SEALDD traffic descriptors for multiple flows from SEALDD server side (e.g. address/port for multiple SEALDD-UU flow) to the SEALDD client, and may send the protocol description (UL related info) received from the VAL server to the SEALDD client in the XR transmission connection response. The response also includes a multi-modal SEALDD-UU flow ID which the SEALDD server allocates.

7-8.	Same as step 8-9 of clause 9.2.2.2.

The XR application traffic is exchanged between VAL client and VAL server via SEALDD layer as described in clause 9.2.2.2. If packetization indication indicates that SEALDD layer needs to perform packetization, the SEALDD server performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD client for downlink application traffic. Similarly, the SEALDD client performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD server for uplink application traffic. The SEALDD server and client also perform PDU Set inclusion (e.g. in RTP extension as defined in 3GPP TS 26.522 [TS26522]), and if needed, stream session and transport management (e.g. RTCP, RTSP).

* * * Next Change * * * *

9.12.2.2	SEALDD enabled multi-modal flow synchronization

9.12.2.2.1	General

The following clauses specify procedures, information flows and APIs about SEALDD enabled data transmission for XR application, including SEALDD enabled multi-modal flow synchronization.

9.12.2.2.2	SEALDD enabled multi-modal flow synchronization

Figure 9.12.2.2.2-1 illustrate the procedure for SEALDD enabled multi-modal flow synchoronization, the SEALDD server determines/updates the required QoS information for multi-modal flow(s), and further interacts with 5G network.

Pre-condition:

-	The VAL server can discover and select the SEALDD server by CAPIF functions.

-	The SEALDD server has been provisioned with a multi-modal XR policy including the synchronization threshold, as specified in clause 9.10.3.1.

-	The SEALDD client has been provisioned with a multi-modal SEALDD policy, as specified in clause 9.10.2.4.

Figure 9.12.2.2.2-1: SEALDD enabled multi-flow synchronization procedure

1.	An on-going multi-modal data transmission connection is established according to the steps 1-8 of clause 9.12.2.1.2.

2.	Upon the multi-modal flows alignment policy triggered, the SEALDD server may help to provide the flows alignment assistance information (e.g. timestamp in the RTP header, RTCP). If the maximum acceptable duration for traffic flow alignment is not provided, then the SEALDD server may determine the maximum acceptable duration for traffic flow alignment based on VAL service ID, flows transmission requirement, transmission quality, and the synchronization threshold. The server may translate the traffic descriptor into multi-modal SEALDD flow ID.

NOTE 1:	The flow alignment assistance information can be obtained by SEALDD server based on SEALDD policy.

NOTE 2:	The SEALDD client performs the caching and transmission to align multi-modal flows based on the flow alignments assistance information and maximum acceptable time duration.

Editor's note: Whether and how to make use of existing CN functionality (e.g. PDU set related feature) is FFS.

NOTE 3:	The NTP timestamp and RTP timestamp in RTCP sender report (SR) can be used to to identify the associated packets among muti-modal flow, and further be used to perform alignment in SEALDD client.

3.	Upon the multi-modal flows alignment policy triggered, the SEALDD client initiates the multi-modal flows alignment based on the policy. The flows need to be aligned are identified by the VAL service ID, multi-modal SEALDD flow ID, and flow alignment assistance information.

If the flow alignment assistance information is provided, the SEALDD client can identify the associated packets (e.g., those with the same RTP timestamp) in the multi-modal flows. After all associated packets in the multi-modal flows have arrived, the SEALDD client sends the associated packets to the application client. If the maximum acceptable time duration is provided, once this maximum acceptable time is reached, the SEALDD client will no longer wait for the associated packets in multi-modal flows, even if they have not arrived yet.

4.	The SEALDD server performs data transmission quality measurement, as defined in clause 9.7.2.1 or clause 9.7.2.3, in SEALDD-UU interface based on the mapping information for multiple flow association information between SEALDD-S interface and SEALDD-UU interface. Upon receiving the packets from multiple associated flows in SEALDD-S interface, the SEALDD server performs the packet encapsulation with sending timestamp information in the corresponding SEALDD-UU interface, and can calculate the transmission delay measurement result of multiple associated flows after obtaining the receiving timestamp from the SEALDD client. As one of key parameters for XR application, delay difference can be measured as procedure in clause 9.7.2.3 step 10.

5.	Based on the data transmission quality measurement results obtained for multiple associated flow over SEALDD-UU interface in step 4, and the synchronization threshold for multi-modal application as described in pre-condition, the SEALDD server can determine the service flow(s) (i.e. address/port for SEALDD-UU flow) that needs to be adjusted among the multiple associated flows in SEALDD-UU interface, and the corresponding required QoS information (i.e. transmission delay).

6.	The SEALDD server sends the AF request to 5GC via N33/N5 with the SEALDD traffic descriptor of the adjusted flow(s) (i.e. address/port for the adjusted SEALDD-UU flow) and the corresponding required QoS information determined in step 5, by utilizing the AF session with required QoS procedure in clause 4.15.6.6 of TS 23.502 [6]. The SEALDD traffic descriptor of the adjusted flow(s) contains the address or port in SEALDD server side, and/or SEALDD client side.

NOTE:	This procedure is applicable for both downlink and uplink synchronization of multi-modal flow. For downlink and/or uplink, the step 5 is determined according to the measured downlink and/or uplink data transmission quality measurements in step 4. For uplink synchronization, the SEALDD server performs the caching and transmission to align multi-modal flows based on the flow alignment assistance information before sending the associated packets to the VAL server.

After requesting the transmission quality optimization on 5G network with the required QoS for the adjusted flow(s), the multi-flow synchronization of multi-modal application can be satisfied.

Editor's note: Whether and how the multi-modal flows alignment monitoring will be defined is FFS.

* * * END of Change * * * *