3GPP TSG-SA WG6 Meeting #65	S6-250551

Athens, Greece, 17th – 21st February 2025	(revision of S6-250441)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0140 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of SEALDD-enabled flow Alignment procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2025-02-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current flow synchronization is based on the RTP timestamp synchronization mechanism. In the existing procedure, the packets wait until all the flows arrive and discard all the packets arriving after the acceptable delay or synchronization threshold. It performs synchronization based on the matching same timestamp. Generally, it may work fine but since the XR devices communicate over RAN, RAN may induce delays due to dynamic wireless channel like poor channel, user mobility, congestion etc and put the flows out of synchronization. Mind that the timestamp matching-based synchronization does not reflect the delays, but just set the same timestamp in packets that need to arrive on synch. If the delay is greater than the synchronization threshold, the packet is discarded, and no record is kept e.g. with the time difference between the arrival of the first and last packet with the same time stamp The current procedure does not account either for the RAN congestion or does not predict the RAN congestion to maintain synchronization. If the SEALDD server has information about the congestion, then it can do something like the adjustment of flows to maintain synchronization accordingly to the delays observed during RAN congestion. Also, the current procedure mentions adjusting the QoS to enable the synchronization and this will not work because the RAN is already congested and it will not be able to enforce the requested update of QoS. Therefore, the proposed solution provides timing adjustments like sending packets in advance or delaying the packets relative to each other based on RAN congestion and delay predictions to support the existing synchronization in the procedure. It helps to deliver all assocaited packets within the wait time window, e.g. sending earlier those packets that would be discarded because would not arrive on time and would exceed the time window. So, essentially would enable reducing the wait time window for the SEALDD layer(client or server) to receive all the packets which would also contribute to improve the user experience. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing RAN congestion and predictions-based timing adjustments to support the synchronization of the flows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SEALDD server and client could not perfom flow alignment and may lead to drop of packets due to RAN congestion and non-syncrhonization of flows. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.12.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 9.12.2.2.2	SEALDD enabled multi-modal flow synchronization

Figure 9.12.2.2.2-1 illustrate the procedure for SEALDD enabled multi-modal flow synchoronization, the SEALDD server determines/updates the required QoS information for multi-modal flow(s), and further interacts with 5G network.

Pre-condition:

-	The VAL server can discover and select the SEALDD server by CAPIF functions.

-	The SEALDD server has been provisioned with a multi-modal XR policy including the synchronization threshold, as specified in clause 9.10.3.1.

-	The SEALDD client has been provisioned with a multi-modal SEALDD policy, as specified in clause 9.10.2.3.

Figure 9.12.2.2.2-1: SEALDD enabled multi-flow synchronization procedure

1.	An on-going multi-modal data transmission connection is established according to the steps 1-8 of clause 9.12.2.1.2.

2.	Upon the multi-modal flows alignment policy triggered, the SEALDD server may help to provide the flows alignment assistance information (e.g. timestamp in the RTP header, RTCP) to the SEALDD client. If the maximum acceptable duration for traffic flow alignment is not provided, then the SEALDD server may determine the maximum acceptable duration for traffic flow alignment based on VAL service ID, flows transmission requirement, transmission quality, and the synchronization threshold. The server may translate the traffic descriptor into multi-modal SEALDD flow ID.

The SEALDD server may communicate with the 5GS to get the analytics and prediction for the RAN congestion as per clause  6.6, clause 6.7, and clause 6.8 3GPP TS 23.288 [19]. Based on the RAN congestion analytics and prediction information, synchronization threshold and/or multi-modal flow alignment policy, the SEALDD server derives the delay introduced by the RAN and adjusts the flow alignment assistance information like DL/UL transmission timing advance or delay period. In case flow alignment is required, the flow alignment assistance information may contain UL/DL transmission timing advance period, UL/DL transmission timing delay period and the associated flow IDs in the multi-modal flows.

The UL flow alignment assistance information may be sent to the SEALDD client in the Transmission quality management request specified in clause 9.9.3.1.

NOTE 1:	The flow alignment assistance information can be obtained by SEALDD server based on SEALDD policy.

NOTE 2:	The SEALDD client performs the caching and transmission to align multi-modal flows based on the flow alignments assistance information and maximum acceptable time duration.

Editor's note: Whether and how to make use of existing CN functionality (e.g. PDU set related feature) is FFS.

NOTE 3:	The NTP timestamp and RTP timestamp in RTCP sender report (SR) can be used to to identify the associated packets among muti-modal flow, and further be used to perform alignment in SEALDD client.

NOTE 4: The option to rely on UE congestion analytics and RAN congestion analytics, as exposed by the 5GC via NWDAF has a margin of error, as with any predictions. To handle the real-time/near real-time constraint of these predictions, these UE congestion analytics can be requested ahead of time from the 5GC/NWDAF, e.g., once the XR session is started by the UE.

3.	Upon the multi-modal flows alignment policy triggered, the SEALDD client initiates the multi-modal flows alignment based on the policy. The flows need to be aligned are identified by the VAL service ID, multi-modal SEALDD flow ID, and flow alignment assistance information.

If the flow alignment assistance information is provided, the SEALDD client can identify the associated packets (e.g., those with the same RTP timestamp) in the multi-modal flows. After all associated packets in the multi-modal flows have arrived, the SEALDD client sends the associated packets to the application client. If the maximum acceptable time duration is provided, once this maximum acceptable time is reached, the SEALDD client will no longer wait for the associated packets in multi-modal flows, even if they have not arrived yet.

For the UL synchronization, the SEALDD client sends the UL packets as per the flow alignment assistance information. The UL transmission timing advance period indicates SEALDD client to advance the UL packet transmission by advance period. The UL transmission timing delay period correction mode indicates SEALDD client to delay the transmission of the UL packet by UL delay period units. After all associated packets in the multi-modal flow have arrived at the SEALDD server, then the SEALDD server sends the associated UL packets to the VAL server.

4.	The SEALDD server performs data transmission quality measurement, as defined in clause 9.7.2.1 or clause 9.7.2.3, in SEALDD-UU interface based on the mapping information for multiple flow association information between SEALDD-S interface and SEALDD-UU interface. Upon receiving the packets from multiple associated flows in SEALDD-S interface, the SEALDD server performs the packet encapsulation with sending timestamp information in the corresponding SEALDD-UU interface, and can calculate the transmission delay measurement result of multiple associated flows after obtaining the receiving timestamp from the SEALDD client.

5.	Based on the data transmission quality measurement results obtained for multiple associated flow over SEALDD-UU interface in step 4, and the synchronization threshold for multi-modal application as described in pre-condition, the SEALDD server can determine the service flow(s) (i.e. address/port for SEALDD-UU flow) that needs to be adjusted among the multiple associated flows in SEALDD-UU interface, and the corresponding required QoS information (i.e. transmission delay). E.g., the SEALDD server evaluates whether the difference of the packet delay measurements for the flows that need to be in synchrony is above the synchronization threshold, and if it is so, the SEALDD server takes the corresponding corrective actions. The SEALDD server may advance or delay the transmission of the DL packets according to the flow assistance information to maintain the synchronization of DL flows. The DL transmission timing advance period correction mode indicates SEALDD server to advance the DL packet transmission by advance period. The DL transmission timing delay period correction mode indicates SEALDD server to delay the transmission of the DL packet by DL delay period units.

The SEALDD server may further update the flow alignment assistance information like DL/UL transmission timing advance or delay period to fine-tune as per the data transmission quality measurement results.

6.	The SEALDD server sends the AF request to 5GC via N33/N5 with the SEALDD traffic descriptor of the adjusted flow(s) (i.e. address/port for the adjusted SEALDD-UU flow) and the corresponding required QoS information determined in step 5, by utilizing the AF session with required QoS procedure in clause 4.15.6.6 of TS 23.502 [6]. The SEALDD traffic descriptor of the adjusted flow(s) contains the address or port in SEALDD server side, and/or SEALDD client side.

NOTE:	This procedure is applicable for both downlink and uplink synchronization of multi-modal flow. For downlink and/or uplink, the step 5 is determined according to the measured downlink and/or uplink data transmission quality measurements in step 4. For uplink synchronization, the SEALDD server performs the caching and transmission to align multi-modal flows based on the flow alignment assistance information before sending the associated packets to the VAL server.

After requesting the transmission quality optimization on 5G network with the required QoS for the adjusted flow(s), the multi-flow synchronization of multi-modal application can be satisfied.

Editor's note: Whether and how the multi-modal flows alignment monitoring will be defined is FFS.

* * * END of Change * * * *