3GPP TSG-SA WG6 Meeting #65	S6-250450

Athens, Greece 17th – 21st February 2025	(revision of S6-250177)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0368 | rev | 1 | Current version: | 19.4.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Uniform the IE description for the velocity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eLSAPP |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The texts for the IE of the velocity information in different information flow is different, but they almost indicate the same meaning in the Description. So it’s needed to uniform the IE description for the velocity information which has the same meaning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Uniform the IE description for the velocity information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Readers may confuse how to use the velocity information in different information flow. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.1, 9.3.2.3, 9.3.2.5, 9.5.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.3.2.1	Location reporting configuration response

Table 9.3.2.1-1 describes the information flow from the location management server to the location management client for the location reporting configuration. This information flow may be sent individually addressed or group addressed on unicast or multicast.

Table 9.3.2.1-1: Location reporting configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or VAL group to which the location reporting configuration is targeted or identity of the VAL UE. |
| Requested location information | O (see NOTE 1) | Identifies what location information is requested |
| Requested location access type (see NOTE 2) | O (see NOTE 1) | Identifies the location access type for which the location information is requested, e.g. as described in TS 23.273 [50] and TS 29.572 [51]. |
| Requested positioning method | O (see NOTE 1) | Identifies the positioning method for which the location information is requested, e.g. as described in TS 23.273 [50] and TS 29.572 [51]. |
| Requested velocity information | O (see NOTE 1) | It indicates the velocity of the requested VAL users/UEs is needed to report. |
| Triggering criteria | O (see NOTE 1) | Identifies when the location management client will send the location report as specified in table 9.3.2.4-2. The triggering may be e.g., SAI changes, ECGI changes, RAT changes. |
| Minimum time between consecutive reports | O (see NOTE 1) | Defaults to 0 if absent otherwise indicates the time interval between consecutive reports |
| NOTE 1: If none of the information element is present, this represents a cancellation for location reporting. NOTE 2: The non-3GPP access as defined in 23.273[50] and TS 29.572 [51] is out of scope of the present specification. |  |  |

* * * Next Change * * * *

9.3.2.3	Location information request

Table 9.3.2.3-1 describes the information flow from the VAL server to the location management server and/or from the location management server to the location management client for requesting an immediate location information report.

Table 9.3.2.3-1: Location information request

| Information element | Status | Description |
| --- | --- | --- |
| Identity list | M | List of VAL users or VAL UEs whose location information is requested |
| VAL service ID | O | Identity of the VAL service for which the location information is requested. |
| Requested location information | O | Identifies what location information is requested |
| Requested location access type (NOTE 4) | O (NOTE 1) | Identifies the location access type for which the location information is requested, e.g. as described in TS 23.273[50] and TS 29.572[51]. |
| Requested positioning method | O (NOTE 1) | Identifies the positioning method for which the location information is requested, e.g. as described in TS 23.273[50] and TS 29.572[51]. If the requested VAL user/UE is not the receiving VAL user/UE hosting the location management client, the UE positioning method includes PC5 SEAL LM, BT, WiFi. |
| Location QoS | O (NOTE 3) | Definition of the location Quality of Service for which the location information is requested (see NOTE 2). |
| Requested velocity indication | O (NOTE 5) | It indicates whether the velocity of the requested VAL users/UEs is needed. |
| List of association IDs | O (NOTE 6) | Identifies list of associations with other UEs may share the location information when they are at the same location. |
| NOTE 1: This e element is only applicable for request sent from the location management server to the location management client. NOTE 2: The definition of location QoS has been defined in clause 4.1b of TS 23.273 [50] and the clause 6.1.6.2.13 of TS 29.572 [51]. NOTE 3: The element is only applicable for the information flow from the VAL server to the location management server. NOTE 4: The non-3GPP access as defined in TS 23.273[50] and TS 29.572[51] is out of scope of the present specification. NOTE 5: The definition of the velocity refers to 3GPP TS 23.271 [60]. NOTE 6: The association IDs are known to the UE based on offline methods and are out of scope of this specification. The requestor can be part of multiple associations and one association can have two or more UEs. |  |  |

* * * Next Change * * * *

9.3.2.5	Location information subscription request

Table 9.3.2.5-1 describes the information flow from the VAL server or location management client to the location management server for location information subscription request or for updating an existing location information subscription.

Table 9.3.2.5-1: Location information subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL user or VAL UE (see NOTE 3) |
| Identities list | M | List of VAL users or VAL UEs whose location information is requested. |
| VAL service ID | O | Identity of the VAL service for which the location information is subscribed. |
| Time between consecutive reports | M | It indicates the interval time between consecutive reports |
| Supplementary location information indication | O | Indicates that supplementary location information is required. |
| Location QoS | O (NOTE 2). | Definition of the location Quality of Service for which the location information is requested (see NOTE 1). |
| Indication for the location data statistic | O | It indicates whether the statistic/calculation of target UE location data is needed per e.g. time/location. |
| Requested time information | O (NOTE 4) | It indicates the time information when the target UE location data is calculated per time. |
| Requested location information | O (NOTE 4) | It indicates the location information when the target UE location data is calculated per location. |
| Requested velocity indication | O | It indicates whether the velocity of the requested VAL users/UEs is needed. |
| NOTE 1: The definition of location QoS has been defined in clause 4.1b of TS 23.273 [50] and the clause 6.1.6.2.13 of TS 29.572 [51]. NOTE 2: The element is only applicable for the information flow from the VAL server to the location management server. NOTE 3: This information element shall not be updated via location information subscription update procedure in clause 9.3.7a. NOTE 4: This information element only applies when the indication for the statistic of target UE location data is included. |  |  |

* * * Next Change * * * *

9.5.2.7	Off-network location request

Table 9.5.2.7-1 describes the information flow from the location management client-1 to the location management client-2 for the off-network location request.

Table 9.5.2.7-1: Off-network location request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user to which the location request is targeted or identity of the VAL UE. |
| Requested location information | M | Identifies what location information is requested |
| Requested velocity indication | O | It indicates whether the velocity of the requested VAL users/UEs is needed. |

* * * End of Changes * * * *