3GPP TSG SA Meeting #107	SP-250245

Incheon, South Korea, March 12 – 14, 2025

Source:	KDDI, SK Telecom

Title:	Discussion on IMS resiliency study

Document for:	Discussion

Agenda Item:	6.4.2

Work Item / Release:	T.B.D / Rel-20

Abstract: This paper provides backgrounds and analyses (issues and potential solutions) for IMS resiliency study.

1. Introduction

The contribution provides backgrounds and analyses (issues and potential solutions) for IMS resiliency study.

For details of each solution mentioned in this paper and its impact on TSs, please refer to the attached draft CRs.

2. Discussion

2.1	Background

From July 2 to July 4, 2022, KDDI experienced a significant communication failure. Failures caused by maintenance work cascaded and nationwide area was affected. The number of affected customers was approximately 22.78 million for Voice (VoLTE) and 7.65 million for Data (4G/5G). The impact extended widely across industries using smartphones and IoT, affecting logistics, automotive, government services, banking, and transportation, thereby influencing essential living infrastructure (see clause 2.2.for detail).

Most MNOs apply the IMS framework for their voice services and will still need to continue operating IMS even in the 6G era. However, large-scale communication failures related to or affected by IMS have occasionally occurred on not few operators’ mobile network in the past few years.

Based on the lessons learned from the significant failure and aiming to achieve stable voice services with IMS, we provide issues and potential solutions for IMS resiliency.

2.2	Overview of KDDI Communication Failure and Response

The communication failure and the response progressed through the following three phases:

Phase 1: Congestion at IMS

The routers were incorrectly configured during maintenance of the nationwide transport network routers at the NC in Tama. The SIP Registration request was abandoned, rapidly triggering numerous retransmissions in network and Tama’s VoLTEs Nodes became congested. The VoLTEs Nodes at other sites became congested because distributed processing was performed via the nationwide transport network.

NOTE:	During this failure, the VoLTEs Nodes were congested, and as a result, the signals from the VoLTEs Nodes were not sent to the HSS. However, if the VoLTEs Nodes had been able to send signals to the HSS, there is a high possibility that the HSS would have received an excessive number of signals and become congested.

Phase 2: Congestion at PCRF

The congestion in the VoLTEs nationwide led to an increase in the retransmission of SIP Registration requests from the UE to the PCRF, resulting in congestion in the PCRF as well. Due to congestion in the PCRF, the PCRF returned error responses to session requests from the PGW. As a result, the UEs that received these error responses retransmitted signals to the PCRF, leading to further and continued congestion in the PCRF. As a result, it became difficult to process SIP Registration and establish voice communication nationwide. Additionally, some UEs are believed to have affected data communication as well.

Phase 3: IMS disconnection (recovery)

In order to reduce congestion on PCRF, flow rate control was implemented, but congestion was not eliminated. Some IMS nodes continued to send excessive signals, so the overloaded status in PCRF continued. The VoLTEs Nodes nodes (6 out of 18 nationwide) were isolated, thus eliminating retransmission from devices.

Source: The July 2 Communication Failure and Our Response

https://www.kddi.com/english/important-news/20220729_01/

2.3	Analysys #1: Efficient authentication

When a UE subscribes to IMS service, it sends a USIM authentication request to 5GC first, then to the IMS second. This means the UDM must process the same authentication twice, causes extra signaling. Even if the service is being provided normally, when congestion occurs and the response from IMS/5GC becomes slow, massive UEs attempt to register to the network in cycles, increasing the load on the UDM and leading to cascade failures. Therefore, as a measure, we aim to reduce the procedures for UDM by even one round-trip. In large-scale failures, signals at the unit of 10 million subscribers may flow into UDM, so by reducing just one step, we can cut down signal processing at the 10 million level and help prevent congestion in UDM. Thus, we propose the following issues and potential solutions for efficient authentication:

2.3.1	Issue #1: Multiple USIM authentications

When a UE subscribes to IMS service, it sends a USIM authentication request to 5GC first (Step 1 and 2), then to the IMS second (Cx-Query/Cx-Select-Pull (Step 4)). This means the UDM must process same authentication twice.

When I-CSCF sends Cx-Query/Cx-Select-Pull request to the UDM, the UDM checks whether the UE is allowed to register in the P-CSCF/ the UE is registered already and sends Cx-Query/Cx-Select-Pull response with the S-CSCF name (Step 4-2) as defined in TS 23.228.

If the I-SCSF knows that the USIM authentication is complete in 5GC or knows the registered S-CSCF name for the UE, the Cx-Query/Cx-Select-Pull procedures can be skipped.

2.3.1.1	Solution #1: Efficient USIM authentication

The solution addresses Issue #1: Multiple USIM authentications.

The following working groups are involved in this solution:

- SA2 for stage 2 procedures aspects, SA3 for the Security aspects, CT1 for NAS/SDP/SIP protocol aspects, CT4 for Cx/Dx/SBI protocol aspects.

For the UE registration procedures in 5GS, the UDM checks the UE registration status for the IMS in step 2. If already registered, the UDM retrieves the registered S-CSCF name for the UE and processes the registered S-CSCF name using one of the following security methods (Step 3) and sends it to the UE via AMF in step 4 and 5.

- Opt.1: Message authentication (e.g. HMAC, CMAC)

- Opt.2: Encryption.

- NOTE: Other options are FFS.

When the UE performs SIP Registration request, it sends the data received from the UE to the P/I-CSCF in step 5. If the I-CSCF can retrieve the S-CSCF name with the security method above and determines the S-CSCF address through a name-address resolution mechanism (Step 6), allowing it to skip the Cx Query/Cx-Select-Pull procedures in step 7.

The impacted TSs are enclosed in red in the table below.

2.3.1.2	Solution #2: Efficient USIM authentication for IMS AKA

The solution addresses Issue #1: Multiple USIM authentications.

The following working groups are involved in this solution:

- SA2 for stage 2 procedures aspects, SA3 for the Security aspects, CT1 for NAS/SDP/SIP protocol aspects, CT4 for Cx/Dx/SBI protocol aspects.

This solution is applicable to non-IPSec environment for IMS AKA (authentication and key agreement). For non-IPSec environment, mutual authentication with CK and IK is not necessary. By utilizing solution #1 (efficient USIM authentication), subsequent authentication procedures can be skipped. If the S-CSCF retrieves the registered S-CSCF name in step 9 with one of the security methods as described in Sol #1, it can identify that the USIM authentication in the 5GC (UDM) is complete, allowing it to skip the subsequent authentication procedures (Step 10-14). If the S-CSCF can identify the USIM authentication in Step 9, the procedure to compare RES and XRES can be skipped (Step 15).

The impacted TSs are enclosed in red in the table below.

2.3.2	Issue #2: Unnecessary procedures in IMS AKA using non-IPSec

The procedures for IMS AKA (authentication and key agreement) are defined in TS 33.203 (detailed in TS 24.228). When the UE initiates IMS registration, the S-CSCF retrieves RAND, XRES, CK and IK from the UDM and performs the IMS AKA procedures with the I/P-CSCF and the UE. However, in the non-IPSec environment, mutual authentication using CK and IK is not necessary.

If the S-CSCF can identify a trusted UE for the registration procedures, it might not be necessary for the S-CSCF to compare RES and XRES (Step 15).

2.3.2.1	Solution #3: Efficient IMS AKA with a proof of USIM auth/ roaming status

The solution addresses Issue #2: Unnecessary procedures in IMS AKA using non-IPSec.

The following working groups are involved in this solution:

- SA2 for stage 2 procedures aspects, SA3 for the Security aspects, CT1 for NAS/SDP/SIP protocol aspects, CT4 for Cx/Dx/SBI protocol aspects.

This solution is applicable to non-IPSec environment and simplifies the process by combining a proof of USIM authentication (i.e., a token valid for about 1 minute) with Home PLMN. If the S-CSCF identifies a proof of USIM authentication and Home PLMN in SIP Register message from I-CSCF in step 7, Cx-AV and subsequent authentication procedures can be skipped.

Trigger to generate the proof of USIM authentication:

- Opt.1: The UDM generates the proof of USIM authentication and sends it with the Registration complete message to the AMF (Step 3-1). The UDM verifies the token in a proof of USIM authentication received in the Cx-Put message against the token held by the UDM (Step 15).

- Opt.2: Upon receiving the Registration accept or TAU message in the UE (Step 3-2), the UE generates the proof of USIM authentication in the SIP Register messages.

When a roaming UE sends a SIP Registration request through the Home PLMN, the authenticity of the UE cannot be verified, leading to security issues. For roaming, there are cases where IMS PDN establishment involves some internet segments, raising the possibility of malicious third parties establishing PDNs. Therefore, there are security concerns, and it is preferable to perform roaming detection and skip authentication within the Home PLMN to mitigate these issues.

The S-CSCF identifies the roaming status by using the combination of the following three methods:

- Identify the MNC/MCC in the PANI header included in the SIP message (Step 7). If the roaming status is Home PLMN, the S-CSCF skips step 8-13. If not, the S-CSCF does not skip them and continues normal procedures.

- Identify the serving network information generated by the UDM to Cx-Pull message (Step 14-2). If the roaming status is Home PLMN, the S-CSCF continues normal procedures. If not, S-CSCF performs error operation.

- Identify the roaming status in User-Data-Request (Step 18). If the roaming status is Home PLMN, the S-CSCF continues normal procedures. If not, S-CSCF performs error operation.

The impacted TSs are enclosed in red in the table below.

2.3.2.2	Solution #4: Efficient IMS AKA with access type/ roaming status

The solution addresses Issue #2: Unnecessary procedures in IMS AKA using non-IPSec.

The following working groups are involved in this solution:

- SA3 for the Security aspects, CT1 for NAS/SDP/SIP protocol aspects, CT4 for Cx/Dx/SBI protocol aspects.

This solution is applicable to non-IPSec environment. Unlike Sol #3, the S-CSCF performs a combination of following methods and when determined as Home PLMN + 3GPP access, the S-CSCF skips subsequent authentication procedures.

- Determine roaming status using the MNC/MCC in the PANI header in the SIP Registration message.

- Determine access network type using PANI header in the SIP Registration message.

Receiving the SIP Register message means that the IMS PDN has been established after successful registration in the 5GS.

If the registration in the 5GS fails and the IMS PDN is not established, the SIP Register message cannot be sent to the S-CSCF. However, it is possible to establish only the IMS PDN as an operator’s policy (without UE registration in the 5GS). In such cases, there is a risk of spoofing, tampering, etc. To avoid this, this solution is used exclusively within the Home PLMN (not used when the UE is in a visited network).

The impacted TSs are enclosed in red in the table below.

2.4	Analysys #2: Mitigating IMS failures

2.4.1	Issue #3: Error handling between the IMS and the UE.

Nowadays, the IMS has no response or returns a response with error code which does not contain the appropriate reason for the failures. If the IMS and/or EPC/5GC providing MMTEL service is under failure condition, UEs may not be able to receive SIP messages and initiate re-attach/re-registration procedure to the EPC/5GC and/or IMS. When the IMS does not respond to the SIP message, the UE cannot determine whether there is an abnormality in the IMS or if there is an issue with data connectivity in the IMS PDN. In such cases, the UE will attempt to re-establish the IMS PDN or re-attach/re-register to the EPC/5GC. This leads the UEs to initiate attach or registration procedure in cycle, causing unexpected overload, accelerating the signaling storm in the network and affecting data services for the EPC/5GC dependent on UE’s implementation.

2.4.1.1	Solution #5: IMS failure notification to UE with IMSvoPS

The following working groups are involved in this solution:

- SA2 for stage 2 procedures aspects, CT1 for NAS/SDP/SIP protocol aspects, CT4 for Cx/Dx/SBI protocol aspects.

The following methods are envisaged for notifying IMS-related failure from EPS/5GS to UE:

- C-Plane: NAS

  - MM: IMSvoPS (IMS voice over PS Session Indicator)

  - SM: PCO/ePCO

- C-Plane: RRC

  - SIB

- U-Plane: ICMP/ICMPv6, SIP, DHCP, DNS

This solution provides analysys for IMS failure notification by using IMSvoPS.

IMSvoPS (IMS voice over PS Session Indicator) is defined as TS 24.292. The behavior when this flag indicates "Not Supported" is specified in clause 5.76 of TS 24.167.

5.76	/<X>/IMS_Registration_Policy/<X>/Stay_Registered_When_VoPS_Not_Supported

The leaf indicates whether the UE stays registered or not from IMS when moving to area where IMS Voice Over PS Session is not supported.

-	Occurrence: One

-	Format: bool

-	Access Types: Get, Replace

-	Values: 0, 1

1 - Indicates that the UE maintains the registration as required for IMS services;

0 - Indicates that the UE deregisters from IMS as defined in annex B, annex L, annex U and annex W in 3GPP TS 24.229 [5].

NOTE:	The value 1 implicitly indicates that there is no services other than voice is to be provided based on IMS, thus if the value 1 is set then the UE does not perform the initial IMS registration when camps on the network and gets the indication that IMS voice over PS session is not supported.

It indicates whether the UE should stay registered or not from IMS when moving to an area where IMS Voice Over PS Session is not supported. Additionally, the indicator is not designed to be conveyed via a network trigger (such as TAU/Reg Update triggers). Furthermore, if a UE applies to this indicator, it is transmitted to the MME/AMF via HSS/UDM, increasing the load on the HSS/UDM and potentially leading to cascade failures.

Thus, the current IMSvoPS mechanism does not include the feature to prevent initial IMS registration.

2.4.1.2	Solution #6: IMS failure/recovery notification to UE with PCO/ePCO

The following working groups are involved in this solution:

- SA2 for stage 2 procedures aspects, CT1 for NAS/SDP/SIP protocol aspects, CT4 for Cx/Dx/SBI protocol and P-CSCF Restoration aspects.

The following methods are envisaged for notifying IMS-related failure from EPS/5GS to UE:

- C-Plane: NAS

  - MM: IMSvoPS (IMS voice over PS Session Indicator)

  - SM: PCO/ePCO

- C-Plane: RRC

  - SIB

- U-Plane: ICMP/ICMPv6, SIP, DHCP, DNS

This solution provides IMS failure/recovery notification by using PCO/ePCO.

The P-CSCF Restoration mechanism is defined in TS 23.380 and TS 24.229. There are two methods for the UE to obtain the P-CSCF address: (1) using DHCP, and (2) using PCO/ePCO. However, the only way to inform about the unavailability of the P-CSCF address is through PCO/ePCO. The method with PCO/ePCO has no impact on the MME or SGW, as the PCO/ePCO information is transparently transferred through these network elements.

The current PCO/ePCO shows only three IEs as defined in clause 10.5.6.3 (Protocol configuration options) of TS 24.008:

- 0001H (P-CSCF IPv6 Address Request), 000CH (P-CSCF IPv4 Address Request): P-CSCF IP address. If there is no valid P-CSCF address, these IEs are not applicable.

- 0012H (P-CSCF Re-selection support): Inform the UE of the valid P-CSCF address. Inform the UE of the P-CSCF Re-selection support.

It is desirable to define new IEs in PCO/ePCO to enable the EPC/5GC to inform the UE about the suppression and resumption of IMS registration.

We propose a potential solution with PCO/ePCO with following steps:

NOTE: This solution is based on EPC, but the corresponding procedures can also be applied to 5GC.

- How the PGW detects IMS failure/recovery?

The following three options are assumed. It is possible to use a combination of these three options, or to use just one according to operator policy.

Option.1-1: Notification via PCRF.

- The PCRF determines the IMS failure/recovery based on the status of Rx (Diameter) and sends it to the PGW.

- Bulk operation: As indicated in the above figures, the P-CSCF and the PCRF are connected via Rx interface. "When the Rx connection is lost and then recovered, the PCRF can notify the PGW about the status of the P-CSCF, indicating whether it is down or up. The issue is whether to notify from the PCRF to the PGW on a per-Gx connection basis or on a bulk basis. The P-CSCF Restoration mechanism can handle per-Gx connection notifications. However, the bulk notification is not specified. When the P-CSCF goes down, handling notifications on a per-Gx connection basis can be burdensome for the PCRF. To prevent one entity from being overloaded, we propose to define new IEs for IMS failure/recovery. When the PCRF/PCF detects the IMS failure/recovery, the PCRF/PCF can send the IMS failure/recovery to the PDN GW. The detail of the Bulk operation is FFS.

Option.1-2: Detect the IMS failure/recovery through liveness monitoring.

- The PCRF conducts liveness monitoring on the P-CSCF (e.g, ping)

NOTE: The monitoring configuration of the PGW depends on operator and implementation policy.

Option.1-3: Manual operation by the Operator.

- How to notify UE of IMS failure?

The following two options are assumed.

Option.2-1: Define new dedicated IEs

The following new IEs for PCOs need be defined in clause 10.5.6.3 (Protocol configuration options) of TS 24.008.

- Request Suppression Timer: The IE is used to set the time until the UE initiates a SIP Registration Request from the UE. The IE is used to not only suppresse SIP Registration from the UE for a certain period but also prevent deadlock in the event that the recovery notification is dropped. Even when this timer is running, if the IMS PDN is disconnected from the network side, the UE will clear this timer and release the suppression of SIP Registration.

- IMS status: The IE is used to notify that the IMS is down and unavailable (e.g., Not applicable).

It is also necessary to clearly define the behavior of the UE upon receiving this information. In the event of IMS failure, the UE will try to prioritize the restoration of MMTEL service and attempt initial registration, but this action should be suppressed.

Option.2-2: Notify the P-CSCF address as Null

As stated above, the current PCO/ePCO standards specify informing the UE of the valid P-CSCF address. If there is no valid P-CSCF address, the IE is not applicable. Thus, we propose to enhance the following IEs to support Null. If the PCO value of the following IEs set to Null, the UE can recognize that the IMS is not applicable.

- 0001H (P-CSCF IPv6 Address):  000CH (P-CSCF IPv4 Address): P-CSCF IP addresss.

It is also necessary to clearly define the behavior of the UE upon receiving this information. In the event of IMS failure, the UE will try to prioritize the restoration of MMTEL service and attempt initial registration, but this action should be suppressed.

Regardless of the options above, it is necessary to define a new IE to notify whether the UE supports the feature or not. If the network sends the notification to the UE which does not support the option, the UE may cause unexpected behavior. Thus, it may be safer to apply these options based on the response from the UE. We propose to define the following new IE in clause 10.5.6.3 (Protocol configuration options) of TS 24.008. This optional extension is based on the possibility for the P-GW to identify whether or not the UE supports the new IE. The UE shall indicate this capability to the P-GW at the activation of the IMS PDN connection in the new IE in the PCO parameter. The P-GW shall store this UE capability.

- 000xx (UE support IMS status indication): Inform the UE of the IMS status support.

- How to notify UE of IMS Recovery?

The following two options are assumed. In any pattern, notify at a rate that considers the load on IMS and HSS (depending on implementation/operator policy)

Option.3-1: Define a new dedicated IE

If we apply option 2-1, the following new IEs can be defined in clause 10.5.6.3 (Protocol configuration options) of TS 24.008. When the IMS is recovered and available, the PGW informs the UE of the IEs.

- Recovery timer: Set the timing for resuming the SIP Registration from the UE. This timer is designed to stagger the timing of each UE's Registration requests, preventing all UEs from making requests simultaneously (e.g., a random value to avoid simultaneous requests).

- A new value (e.g., available) is added to the IMS status IE which is described in option 2-1: IMS status: It is used to notify that the IMS is recovered and available.

Option.3-2: Notify the valid P-CSCF address

If we apply option 2-2, we can reuse the following IEs as defined in clause 10.5.6.3 (Protocol configuration options) of TS 24.008. When the IMS is recovered and available, the PGW informs the UE of the valid P-CSCF address.

- 0001H (P-CSCF IPv6 Address Request):  000CH (P-CSCF IPv4 Address Request): P-CSCF IP address.

In order to avoid simultaneous SIP registration requests from UE, the PGW may send the valid P-CSCF address to each UE at staggered times based on operator policy.

Option. 3-3: Disconnect the IMS PDN

This option is reused to the P-CSCF restoration mechanism as defined in TS 23.380, allowing the IMS PDN reconnection and registration triggered by the IMS PDN disconnection. The advantage of this option is that if the PCRF has established junk sessions, it can prompt the UE to reconnect according to the operator's policy.

If the PCRF applies the Bulk operation as described in Option 1-1, the PGW or the PCRF needs to determine which bearers to disconnect and make decisions to disconnect them at a pre-set rate. It needs to be decided whether to have the PCF handle this or the SMF. This is FFS.

- How to prevent Deadlock?

Due to some issues, the recovery notification as described in option 3-1 and option 3-2 was dropped, or the UE does not send a SIP Registration. There are two methods to prevent the situation. These options can be combined based on the deadlock status.

Option.4-1: Re-establish the IMS PDN connection.

- When the Request Suppression Timer as described in option 2-1 expires, the IMS PDN connection will be re-established using option 3-3. Then, the UE obtains the latest status of the IMS and PCRF.

Option.4-2: New IE can be defined in clause 10.5.6.3 (Protocol configuration options) of TS 24.008.

- Ack required: If the PGW sends a notification of the IMS Recovery, it may include this IE in PCO. If the UE receives the notification from the PGW, it shall send an ACK message.

2.4.1.3	Solution #7: IMS failure notification to UE with SIB

The following working groups are involved in this solution:

- SA2 for stage 2 procedures aspects, CT1 for NAS/SDP/SIP protocol aspects, CT4 for Cx/Dx/SBI protocol aspects.

The following methods are envisaged for notifying IMS-related failure from EPS/5GS to UE:

- C-Plane: NAS

  - MM: IMSvoPS (IMS voice over PS Session Indicator)

  - SM: PCO/ePCO

- C-Plane: RRC

  - SIB

- U-Plane: ICMP/ICMPv6, SIP, DHCP, DNS

This solution provides analysys for IMS failure notification by using SIB.

There are two methods to communicate SIB to UE:

- via EMS of eNB/gNB

- via MME/AMF

Communicating the IMS status to eNB/gNB in real-time presents significant technical challenges. In case of communicating via MME/AMF, it is preferable to avoid using HSS/UDM.

The granularity of control becomes at the PLMN level, making fine-grained control at the UE level difficult. Although call barring (radio restrictions) is defined in 3GPP standards, further investigation is needed to determine the extent to which SIP registration to IMS can be suppressed. Due to the large protocol disparity between the RRC layer and the SIP layer, the implementation impact on the UE is likely to be substantial.

This solution is not applicable to roaming scenarios.

2.4.1.4	Solution #8 for Issue #3: IMS failure notification to UE with U-Plane signaling

The following working groups are involved in this solution:

- SA2 for stage 2 procedures aspects, CT1 for NAS/SDP/SIP protocol aspects, CT4 for Cx/Dx/SBI protocol aspects.

The following methods are envisaged for notifying IMS-related failure from EPS/5GS to UE:

- C-Plane: NAS

  - MM: IMSvoPS (IMS voice over PS Session Indicator)

  - SM: PCO/ePCO

- C-Plane: RRC

  - SIB

- U-Plane: ICMP/ICMPv6, SIP, DHCP, DNS

This solution provides analysys for IMS failure notification by using U-Plane signaling such as ICMP/ICMPv6, SIP, DHCP, DNS.

ICMP/ICMPv6

Using ICMP, in cases where the P-CSCF cannot respond to the SIP Registration, the PGW/P-SCSF can be triggered to notice by returning ICMP errors (e.g. Host Unreachable, Port Unreachable). The SIP is strictly defined by RFC, which is no provision for handling ICMP errors to suppress registration. It would be necessary for 3GPP standard to make an independent interpretation and modify the RFC for SIP control. This presents a significant challenge.

The SIP RFC does not support a mechanism to handle ICMP/ICMPv6 Errors to suppress SIP Registration. Although it is possible to return an error to SIP messages from the UE to the IMS, it is not possible to push ICMP Errors from the PGW/IMS to the UE. If push notifications cannot be used, it becomes difficult to quickly notify IMS recovery (lifting suppression). To ensure that the UE quickly recognizes IMS recovery (lifting suppression), it would need to poll at relatively high frequencies.

SIP

In clause 5.1.1.2.1 of TS 24.229, the UE behaviors when receiving a SIP failure response during initial registration are specified as follows:

- 423 (Interval Too Brief) with Min-Expires header:  The UE sends another SIP Registration Request with an expire timer value greater than the value of Min-Expires header field of the 423 response.

- 408 (Request Timeout), 500 (Server Internal Error), 504 (Server Time-Out), 600 (Busy Everywhere), 403 (Forbidden): UE resends the SIP Registration.

- 503 (Service Unavailable) with Retry-After header: The UE attempts an initial SIP registration after the time indicated in the Retry-After header field has elapsed.

The method of using SIP has been previously researched, and there is a mechanism to return various error codes. However, how the UE interprets SIP errors depends on UE/chipset vendors’ implementation. Following NOTE in clause 5.1.1.2.1 of TS 24.229 shows that the initial SIP registration from the UE with network again is based on the local configuration and outside the scope.

NOTE 19: If the UE stops initiating initial registration, the UE can attempt to perform initial registration with the network again based on the local configuration and not wait till power-cycled. The local configuration is outside the scope of this specification.

DHCPv4/v6

This is a method for responding to the UE with the FQDN/ IP address of the P-CSCF. However, there is no mechanism to invalidate the FQDN/ IP address with server push (A combination with PCO/ePCO is required). It is not possible to push error from the IMS to the UE and informing the UE of the recovery notification is difficult.

DNS

This is used to respond with the FQDN of the P-CSCF in the DHCP method. Similarly to the DHCP method, there is no mechanism to invalidate the P-CSCF address with server push.

2.4.1.5	Solution #9 for Issue #3: PCRF failure/recovery notification to UE with PCO/ePCO

The following working groups are involved in this solution:

- SA2 for stage 2 procedures aspects, CT1 for NAS/SDP/SIP protocol aspects, CT4 for Cx/Dx/SBI protocol and P-CSCF Restoration aspects.

This solution provides analysys for PCRF failure/recovery notification by using PCO/ePCO. We reuse the method described in clause 2.4.1.2 with the solution.

When the PCRF is down, the Gx connection is lost. If the IMS PDN connection is maintained and the PGW notifies through the new dedicated IEs in the PCO as described in option.2-1 or option.2-2 of clause2.4.1.2, it is possible to prevent frequent SIP Registration from the UE and the load on the PGW and PCRF can be mitigated.

It is also necessary to clearly define the behavior of the UE upon receiving this information. In the event of PCRF failure, the UE will try to prioritize the restoration of MMTEL service and attempt initial registration, but this action should be suppressed.

When the PCRF is recovered and the Gx connection is up, the PGW initiates to disconnect and re-establish the IMS PDN.

2.5.	Analyses #3: Network congestion control

Editor's note:	The solution for issue #4 is FFS.

2.5.1	Issue #4: Network congestion control

For 5GS/EPS deployment, NAS level congestion control is achieved by providing a Back-off Timer and/or cause values to UEs. However, following issues are identified:

- UEs need to be switched OFF/ON after receiving a Back-off Timer IE from a network (incl, IMS). So, the network using a Back off Timer does not solve the issue of the manual OFF/ON caused by the variety of UE implementations. If UEs receive a Back-off Timer IE, how the UEs apply to the Back-off Timer dependent on vendor’s implementation. The network also needs to identify the UE's implemented release to assess their support of a Back off Timer.

- The flexibility allowed by 3GPP standards for vendors to decide on signaling behavior after receiving cause values can lead to inconsistent UE behavior, some of which a mobile operator does not expect.

3. Conclusions

Based on the analyses in clause 2, we propose to study following objectives.

-	WT#1: Study whether and how the UDM provides single authentication between UEs and the 5GC and the IMS during the registration procedure.

-	To mitigate signaling load, efficient authentication procedure (e.g., unified procedure, simplification) to be studied.

-	WT2: Study whether and how 5GC notify the IMS status indicating failure or recovery conditions to UEs to avoid signaling storms and to mitigate IMS failure.

-	To avoid unnecessary registrations during IMS failure or 5GC failure.

-	Appropriate error handling between the IMS and the UE.

-	WT3: Study whether and how the 5GC in coordination with the UE configure the signaling frequency and timing of signaling based on the operator’s policy to mitigate negative impacts on the 5GC and/or the IMS.

-	The 5GC and/or the IMS notifies UEs the network status when the 5GC and/or the IMS is in the failure condition.

-	The UE configures signaling settings e.g., maximum allowed number of signaling events, timing of signaling, based on the notification.