TSG-SA Meeting #107	SP-250323

Incheon, South Korea, Mar 12 – 14, 2025

Agenda Item:		21.5

Source:				China Mobile

Title:			Summary for WI System architecture for Next Generation Real time Communication services (NG_RTC)

WI code(s):			940066, 970014, 990023, 990086, 990087, 1030027

leading WG:		SA2

Release:				Rel-18

This contribution provides WI summary of general aspects on related work for "Next Generation Real Time Communication services" which is captured in clause 17.4 of TR 21.918.

The summary covers the following aspects:

| 940066 | Study on system architecture for next generation real time communication services | FS_NG_RTC | S2 | SP-220288 | Yi Jiang, China Mobile |
| --- | --- | --- | --- | --- | --- |
| 970014 | System architecture for Next Generation Real time Communication services | NG_RTC | S2 | SP-230098 | Yi Jiang, China Mobile |
| 990023 | CT1 aspects of NG_RTC | NG_RTC | C1 | CP-240271 | Chen Xu, China Mobile |
| 990086 | CT3 aspects of NG_RTC | NG_RTC | C3 | CP-240271 | Chen Xu, China Mobile |
| 990087 | CT4 aspects of NG_RTC | NG_RTC | C4 | CP-240271 | Chen Xu, China Mobile |
| 1030027 | CT6 aspects of NG_RTC | NG_RTC | C6 | CP-240271 | Chen Xu, China Mobile |
| 990038 | Security support for Next Generation Real Time Communication services | NG_RTC_SEC | S3 | SP-230151 | Fei, Li, Huawei |
| 960032 | Study on security support for Next Generation Real Time Communication services | FS_NG_RTC_SEC | S3 | SP-220530 | Fei, Li, Huawei |

### 1	Introduction

In Study Item FS_NG_RTC (ID 940066), SA2 studied the system architecture enhancements to support next generation real time communication services, with the following key issues:

- KI#1: Enhancement to support Data Channel usage in IMS network

- KI#2: IMS based AR telephony communication

- KI#3: Third party specific user identities

- KI#4: Study of Applicability of Service based principles to IMS media control interfaces

SA2 has concluded the above key issues which was captured in TR 23.700-87. It is concluded that KI#3 will not progress in R18.

Based on the conclusion of FS_NG_RTC, the Work Item System architecture for Next Generation Real time Communication services (ID 970014) introduces concept of IMS Data Channel and enhances the IMS architecture and procedures to support management of Data Channel in IMS network. Further more, the WI also specifies the procedures to provide AR communication based on IMS Data Channel.

The stage 3 WIs (ID 990023, 990086 and 990087) specifies the corresponding protocols of supporting IMS Data Channel and AR communications.

2	Description

The following aspects are specified in Rel-18:

- Enhanced IMS architecture to support IMS Data Channel

- The enhanced IMS architecture introduces signalling function,i.e. Data Channel Signalling Function (DCSF), and media function, i.e. Media Function (MF), supporting data channel services. DCSF provides data channel policy control and data channel media control. The MF provides media capabilities in support of IMS DC and Augmented Reality.

- DC capability negotiation

The UE supporting IMS data channel includes the media feature tag in the initial REGISTER request and any subsequent REGISTER request to allow the home IMS network to discover its IMS data channel capability. If the IMS network supports IMS data channel, the S-CSCF includes a Feature-Caps header field indicating its data channel capability in the 200 OK response to the initial and any subsequent REGISTER request.

- Procedures of IMS DC management

- The IMS network supports the end-to-end procedures supporting establishment, modification and termination for bootstrap DC and application DC management in both P2P and A2P/P2A/P2A2P scenarios.

- NF Services supporting IMS DC

- The NRF services for service registration, discovery and selection used on IMS media control architecture and interfaces are enhanced to support capabilities of DCSF and MF. Services of IMS AS and MF is defined to support interactions between DCSF, IMS AS and MF for IMS DC management.

- AR telephony communication

- The IMS network supports procedure enhancements to support AR telephony communication under both UE rendering mode and network rendering mode, on top of DC architecture.

- Interactions with existing supplementary services

The interactions between IMS Data Channel and the media related supplementary services of the MMTel(e.g. CDIV, HOLD, CONF) are identified, and additional requirements for the IMS network and the UE supporting IMS data channel are specified to support the DC management during services procedures.

References

[1]	TR 23.700-87: "Study on system architecture enhancement for next generation real time communication; Phase 2".

[2]	TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[3]	TS 24.229: "IP Multimedia Call Control based on SIP and SDP; Stage 3".

[4]	TS 24.186: "IMS Data Channel applications; Protocol specification".

[5]	SP-230098: "WID update: System architecture for Next Generation Real time Communication services".

[6]	CP-240271: "Revised WID on Next Generation Real Time Communication services".

[7]	TS 29.175: "IP Multimedia Subsystem; IP Multimedia Subsystem (IMS) Application Server (AS) Services; Stage 3".

[8]	TS 29.176: "IP Multimedia Subsystems (IMS); Media Function (MF) Services; Stage 3".

[9]	TS 29.330: " IP Multimedia Subsystem (IMS); Sc Interface based on the Diameter protocol".