TSG SA Meeting #SP-108	SP-250429

10 - 13 June 2025, Prague, Czech Republic

3GPP TSG-SA2 Meeting #168	S2-2504468

(revision of S2-2504393)

07 - 11 April, 2025, Goteborg

# Title:	Reply LS on OSAppID usage by AppToken use case

# Response to:	LS (S2-2502797) on OSAppID usage by AppToken use case

# Release:	Rel-19

# Work Item:	TEI19

Source:	SA WG2

To:	TSG SA

Cc:	SA WG3

Contact Person:

Fernando Pascual Blanco

fernando.pascualblanco at telefonica.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 thanks GSMA TSGNS for their LS on OSAppID usage by AppToken use case.

SA2 provides the following updates to GSMA TSGNS.

TSGNS Questions:

- Do the existing 3GPP specifications allow the network to use application identities that are generated by an application backend to populate URSP in step 2?

SA2 Answer:

SA2 has not defined how the application identities are generated. Based on the OSAppId definition in TS 23.503, the uniqueness of OSAppIds is required and it is the responsibility of the specific implementation.

- Would 3GPP SA2 prefer an alternate mechanism to preserve end user privacy?

SA2 Answer:

We recommend SA3 answering the user privacy aspect.

- Consider the architectural implications of the proposed mechanism and provide feedback.

SA2 Answer:

SA2 could not reach consensus on the answer to this question.

2. Actions:

To SA.

ACTION:

SA2 kindly asks SA to take above SA2 information into account and accordingly to respond GSMA TSGNS LS that was received in TSGNS44_003v6 (S2-2502797).

3. Date of Next SA2 Meetings:

|  | DATES | LOCATION | CITY |
| --- | --- | --- | --- |
| SA2#169: | 19 - 23 May, 2025 | Fukuoka | Japan |
| SA2#170: | 25 - 29 Aug, 2025 | Goteborg | Sweden |

SA2 Meeting calendar can be found at:

https://portal.3gpp.org/Meetings?tbid=375&SubTB=385#/