3GPP TSG-SA5 Meeting #161	S5-252922

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0235 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.554 Add the calculation of carbon emission related KPIs for gNB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | Energy_OAM_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Renewable energy usage for gNB is discussed and concluded in TR 28.880. A solution of carbon emission calculation is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce carbon emission calculation for gNB |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The solution is imcomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.m |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

6.7.m	gNB Estimated Carbon Emission (ECE)

6.7.m.1	Definition

a)	ECEgNB

b)	This KPI provides the Estimated Carbon Emission of a gNB (ECEgNB) over a time period. The Estimated Carbon Emission of a gNB (ECEgNB) is the Energy Consumption of the gNB (ECgNB) multiplied by the Carbon Emission Factor of the energy source which powers the gNB (CEF). The CEF of the energy supply is the sum of all CEFs of its energy sources as configured in energySourceCef (see TS 28.310 [9]). The KPI object, i.e., ManagedElement is associated with EnergySupplyInfo through IOC EnergyInfoGroup as described in TS 28.310[9]. The unit of ECEgNB is kg CO2eq.

c)	This calculation formula is obtained as:

ECEgNB=ECgNB*CEF

, where:

ECgNB is the Energy Consumption of the gNB as defined in TS 28.554 clause 6.7.3.4.2. Its unit is kWh;

CEFgNB indicates Carbon Emission Factor. The CEF of the energy supply is the sum of all CEFs of its energy sources as configured in energySourceCef (see TS 28.310 [9]). Its unit is kg CO2eq/kWh.

d) ManagedElement

NOTE: This KPI is applicable for the gNBs that are powered using single energy supply.  The accuracy of this KPI is dependent on the accuracy of the CEF information configured by the operator.

End of change