3GPP TSG-SA Meeting   #108	SP-250568

# Prague, CZ, 10th–13th June 2025

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.532 | CR | 0015 | rev |  | Current version: | 18.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of info and externalDocs fields |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |
| Source to TSG: | BBC |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  | 2025-05-29 |
| Category: | F |  |  |  |  |  | Release: |  | Rel-18 |
| Reason for change: |  | API version number of TS29571_CommonData.yaml is incremented. |  |  |  |  |  |  |  |
| Summary of change: |  | Version number of all APIs referencing above file is incremented. |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent version numbering across API release tag “TSG108-Rel18”. |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.1, B.2, B.3, B.4 |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 29.571 CR0637 |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |
| Other comments: |  | Administrative change arising from the related CR cited above. |  |  |  |  |  |  |  |
| This CR's revision history: |  | CR0015 [SP-250568]: Contribution to TSG Plenary. |  |  |  |  |  |  |  |

Change

# B.1	General

This annex is based on the OpenAPI 3.0.0 specification [16] and provides corresponding representations of all APIs defined in the present document.

NOTE 1:	An OpenAPIs representation embeds JSON Schema representations of HTTP message bodies.

This annex shall take precedence when being discrepant to other parts of the present document with respect to the encoding of information elements and methods within the API(s).

NOTE 2:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

# B.2	Data types applicable to multiple services

For the purpose of referencing entities defined in this clause, it shall be assumed that the OpenAPI definitions below are contained in a physical file named "TS26532_CommonData.yaml".

openapi: 3.0.0 info: title: Data Collection and Reporting Common Data Types version: 2.1.1 description: ¦ Data Collection and Reporting Common Data Types © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: Data Collection and Reporting Common Data Types description: 'Data Collection and Reporting: Common Data Types' externalDocs: description: 'TS 26.532 V18.5.0; Data Collection and Reporting; Protocols and Formats' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.532/' paths: {} components: schemas: ################################# # Clause 5.4.1: Simple data types ################################# ##################################### # Clause 5.4.2: Structured data types ##################################### DataSamplingRule: description: "Instructions on how UE data is to be sampled by the data collection client." type: object properties: contextIds: readOnly: true type: array minItems: 1 items: type: string minLength: 1 samplingPeriod: $ref: 'TS29571_CommonData.yaml#/components/schemas/Float' locationFilter: $ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G' required: - contextIds DataReportingCondition: description: "A condition that triggers data reporting by a data collection client to the Data Collection AF." type: object properties: contextIds: readOnly: true type: array minItems: 1 items: type: string minLength: 1 type: $ref: '#/components/schemas/DataReportingConditionType' period: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' parameter: type: string threshold: anyOf: - $ref: 'TS29571_CommonData.yaml#/components/schemas/Double' - $ref: 'TS29571_CommonData.yaml#/components/schemas/Float' - $ref: 'TS29571_CommonData.yaml#/components/schemas/Int32' - $ref: 'TS29571_CommonData.yaml#/components/schemas/Int64' - $ref: 'TS29571_CommonData.yaml#/components/schemas/Uint16' - $ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32' - $ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64' - $ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger' reportWhenBelow: type: boolean eventTrigger: $ref: '#/components/schemas/DataReportingEventTrigger' required: - contextIds - type DataReportingRule: description: "Instructions on how collected UE data is to be reported by the data collection client to the Data Collection AF." type: object properties: contextIds: readOnly: true type: array minItems: 1 items: type: string minLength: 1 reportingProbability: $ref: 'TS26510_CommonData.yaml#/components/schemas/Percentage' reportingFormat: deprecated: true $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' dataPackagingStrategy: type: string required: - contextIds ##################################### # Clause 5.4.3: Enumerated data types ##################################### DataCollectionClientType: description: "Indicating a type of data collection client that reports UE data to the Data Collection AF." anyOf: - type: string enum: - DIRECT - INDIRECT - APPLICATION_SERVER - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. DataReportingConditionType: description: "The type of condition that triggers reporting by a data collection client to the Data Collection AF." anyOf: - type: string enum: - INTERVAL - THRESHOLD - EVENT - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. DataReportingEventTrigger: description: "The type of event that triggers reporting by a data collection client to the Data Collection AF." anyOf: - type: string enum: - LOCATION - DESTINATION - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API.

# B.3	Ndcaf_DataReportingProvisioning service API

For the purpose of referencing entities defined in this clause, it shall be assumed that the OpenAPI definitions below are contained in a physical file named "TS26532_Ndcaf_DataReportingProvisioning.yaml".

openapi: 3.0.0 info: title: Ndcaf_DataReportingProvisioning version: 2.0.1 description: ¦ Data Collection AF: Provisioning Sessions API © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: Ndcaf_DataReportingProvisioning description: 'Data Collection and Reporting: Application Service Provider Provisioning (R1) APIs' externalDocs: description: 'TS 26.532 V18.5.0; Data Collection and Reporting; Protocols and Formats' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.532/' servers: - url: '{apiRoot}/3gpp-ndcaf_data-reporting-provisioning/v1' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.532 clause 5.2. security: - {} - oAuth2ClientCredentials: [] paths: /sessions: post: operationId: CreateSession summary: 'Create a new Data Reporting Provisioning Session' requestBody: required: true content: application/json: schema: $ref: '#/components/schemas/DataReportingProvisioningSession' responses: '201': description: 'Data Reporting Provisioning Session successfully created' headers: Location: description: 'URL including the resource identifier of the newly created Data Reporting Provisioning Session.' required: true schema: $ref: 'TS26510_CommonData.yaml#/components/schemas/Url' content: application/json: schema: $ref: '#/components/schemas/DataReportingProvisioningSession' '400': $ref: 'TS29571_CommonData.yaml#/components/responses/400' '401': $ref: 'TS29571_CommonData.yaml#/components/responses/401' '403': $ref: 'TS29571_CommonData.yaml#/components/responses/403' '404': $ref: 'TS29571_CommonData.yaml#/components/responses/404' '411': $ref: 'TS29571_CommonData.yaml#/components/responses/411' '413': $ref: 'TS29571_CommonData.yaml#/components/responses/413' '415': $ref: 'TS29571_CommonData.yaml#/components/responses/415' '429': $ref: 'TS29571_CommonData.yaml#/components/responses/429' '500': $ref: 'TS29571_CommonData.yaml#/components/responses/500' '503': $ref: 'TS29571_CommonData.yaml#/components/responses/503' default: $ref: 'TS29571_CommonData.yaml#/components/responses/default' /sessions/{sessionId}: parameters: - name: sessionId in: path required: true schema: $ref: 'TS26510_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Data Reporting Provisioning Session.' get: operationId: RetrieveSession summary: 'Retrieve an existing Data Reporting Provisioning Session' responses: '200': description: 'Representation of Data Reporting Provisioning Session is returned' content: application/json: schema: $ref: '#/components/schemas/DataReportingProvisioningSession' '307': $ref: 'TS29571_CommonData.yaml#/components/responses/307' '308': $ref: 'TS29571_CommonData.yaml#/components/responses/308' '400': $ref: 'TS29571_CommonData.yaml#/components/responses/400' '401': $ref: 'TS29571_CommonData.yaml#/components/responses/401' '403': $ref: 'TS29571_CommonData.yaml#/components/responses/403' '404': $ref: 'TS29571_CommonData.yaml#/components/responses/404' '406': $ref: 'TS29571_CommonData.yaml#/components/responses/406' '429': $ref: 'TS29571_CommonData.yaml#/components/responses/429' '500': $ref: 'TS29571_CommonData.yaml#/components/responses/500' '503': $ref: 'TS29571_CommonData.yaml#/components/responses/503' default: $ref: 'TS29571_CommonData.yaml#/components/responses/default' delete: operationId: DestroySession summary: 'Destroy an existing Data Reporting Provisioning Session' responses: '204': description: 'Data Reporting Provisioning Session resource successfully destroyed' # No Content '307': $ref: 'TS29571_CommonData.yaml#/components/responses/307' '308': $ref: 'TS29571_CommonData.yaml#/components/responses/308' '400': $ref: 'TS29571_CommonData.yaml#/components/responses/400' '401': $ref: 'TS29571_CommonData.yaml#/components/responses/401' '403': $ref: 'TS29571_CommonData.yaml#/components/responses/403' '404': $ref: 'TS29571_CommonData.yaml#/components/responses/404' '429': $ref: 'TS29571_CommonData.yaml#/components/responses/429' '500': $ref: 'TS29571_CommonData.yaml#/components/responses/500' '503': $ref: 'TS29571_CommonData.yaml#/components/responses/503' default: $ref: 'TS29571_CommonData.yaml#/components/responses/default' /sessions/{sessionId}/configurations: parameters: - name: sessionId in: path required: true schema: $ref: 'TS26510_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Data Reporting Provisioning Session.' post: operationId: CreateConfiguration summary: 'Create a new Data Reporting Configuration subresource within the scope of an existing Data Reporting Provisioning Session' requestBody: required: true content: application/json: schema: $ref: '#/components/schemas/DataReportingConfiguration' responses: '201': description: 'Data Reporting Configuration successfully created' headers: Location: description: 'URL including the resource identifier of the newly created Data Reporting Configuration.' required: true schema: $ref: 'TS26510_CommonData.yaml#/components/schemas/Url' content: application/json: schema: $ref: '#/components/schemas/DataReportingConfiguration' '400': $ref: 'TS29571_CommonData.yaml#/components/responses/400' '401': $ref: 'TS29571_CommonData.yaml#/components/responses/401' '403': $ref: 'TS29571_CommonData.yaml#/components/responses/403' '404': $ref: 'TS29571_CommonData.yaml#/components/responses/404' '411': $ref: 'TS29571_CommonData.yaml#/components/responses/411' '413': $ref: 'TS29571_CommonData.yaml#/components/responses/413' '415': $ref: 'TS29571_CommonData.yaml#/components/responses/415' '429': $ref: 'TS29571_CommonData.yaml#/components/responses/429' '500': $ref: 'TS29571_CommonData.yaml#/components/responses/500' '503': $ref: 'TS29571_CommonData.yaml#/components/responses/503' default: $ref: 'TS29571_CommonData.yaml#/components/responses/default' /sessions/{sessionId}/configurations/{configurationId}: parameters: - name: sessionId in: path required: true schema: $ref: 'TS26510_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Data Reporting Provisioning Session.' - name: configurationId in: path required: true schema: $ref: 'TS26510_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Data Reporting Configuration.' get: operationId: RetrieveConfiguration summary: 'Retrieve an existing Data Reporting Configuration' responses: '200': description: 'Representation of Data Reporting Configuration is returned' content: application/json: schema: $ref: '#/components/schemas/DataReportingConfiguration' '307': $ref: 'TS29571_CommonData.yaml#/components/responses/307' '308': $ref: 'TS29571_CommonData.yaml#/components/responses/308' '400': $ref: 'TS29571_CommonData.yaml#/components/responses/400' '401': $ref: 'TS29571_CommonData.yaml#/components/responses/401' '403': $ref: 'TS29571_CommonData.yaml#/components/responses/403' '404': $ref: 'TS29571_CommonData.yaml#/components/responses/404' '406': $ref: 'TS29571_CommonData.yaml#/components/responses/406' '429': $ref: 'TS29571_CommonData.yaml#/components/responses/429' '500': $ref: 'TS29571_CommonData.yaml#/components/responses/500' '503': $ref: 'TS29571_CommonData.yaml#/components/responses/503' default: $ref: 'TS29571_CommonData.yaml#/components/responses/default' put: operationId: UpdateConfiguration summary: 'Replace an existing Data Reporting Configuration subresource' requestBody: required: true content: application/json: schema: $ref: '#/components/schemas/DataReportingConfiguration' responses: '200': description: 'Data Reporting Configuration successfully replaced and updated resource representation is returned' content: application/json: schema: $ref: '#/components/schemas/DataReportingConfiguration' '204': description: 'Data Reporting Configuration successfully replaced' # No Content. '307': $ref: 'TS29122_CommonData.yaml#/components/responses/307' '308': $ref: 'TS29122_CommonData.yaml#/components/responses/308' '400': $ref: 'TS29571_CommonData.yaml#/components/responses/400' '401': $ref: 'TS29571_CommonData.yaml#/components/responses/401' '403': $ref: 'TS29571_CommonData.yaml#/components/responses/403' '404': $ref: 'TS29571_CommonData.yaml#/components/responses/404' '411': $ref: 'TS29571_CommonData.yaml#/components/responses/411' '413': $ref: 'TS29571_CommonData.yaml#/components/responses/413' '415': $ref: 'TS29571_CommonData.yaml#/components/responses/415' '429': $ref: 'TS29571_CommonData.yaml#/components/responses/429' '500': $ref: 'TS29571_CommonData.yaml#/components/responses/500' '503': $ref: 'TS29571_CommonData.yaml#/components/responses/503' default: $ref: 'TS29571_CommonData.yaml#/components/responses/default' patch: operationId: ModifyConfiguration summary: 'Modify an existing Data Reporting Configuration subresource' requestBody: required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/DataReportingConfigurationPatch' responses: '200': description: 'Data Reporting Configuration successfully replaced and updated resource representation is returned' content: application/json: schema: $ref: '#/components/schemas/DataReportingConfiguration' '204': description: 'Data Reporting Configuration successfully replaced' # No Content. '307': $ref: 'TS29122_CommonData.yaml#/components/responses/307' '308': $ref: 'TS29122_CommonData.yaml#/components/responses/308' '400': $ref: 'TS29571_CommonData.yaml#/components/responses/400' '401': $ref: 'TS29571_CommonData.yaml#/components/responses/401' '403': $ref: 'TS29571_CommonData.yaml#/components/responses/403' '404': $ref: 'TS29571_CommonData.yaml#/components/responses/404' '411': $ref: 'TS29571_CommonData.yaml#/components/responses/411' '413': $ref: 'TS29571_CommonData.yaml#/components/responses/413' '415': $ref: 'TS29571_CommonData.yaml#/components/responses/415' '429': $ref: 'TS29571_CommonData.yaml#/components/responses/429' '500': $ref: 'TS29571_CommonData.yaml#/components/responses/500' '503': $ref: 'TS29571_CommonData.yaml#/components/responses/503' default: $ref: 'TS29571_CommonData.yaml#/components/responses/default' delete: operationId: DestroyConfiguration summary: 'Destroy an existing Data Reporting Configuration' responses: '204': description: 'Data Reporting Configuration resource successfully destroyed' # No Content '307': $ref: 'TS29571_CommonData.yaml#/components/responses/307' '308': $ref: 'TS29571_CommonData.yaml#/components/responses/308' '400': $ref: 'TS29571_CommonData.yaml#/components/responses/400' '401': $ref: 'TS29571_CommonData.yaml#/components/responses/401' '403': $ref: 'TS29571_CommonData.yaml#/components/responses/403' '404': $ref: 'TS29571_CommonData.yaml#/components/responses/404' '429': $ref: 'TS29571_CommonData.yaml#/components/responses/429' '500': $ref: 'TS29571_CommonData.yaml#/components/responses/500' '503': $ref: 'TS29571_CommonData.yaml#/components/responses/503' default: $ref: 'TS29571_CommonData.yaml#/components/responses/default' components: securitySchemes: oAuth2ClientCredentials: type: oauth2 flows: clientCredentials: tokenUrl: '{tokenUri}' scopes: {} description: > For a trusted Provisioning AF, 'ndcaf-datareportingprovisioning' shall be used as 'scopes' and '{nrfApiRoot}/oauth2/token' shall be used as 'tokenUri'. schemas: DataReportingProvisioningSession: description: "A representation of a Data Reporting Provisioning Session." type: object properties: provisioningSessionId: readOnly: true allOf: - $ref: 'TS26510_CommonData.yaml#/components/schemas/ResourceId' aspId: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AspId' externalApplicationId: $ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId' internalApplicationId: $ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId' eventId: $ref: 'TS29517_Naf_EventExposure.yaml#/components/schemas/AfEvent' dataReportingConfigurationIds: readOnly: true type: array items: $ref: 'TS26510_CommonData.yaml#/components/schemas/ResourceId' minItems: 0 required: - provisioningSessionId - aspId - externalApplicationId - eventId - dataReportingConfigurationIds DataReportingConfiguration: description: "A Data Reporting Configuration subresource." type: object properties: dataReportingConfigurationId: readOnly: true allOf: - $ref: 'TS26510_CommonData.yaml#/components/schemas/ResourceId' dataCollectionClientType: $ref: 'TS26532_CommonData.yaml#/components/schemas/DataCollectionClientType' authorizationURL: $ref: 'TS26510_CommonData.yaml#/components/schemas/Url' dataSamplingRules: type: array items: $ref: 'TS26532_CommonData.yaml#/components/schemas/DataSamplingRule' minItems: 0 dataReportingConditions: type: array items: $ref: 'TS26532_CommonData.yaml#/components/schemas/DataReportingCondition' minItems: 1 dataReportingRules: type: array items: $ref: 'TS26532_CommonData.yaml#/components/schemas/DataReportingRule' minItems: 0 dataAccessProfiles: type: array items: $ref: '#/components/schemas/DataAccessProfile' minItems: 1 required: - dataReportingConfigurationId - dataCollectionClientType - dataReportingConditions - dataAccessProfiles DataReportingConfigurationPatch: description: "A JSON patch for a Data Reporting Configuration." type: object properties: authorizationURL: $ref: 'TS26510_CommonData.yaml#/components/schemas/Url' dataSamplingRules: type: array items: $ref: 'TS26532_CommonData.yaml#/components/schemas/DataSamplingRule' minItems: 0 dataReportingConditions: type: array items: $ref: 'TS26532_CommonData.yaml#/components/schemas/DataReportingCondition' minItems: 1 dataReportingRules: type: array items: $ref: 'TS26532_CommonData.yaml#/components/schemas/DataReportingRule' minItems: 0 dataAccessProfiles: type: array items: $ref: '#/components/schemas/DataAccessProfile' minItems: 1 DataAccessProfile: description: "A data access profile." type: object properties: dataAccessProfileId: type: string targetEventConsumerTypes: type: array items: $ref: '#/components/schemas/EventConsumerType' minItems: 0 uniqueItems: true parameters: type: array items: type: string minItems: 0 uniqueItems: true timeAccessRestrictions: type: object properties: duration: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' aggregationFunctions: type: array items: $ref: '#/components/schemas/DataAggregationFunctionType' minItems: 0 uniqueItems: true required: - duration - aggregationFunctions userAccessRestrictions: type: object properties: groupIds: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId' minItems: 0 uniqueItems: true userIds: type: array items: anyOf: - $ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi' - $ref: 'TS29571_CommonData.yaml#/components/schemas/Supi' minItems: 0 uniqueItems: true aggregationFunctions: type: array items: $ref: '#/components/schemas/DataAggregationFunctionType' minItems: 0 uniqueItems: true required: - groupIds - userIds - aggregationFunctions locationAccessRestrictions: type: object properties: locationAreas: type: array items: $ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G' minItems: 1 uniqueItems: true aggregationFunctions: type: array items: $ref: '#/components/schemas/DataAggregationFunctionType' minItems: 0 uniqueItems: true required: - locationAreas - aggregationFunctions required: - dataAccessProfileId - targetEventConsumerTypes - parameters EventConsumerType: description: "The type of event consumer." anyOf: - type: string enum: - NWDAF - EVENT_CONSUMER_AF - NEF - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. DataAggregationFunctionType: description: "The type of data aggregation function." anyOf: - type: string enum: - NONE - COUNT - MEAN - MAXIMUM - MINIMUM - SUM - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API.

# B.4	Ndcaf_DataReporting service API

For the purpose of referencing entities defined in this clause, it shall be assumed that the OpenAPI definitions below are contained in a physical file named "TS26532_Ndcaf_DataReporting.yaml".

openapi: 3.0.0 info: title: Ndcaf_DataReporting version: 2.1.2 description: ¦ Data Collection AF: Data Collection and Reporting Configuration API and Data Reporting API © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: Ndcaf_DataReporting description: 'Data Collection and Reporting: Client Configuration and Data Reporting (R2/R3/R4) APIs' externalDocs: description: 'TS 26.532 V18.5.0; Data Collection and Reporting; Protocols and Formats' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.532/' servers: - url: '{apiRoot}/3gpp-ndcaf_data-reporting/v1' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.532 clause 5.2. security: - {} - oAuth2ClientCredentials: [] paths: /sessions: post: operationId: CreateSession summary: 'Create a new Data Reporting Session' requestBody: required: true content: application/json: schema: $ref: '#/components/schemas/DataReportingSession' responses: '201': description: 'Data Reporting Session successfully created' headers: Location: description: 'URL including the resource identifier of the newly created Data Reporting Session.' required: true schema: $ref: 'TS26510_CommonData.yaml#/components/schemas/Url' content: application/json: schema: $ref: '#/components/schemas/DataReportingSession' '400': $ref: 'TS29571_CommonData.yaml#/components/responses/400' '401': $ref: 'TS29571_CommonData.yaml#/components/responses/401' '403': $ref: 'TS29571_CommonData.yaml#/components/responses/403' '404': $ref: 'TS29571_CommonData.yaml#/components/responses/404' '411': $ref: 'TS29571_CommonData.yaml#/components/responses/411' '413': $ref: 'TS29571_CommonData.yaml#/components/responses/413' '415': $ref: 'TS29571_CommonData.yaml#/components/responses/415' '429': $ref: 'TS29571_CommonData.yaml#/components/responses/429' '500': $ref: 'TS29571_CommonData.yaml#/components/responses/500' '503': $ref: 'TS29571_CommonData.yaml#/components/responses/503' default: $ref: 'TS29571_CommonData.yaml#/components/responses/default' /sessions/{sessionId}: parameters: - name: sessionId in: path required: true schema: $ref: 'TS26510_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Data Reporting Session.' get: operationId: RetrieveSession summary: 'Retrieve an existing Data Reporting Session' responses: '200': description: 'Representation of Data Reporting Session is returned' content: application/json: schema: $ref: '#/components/schemas/DataReportingSession' '307': $ref: 'TS29571_CommonData.yaml#/components/responses/307' '308': $ref: 'TS29571_CommonData.yaml#/components/responses/308' '400': $ref: 'TS29571_CommonData.yaml#/components/responses/400' '401': $ref: 'TS29571_CommonData.yaml#/components/responses/401' '403': $ref: 'TS29571_CommonData.yaml#/components/responses/403' '404': $ref: 'TS29571_CommonData.yaml#/components/responses/404' '406': $ref: 'TS29571_CommonData.yaml#/components/responses/406' '429': $ref: 'TS29571_CommonData.yaml#/components/responses/429' '500': $ref: 'TS29571_CommonData.yaml#/components/responses/500' '503': $ref: 'TS29571_CommonData.yaml#/components/responses/503' default: $ref: 'TS29571_CommonData.yaml#/components/responses/default' delete: operationId: DestroySession summary: 'Destroy an existing Data Reporting Session' responses: '204': description: 'Data Reporting Session resource successfully destroyed' # No Content '307': $ref: 'TS29571_CommonData.yaml#/components/responses/307' '308': $ref: 'TS29571_CommonData.yaml#/components/responses/308' '400': $ref: 'TS29571_CommonData.yaml#/components/responses/400' '401': $ref: 'TS29571_CommonData.yaml#/components/responses/401' '403': $ref: 'TS29571_CommonData.yaml#/components/responses/403' '404': $ref: 'TS29571_CommonData.yaml#/components/responses/404' '429': $ref: 'TS29571_CommonData.yaml#/components/responses/429' '500': $ref: 'TS29571_CommonData.yaml#/components/responses/500' '503': $ref: 'TS29571_CommonData.yaml#/components/responses/503' default: $ref: 'TS29571_CommonData.yaml#/components/responses/default' /sessions/{sessionId}/report: parameters: - name: sessionId in: path required: true schema: $ref: 'TS26510_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Data Reporting Session.' post: operationId: Report summary: 'Report UE data in the context of an existing Data Reporting Session' requestBody: required: true content: application/json: schema: $ref: '#/components/schemas/DataReport' responses: '200': description: 'Data Report accepted and updated Data Reporting Session is returned' headers: Location: description: 'URL including the resource identifier of the returned Data Reporting Session.' required: true schema: $ref: 'TS26510_CommonData.yaml#/components/schemas/Url' content: application/json: schema: $ref: '#/components/schemas/DataReportingSession' '204': description: 'Data Report accepted' # No Content '400': $ref: 'TS29571_CommonData.yaml#/components/responses/400' '401': $ref: 'TS29571_CommonData.yaml#/components/responses/401' '403': $ref: 'TS29571_CommonData.yaml#/components/responses/403' '404': $ref: 'TS29571_CommonData.yaml#/components/responses/404' '411': $ref: 'TS29571_CommonData.yaml#/components/responses/411' '413': $ref: 'TS29571_CommonData.yaml#/components/responses/413' '415': $ref: 'TS29571_CommonData.yaml#/components/responses/415' '429': $ref: 'TS29571_CommonData.yaml#/components/responses/429' '500': $ref: 'TS29571_CommonData.yaml#/components/responses/500' '503': $ref: 'TS29571_CommonData.yaml#/components/responses/503' default: $ref: 'TS29571_CommonData.yaml#/components/responses/default' components: securitySchemes: oAuth2ClientCredentials: type: oauth2 flows: clientCredentials: tokenUrl: '{tokenUri}' scopes: {} description: > For a trusted data collection client, 'ndcaf-datareporting' shall be used as 'scopes' and '{nrfApiRoot}/oauth2/token' shall be used as 'tokenUri'. schemas: DataReportingSession: description: "A representation of a Data Reporting Session." type: object properties: sessionId: readOnly: true allOf: - $ref: 'TS26510_CommonData.yaml#/components/schemas/ResourceId' validUntil: deprecated: true readOnly: true allOf: - $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' externalApplicationId: $ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId' supportedDomains: type: array items: $ref: '#/components/schemas/DataDomain' minItems: 0 samplingRules: readOnly: true type: object additionalProperties: type: array items: $ref: 'TS26532_CommonData.yaml#/components/schemas/DataSamplingRule' minItems: 0 reportingConditions: readOnly: true type: object additionalProperties: type: array items: $ref: 'TS26532_CommonData.yaml#/components/schemas/DataReportingCondition' minItems: 0 reportingRules: readOnly: true type: object additionalProperties: type: array items: $ref: 'TS26532_CommonData.yaml#/components/schemas/DataReportingRule' minItems: 0 required: - sessionId - externalApplicationId - supportedDomains - samplingRules - reportingConditions - reportingRules DataReport: description: "A data report sent by a data collection client to the Data Collection AF." type: object properties: externalApplicationId: $ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId' expedite: type: boolean serviceExperienceRecords: type: array items: $ref: '#/components/schemas/ServiceExperienceRecord' minItems: 1 locationRecords: type: array items: $ref: '#/components/schemas/LocationRecord' minItems: 1 communicationRecords: type: array items: $ref: '#/components/schemas/CommunicationRecord' minItems: 1 performanceDataRecords: type: array items: $ref: '#/components/schemas/PerformanceDataRecord' minItems: 1 applicationSpecificRecords: type: array items: $ref: '#/components/schemas/ApplicationSpecificRecord' minItems: 1 tripPlanRecords: type: array items: $ref: '#/components/schemas/TripPlanRecord' minItems: 1 aNBRNetworkAssistanceInvocationRecords: type: array items: $ref: 'TS26512_R2_DataReporting.yaml#/components/schemas/ANBRNetworkAssistanceInvocationRecord' minItems: 1 mediaStreamingAccessRecords: type: array items: $ref: 'TS26512_R4_DataReporting.yaml#/components/schemas/MediaStreamingAccessRecord' minItems: 1 required: - externalApplicationId DataDomain: description: "A data reporting domain." anyOf: - type: string enum: - SERVICE_EXPERIENCE - LOCATION - COMMUNICATION - PERFORMANCE - APPLICATION_SPECIFIC - MS_ANBR_NETWORK_ASSISTANCE - MS_ACCESS_ACTIVITY - PLANNED_TRIPS - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. BaseRecord: description: "Abstract base data type for UE data reporting records." type: object properties: timestamp: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' contextIds: type: array minItems: 1 items: type: string minLength: 1 required: - timestamp - contextIds ServiceExperienceRecord: description: "A data reporting record for UE service experience." allOf: - $ref: '#/components/schemas/BaseRecord' - type: object properties: serviceExperienceInfos: type: array items: $ref: '#/components/schemas/PerFlowServiceExperienceInfo' required: - serviceExperienceInfos PerFlowServiceExperienceInfo: description: "Information about the service experience of a single flow." type: object properties: serviceExperience: $ref: 'TS29517_Naf_EventExposure.yaml#/components/schemas/SvcExperience' timeInterval: $ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow' remoteEndpoint: $ref: 'TS29517_Naf_EventExposure.yaml#/components/schemas/AddrFqdn' required: - serviceExperience - timeInterval - remoteEndpoint LocationRecord: description: "A data reporting record for UE location." allOf: - $ref: '#/components/schemas/BaseRecord' - type: object properties: location: $ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LocationData' required: - location CommunicationRecord: description: "A data reporting record for UE communication." allOf: - $ref: '#/components/schemas/BaseRecord' - type: object properties: timeInterval: $ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow' sliceInfo: $ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai' dataNetworkName: $ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn' location: type: array minItems: 1 items: $ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G' uplinkVolume: $ref: 'TS29122_CommonData.yaml#/components/schemas/Volume' downlinkVolume: $ref: 'TS29122_CommonData.yaml#/components/schemas/Volume' required: - timeInterval PerformanceDataRecord: description: "A data reporting record for UE performance." allOf: - $ref: '#/components/schemas/BaseRecord' - type: object properties: timeInterval: $ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow' location: $ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G' remoteEndpoint: $ref: 'TS29517_Naf_EventExposure.yaml#/components/schemas/AddrFqdn' packetDelayBudget: $ref: 'TS29571_CommonData.yaml#/components/schemas/PacketDelBudget' packetLossRate: $ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate' uplinkThroughput: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' downlinkThrougput: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' required: - timeInterval ApplicationSpecificRecord: description: "A typed application-specific UE data reporting record." allOf: - $ref: '#/components/schemas/BaseRecord' - type: object properties: recordType: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' recordContainer: {} # (Syntax determined by recordType.) required: - recordType - recordContainer TripPlanRecord: description: "A data reporting record for UE performance." allOf: - $ref: '#/components/schemas/BaseRecord' - type: object properties: startingPoint: $ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LocationData' waypoints: type: array items: $ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LocationData' minItems: 1 destination: $ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LocationData' estimatedAverageSpeed: $ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/HorizontalSpeed' estimatedArrivalTime: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' required: - startingPoint - destination

End of changes