3GPP TSG-SA WG6 Meeting #67	S6-252446

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-252212)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0231 | rev | 1 | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove ENs regarding security aspects of bulk registration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2025-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-18 version TS23.554, the following ENs related to bulk registration are existing: Editor's note: Security aspects of bulk registration is the responsibility of SA3. Editor's note: Security aspects of Non-MSGin5G UE bulk registration is the responsibility of SA3. Editor's note: Security aspects of Non-MSGin5G UE bulk de-registration is the responsibility of SA3. Editor's note: Security aspects of MSGin5G UE bulk de-registration is the responsibility of SA3. SA3 has finished the related work in and the result of work is documented in Rel-19 in S3-241230. Therefore, Security aspects of bulk registration is implementation specific in Rel-18. The related ENs in TS23.554 can be revised to NOTEs to mention that the security aspects of bulk registration are implementation specific in Rel-18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR is proposed to remove ENs regarding security aspects of bulk registration in Rel-18. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unnecessary EN exists in the text. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.7, 8.2.9, 8.2.10, 8.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: numbered the notes |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.2.7	MSGin5G UE bulk registration over MSGin5G-6 reference point

When MSGin5G UE-1 and UE-2, which support an MSGin5G Client, perform registration via MSGin5G Gateway UE to contact the MSGin5G Server, the MSGin5G Gateway service functionality in the MSGin5G Gateway UE may decide to use bulk registration procedure specified in this clause based on the registration request sent from MSGin5G Clients in MSGin5G UE-1 and MSGin5G UE-2, if allowed by service policy. The procedure for MSGin5G UE bulk registration is illustrated in figure 8.2.7-1.

Pre-conditions:

1.	The MSGin5G Gateway UE has registered to the MSGin5G Server successfully.

2.	MSGin5G UE-1 and MSGin5G UE-2 have discovered and selected an MSGin5G Gateway UE as specified in clause 8.2.8 and connected to the serving network via MSGin5G Gateway UE successfully.

3.	UE Service IDs and the MSGin5G Server addresses have been configured on the MSGin5G Gateway UE, MSGin5G UE-1 and MSGin5G UE-2. All these MSGin5G UEs are served by a same MSGin5G Server.

4.	Both the MSGin5G UEs and MSGin5G Server have been configured with the necessary credentials to enable authenticating one another.

Figure 8.2.7-1: MSGin5G Client bulk registration

1.	The MSGin5G Client-1 sends an MSGin5G UE registration request to the MSGin5G Server as specified in clause 8.2.1, In addition to the Information Elements specified in table 8.2.1-1, the MSGin5G Client-1 may also add the Information Elements specified in table 8.2.7-1 in the MSGin5G UE registration request.

Table 8.2.7-1: MSGin5G UE registration request

| Information element | Status | Description |
| --- | --- | --- |
| Registration urgent degree | O | Indicates whether the registration request is urgent or not. |
| Wait time | O | Indicates the maximum wait time of the registration request allowed by the MSGin5G Client which sent the registration request before the bulk registration request is sent to the MSGin5G Server. |

2.	Upon receiving the MSGin5G UE registration request, the MSGin5G Gateway Client checks whether this request can be queued for bulk-handling based on the service policy and the information included in the MSGin5G UE registration request, e.g. whether the MSGin5G UE registration request is urgent.

NOTE 1:	Whether bulk registration is supported can be configured in the MSGin5G Gateway Client.

NOTE 2:	The bulk registration conditions are specified by the application provider or MSGin5G Service provider and are out of scope of this document.

3.	Based on step 2:

a)	if the MSGin5G UE registration request is urgent, MSGin5G Gateway Client may also include other registration requests; or

b)	if the MSGin5G UE registration request can be bulk-handled, MSGin5G Gateway Client sends a bulk registration notification to MSGin5G Client-1 and caches/stores the MSGin5G UE registration request until the bulk registration conditions can be fulfilled. The Information Elements specified in table 8.2.7-2 are included in the bulk registration notification. The steps 4 and 5 are skipped.

Table 8.2.7-2: bulk registration notification

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID | M | UE service identifier assigned to the requesting MSGin5G UE. |
| Expected registration time | O | The expected time when the bulk registration can be handled and the MSGin5G UE registration Response can be received. |

4.	The MSGin5G Server handles the MSGin5G UE registration request as specified in clause 8.2.1 and sends the corresponding MSGin5G UE registration response to MSGin5G Gateway Client.

5.	The MSGin5G Gateway Client sends the MSGin5G UE registration response to MSGin5G Client-1. The MSGin5G Client-1 handles the MSGin5G UE registration response as specified in clause 8.2.1 and skips all the remaining steps in the present clause.

6.	MSGin5G Client-2 sends an MSGin5G UE registration request and the MSGin5G UE registration request is handed as in steps 2-5.

7.	The MSGin5G Gateway Client finds that the conditions of sending a bulk registration request can be fulfilled, e.g.

a)	the registration request expiration time in the MSGin5G UE registration request sent from MSGin5G Client-1 or MSGin5G Client-2 is reached; or

b)	the periodic bulk registration interval is reached; or

c)	the maximum MSGin5G UE registration request number in a MSGin5G UE bulk registration request is reached.

NOTE 3:	The maximum registration time, periodic bulk registration interval and maximum MSGin5G UE registration request number are implementation specific and out of the scope of the current specification.

8.	The MSGin5G Gateway Client includes all cached/stored MSGin5G UE registration requests in an MSGin5G UE bulk registration request and sends it to the MSGin5G Server. The Information Elements specified in table 8.2.7-3 are included in the MSGin5G UE bulk registration request.

Table 8.2.7-3: MSGin5G UE bulk registration request

| Information element | Status | Description |
| --- | --- | --- |
| Number of individual MSGin5G UE registration requests | M | Indicates total number of MSGin5G UE registration requests which are bulked in this MSGin5G UE bulk registration request |
| List of individual MSGin5G UE registration request | M | Each element in this list contains information as specified in Table 8.2.1-1. |

9.	Upon receiving the MSGin5G UE bulk registration request, the MSGin5G Server splits the MSGin5G UE bulk registration request, executes the authentication, authorization and registration procedures for each MSGin5G Client individually as specified in clause 8.2.1.

NOTE 4:	Security aspects of bulk registration is implementation specific in this release.

10.	The MSGin5G Server includes all corresponding MSGin5G UE registration responses in an MSGin5G UE bulk registration response and sends it to MSGin5G Gateway Client. The Information Elements specified in table 8.2.7-4 are included in the MSGin5G UE bulk registration response.

Table 8.2.7-4: MSGin5G UE bulk registration response

| Information element | Status | Description |
| --- | --- | --- |
| Number of individual MSGin5G UE registration responses | M | Indicates total number of MSGin5G UE registration responses which are bulked in this MSGin5G UE bulk registration response |
| List of individual MSGin5G UE registration response | M | Each element in this list contains information as specified in Table 8.2.1-4. |

11.	Upon receiving the MSGin5G UE bulk registration response, the MSGin5G Gateway Client splits the MSGin5G UE bulk registration response into multiple individual MSGin5G UE registration responses. Each individual MSGin5G UE registration response contains information as specified in Table 8.2.1-4.

12.	MSGin5G Gateway Client sends the MSGin5G UE registration Response 1 to MSGin5G Client-1.

13.	MSGin5G Gateway Client sends the MSGin5G UE registration Response 2 to MSGin5G Client-2.

* * * Next Change * * * *

8.2.9	Non-MSGin5G UE bulk registration

Non-MSGin5G UEs (i.e., Legacy 3GPP UEs or Non-3GPP UEs) are connected to the MSGin5G Server through a Message Gateway. The Message Gateway may decide to use bulk registration procedure specified in this clause based on service policy. The procedure for Non-MSGin5G UE bulk registration is illustrated in figure 8.2.9-1.

Pre-conditions:

1.	The Message Gateway has been pre-configured with the MSGin5G Server address.

2.	The Message Gateway has been configured with the necessary information as specified in clause 8.1.3. If the UE Service ID and Non-MSGin5G UE credentials have been configured and the Non-MSGin5G UEs' Profile is available, this pre-condition enables authentication and Non- MSGin5G UEs registration at the Message Server.

3.	A secured connection has been established between the Message Gateway and the MSGin5G Server.

Figure 8.2.9-1: Non-MSGin5G UE bulk registration

1.	The Message Gateway determines to perform Non-MSGin5G UE bulk registration.

NOTE 1:	The conditions of whether the Non-MSGin5G UE bulk registration is used are implementation specific and out of the scope of the current specification.

2.	The Message Gateway sends a Non-MSGin5G UE bulk registration request to the MSGin5G Server. The Information Elements specified in table 8.2.9-1 are included in the Non-MSGin5G UE bulk registration request.

Table 8.2.9-1: Non-MSGin5G UE bulk registration request

| Information element | Status | Description |
| --- | --- | --- |
| Number of individual MSGin5G UE registration requests | M | Indicates total number of Non-MSGin5G UE registration requests which are bulked in this Non-MSGin5G UE bulk registration request |
| List of individual Non-MSGin5G UE information | M | Each element in this list contains information as specified in Table 8.2.3-1. |

3.	Upon receiving the Non-MSGin5G UE bulk registration request, the MSGin5G Server processes each of the UE registrations from the list individually as specified in clause 8.2.3. For each individual Non-MSGin5G UE registration that successful, the MSGin5G Server stores that UE Service ID and associated Non-MSGin5G UE Profile information.

NOTE 2:	Security aspects of bulk registration is implementation specific in this release.

4.	The MSGin5G Server includes all corresponding MSGin5G UE registration responses in a Non-MSGin5G UE bulk registration response and sends it to the Message Gateway. The Information Elements specified in table 8.2.9-2 are included in the Non-MSGin5G UE bulk registration response.

Table 8.2.9-2: MSGin5G UE bulk registration response

| Information element | Status | Description |
| --- | --- | --- |
| Number of individual Non-MSGin5G UE registration responses | M | Indicates total number of Non-MSGin5G UE registration responses which are bulked in this MSGin5G UE bulk registration response |
| List of individual Non-MSGin5G UE registration response | M | Each element in this list contains information as specified in Table 8.2.1-4. |

* * * Next Change * * * *

8.2.10	Non-MSGin5G UE bulk de-registration

The procedure assumes that the Message Gateway is responsible for initiating the bulk de-registration from the MSGin5G Server on behalf of the Non-MSGin5G UEs. The signaling flow for Non-MSGin5G UE bulk de-registration is illustrated in figure 8.2.10-1.

Pre-conditions:

1.	The Message Gateway successfully performed registration with the MSGin5G Server on behalf of a set of Non-MSGin5G UEs.

Figure 8.2.10-1: Non-MSGin5G UE bulk de-registration

1.	The Message Gateway determines to perform Non-MSGin5G UE bulk de-registration.

NOTE 1:	The conditions of whether the Non-MSGin5G UEs can be bulked de-registered are implementation specific and out of the scope of the current specification.

2.	The Message Gateway sends a Non-MSGin5G UE bulk de-registration request to the MSGin5G Server. The Information Elements specified in table 8.2.10-1 are included in the Non-MSGin5G UE bulk de-registration request.

Table 8.2.10-1: Non-MSGin5G UE bulk de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| Number of individual MSGin5G UE de-registration requests | M | Indicates total number of Non-MSGin5G UE de-registration requests which are bulked in this Non-MSGin5G UE bulk de-registration request |
| List of individual UE Service ID | M | Each element in this list contains a UE service identifier assigned to the Non-MSGin5G UE. |

3.	Upon receiving the request, the MSGin5G Server processes each of the UE de-registration from the list individually as specified in clause 8.2.4. The MSGin5G Server deletes any applicable UE Service ID in the List of individual UE Service ID and associated MSGin5G Client Profile information that it has stored.

NOTE 2:	Security aspects of bulk registration is implementation specific in this release.

4.	The MSGin5G Server replies with a Non-MSGin5G UE bulk de-registration response as shown in table 8.2.10-2.

Table 8.2.10-2: Non-MSGin5G UE bulk de-registration response

| Information element | Status | Description |
| --- | --- | --- |
| Number of individual Non-MSGin5G UE de-registration responses | M | Indicates total number of Non-MSGin5G UE de-registration responses which are bulked in this Non-MSGin5G UE bulk de-registration response |
| List of individual Non-MSGin5G UE de-registration response | M | Each element in this list contains information as specified in Table 8.2.4-2. |

* * * Next Change * * * *

8.2.11	MSGin5G UE bulk de-registration over MSGin5G-6 reference point

The MSGin5G Gateway UE may decide to use bulk de-registration procedure specified in this clause based on the de-registration request sent from MSGin5G Clients in MSGin5G UEs or, if allowed by service policy. The procedure for MSGin5G UE bulk de-registration is illustrated in figure 8.2.11-1.

Pre-conditions:

1.	The MSGin5G Gateway UE has registered to the MSGin5G Server successfully.

2.	MSGin5G UE-1 and MSGin5G UE-2 have registered to the MSGin5G Server via MSGin5G Gateway UE successfully as specified in clause 8.2.8.

Figure 8.2.11-1: MSGin5G UE bulk de-registration

1.	The MSGin5G Client-1 sends an MSGin5G UE de-registration request to the MSGin5G Server as specified in clause 8.2.2, In addition to the Information Elements specified in table 8.2.2-1, the MSGin5G Client-1 may also add the Information Elements specified in table 8.2.11-1 in the MSGin5G UE de-registration request.

Table 8.2.11-1: MSGin5G UE de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| De-registration urgent degree | O | Indicates whether the de-registration request is urgent or not. |
| Wait time | O | Indicates the maximum wait time of the de-registration request allowed by the MSGin5G Client-2 before the bulk de-registration request is sent to the MSGin5G Server. |

2.	Upon receiving the MSGin5G UE de-registration request, the MSGin5G Gateway Client checks whether this request can be queued for bulk-handling based on the service policy and the information included in the MSGin5G UE registration request, e.g. whether the MSGin5G UE de-registration request is urgent.

NOTE 1:	Whether bulk de-registration is supported can be configured in the MSGin5G Gateway Client.

NOTE 2:	The bulk de-registration conditions are specified by the application provider or MSGin5G Service provider and are out of scope of this document.

3.	Based on step 2:

a)	if the MSGin5G UE de-registration request is urgent, the MSGin5G Gateway Client may also include other registration requests; or

b)	if the MSGin5G UE de-registration request can be bulked, MSGin5G Client-1 sends a bulk de-registration notification to MSGin5G Client-1 and caches/stores the MSGin5G UE de-registration request until the bulk de-registration conditions can be fulfilled. The Information Elements specified in table 8.2.11-2 are included in the bulk de-registration notification. The steps 4 and 5 are skipped.

Table 8.2.11-2: bulk de-registration notification

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID | M | UE service identifier assigned to the requesting MSGin5G UE. |
| Expected de-registration time | O | The expected time when the bulk de-registration can be handled and the MSGin5G UE de-registration response can be received. |

4.	The MSGin5G Server handles the MSGin5G UE de-registration request as specified in clause 8.2.2 and sends the corresponding MSGin5G UE de-registration response to MSGin5G Gateway Client.

5.	The MSGin5G Gateway Client sends the MSGin5G UE de-registration response to MSGin5G Client-1. The MSGin5G Client-1 handles the MSGin5G UE de-registration response as specified in clause 8.2.2 and skips all the remaining steps in the present clause.

6.	MSGin5G Client-2 sends an MSGin5G UE de-registration request and the MSGin5G UE de-registration request is handled as in steps 2-5.

7.	The MSGin5G Gateway Client finds that the conditions of sending a bulk de-registration request can be fulfilled, e.g.

a)	the de-registration request expiration time in the MSGin5G UE de-registration request sent from MSGin5G Client-1 or MSGin5G Client-2 is reached; or

b)	the periodic bulk de-registration interval is reached; or

c)	the maximum MSGin5G UE de-registration request number in a MSGin5G UE bulk de-registration request is reached.

NOTE 3:	The maximum de-registration time, periodic bulk de-registration interval and maximum MSGin5G UE de-registration request number are implementation specific and out of the scope of the current specification.

8.	The MSGin5G Gateway Client includes all cached/stored MSGin5G UE de-registration requests in an MSGin5G UE bulk de-registration request and sends it to the MSGin5G Server. The Information Elements specified in table 8.2.11-3 are included in the MSGin5G UE bulk de-registration request.

Table 8.2.11-3: MSGin5G UE bulk de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| Number of individual MSGin5G UE de-registration requests | M | Indicates total number of MSGin5G UE de-registration requests which are bulked in this MSGin5G UE bulk de-registration request |
| List of individual MSGin5G UE de-registration request | M | Each element in this list contains information as specified in Table 8.2.2-1. |

9.	Upon receiving the MSGin5G UE bulk de-registration request, the MSGin5G Server processes each of the MSGin5G UE de-registration from the list individually as specified in clause 8.2.2.

NOTE 4:	Security aspects of bulk registration is implementation specific in this release.

10.	The MSGin5G Server includes all corresponding MSGin5G UE de-registration responses in an MSGin5G UE bulk de-registration response and sends it to MSGin5G Gateway Client. The Information Elements specified in table 8.2.11-4 are included in the MSGin5G UE bulk de-registration response.

Table 8.2.11-4: MSGin5G UE bulk de-registration response

| Information element | Status | Description |
| --- | --- | --- |
| Number of individual MSGin5G UE de-registration responses | M | Indicates total number of MSGin5G UE de-registration responses which are bulked in this MSGin5G UE bulk de-registration response |
| List of individual MSGin5G UE de-registration response | M | Each element in this list contains information as specified in Table 8.2.2-2 |

11.	Upon receiving the MSGin5G UE bulk de-registration response, the MSGin5G Gateway Client splits the MSGin5G UE bulk de-registration response into multiple individual MSGin5G UE de-registration responses. Each individual MSGin5G UE de-registration response contains information as specified in Table 8.2.2-2.

12.	MSGin5G Gateway Client sends the MSGin5G UE de-registration Response 1 to MSGin5G Client-1.

13.	MSGin5G Gateway Client sends the MSGin5G UE de-registration Response 2 to MSGin5G Client-2.