3GPP TSG-SA WG6 Meeting #66	S6-251060

Goteborg, Sweden, 7th – 11th April 2025	(revision of S6-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.283 | CR | 0093 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Interworking support for ad hoc group emergency alerts (Information flows) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Kontron Transportation France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph6-MC |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR adds the information flows for ad hoc group emergency alertbetween the MCPTT server and the IWF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding information flows for ad hoc group emergency alert interworking. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interworking is not fully supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 10.17.2.6, (new) 10.17.2.7, (new) 10.17.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change * * * *

#### 10.17.2.6	IWF ad hoc group emergency alert request (MCPTT server – IWF and IWF - MCPTT server)

Table 10.17.2.6-1 describes the information flow IWF ad hoc group emergency alert request from the MCPTT server to the IWF and from the IWF to the MCPTT server.

Table 10.17.2.6-1 IWF Ad hoc group emergency alert request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The identity of the alerting party |
| Functional alias | O | The functional alias of the alerting party |
| Ad hoc group ID | M | The target group ID associated with the alert |
| Organization name | O | The alerting MCPTT user's mission critical organization name |
| Location information | O | The alerting MC service client's location |
| Additional information | O | The alerting party may provide additional information related to the alert, e.g., medical help needed or reason for the alert |
| Call resulting criteria for determining the participants | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both that the MCPTT server used for determining the participants e.g., it can be a location-based criteria to invite participants in a particular area |

#### 10.17.2.7	IWF ad hoc group emergency alert request return (MCPTT server – IWF)

Table 10.17.2.7-1 describes the information flow IWF ad hoc group emergency alert request return from the MCPTT server to the IWF.

Table 10.10.2.7-1 IWF ad hoc group emergency alert request return information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The identity of the alerting party |
| Functional alias | O | The functional alias of the alerting party |
| Authorization result | M | Indicate if authorization is success or failure |

#### 10.17.2.8	IWF ad hoc group emergency alert response (IWF - MCPTT server and MCPTT server - IWF)

Table 10.17.2.8-1 describes the information flow IWF ad hoc group emergency alert response from the IWF to the MCPTT server and from the MCPTT server to the IWF.

Table 10.17.2.8-1 IWF Ad hoc group emergency alert response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The identity of the target party |
| Functional alias | O | The functional alias of the target party |
| Ad hoc group ID | M | The target group ID associated with the alert |
| Call resulting criteria for determining the participants | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both that the MCPTT server used for determining the participants e.g., it can be a location-based criteria to invite participants in a particular area |

* * * End of Change * * * *